# Manufacturer Design & Simulation Tools

A reference list of free, vendor-hosted design/simulation tools — the WEBENCH-style "enter your specs, get a circuit" tools, plus configurators, selectors, and calculators from silicon and component manufacturers. Compiled September 2026.

## 1. Power Electronics / Power Supply Design

| Tool | Vendor | What it does |
| --- | --- | --- |
| WEBENCH Power Designer / Circuit Designer | Texas Instruments | The original: enter Vin/Vout/Iout and get a ranked list of complete power-supply designs (schematic, BOM, simulation, CAD export). Still actively maintained. |
| ADI Power Studio | Analog Devices | Umbrella suite launched Oct 2025: Power Studio Planner (system-level power-tree architecture) + Power Studio Designer (IC-level design with live efficiency/loss analysis). Also houses the older LTpowerCAD®/LTpowerPlanner® and LTspice®. |
| PowerEsim | Infineon | Full SMPS + transformer design tool — loss analysis, thermal simulation, many topologies, not Infineon-only. |
| Elite Power Simulator | onsemi | Cloud simulator for EliteSiC MOSFETs/IGBTs/IPMs — device-level switching behavior and PLECS system models, aimed at higher-power SiC designs. |
| WebDesigner+ Power Supply | onsemi | Enter I/O requirements, get recommended PMICs, topology, and passives — closest onsemi equivalent to WEBENCH's original flow. |
| DC/DC Designer Online | Monolithic Power Systems (MPS) | Pick an MPS regulator + target specs → simulated circuit, schematic, layout guide, BOM. |
| eDesignSuite – Power Management Design Center | STMicroelectronics | No-login suite: DC/DC & AC/DC converter design, multi-rail Power Tree Designer, and a Digital Power Workbench for STM32-based digital control loops. |
| Power System Designer (PowerBench) | Vicor | Architect multi-output power chains from Vicor's brick/module portfolio; up to 75% faster than manual selection per Vicor. |
| REDEXPERT | Würth Elektronik | Component search and design tool for inductors/ferrites/capacitors. The DC-DC Converter Designer module simulates real (not datasheet-ideal) inductor/MLCC behavior and recommends a controller IC — very useful alongside any of the above. |

## 2. Analog Electronics / Signal Chain

| Tool | Vendor | What it does |
| --- | --- | --- |
| Analog Filter Wizard | Analog Devices | Design active low/high/band-pass filters with real op-amps; shows ideal vs. real-world response, exports SPICE files. |
| ADIsimRF | Analog Devices | RF signal-chain calculator — cascaded gain, noise figure, IP3, P1dB, power consumption across up to 20 stages. |
| ADIsimPLL | Analog Devices | PLL synthesizer design/simulation — phase noise, fractional-N spurs, anti-backlash pulse. (Same design-tools hub as above.) |
| DiffAmpCalc / Precision ADC Driver Tool / Photodiode Wizard / Virtual Eval | Analog Devices | Suite of interactive web tools for signal-conditioning circuits — differential amps, ADC drivers, photodiode transimpedance stages, and ADC/DAC performance simulation. |
| LTspice | Analog Devices | ADI's preferred SPICE simulator (ADIsimPE was sunsetted Sept 2019); free, huge model library. |
| ANALOG-FILTER-DESIGNER | Texas Instruments | Downloadable filter design tool (TI's counterpart to ADI's Filter Wizard). |

## 3. Embedded Systems — MCU Configuration & Code Generation

| Tool | Vendor | What it does |
| --- | --- | --- |
| STM32CubeMX | STMicroelectronics | Graphical pin-mux, clock-tree, and peripheral configurator for STM32 MCUs/MPUs; auto-generates HAL/LL init C code and detects conflicts. Standalone or built into STM32CubeIDE. |
| SysConfig | Texas Instruments | Graphical pin-mux and driver/stack configurator across MSPM0, C2000, SimpleLink, and MSP430 families; generates commented C code, real-time conflict checking. |
| MPLAB Code Configurator (MCC) | Microchip | Peripheral/driver code generator plugin for MPLAB X — covers 8/16/32-bit PIC (including PIC32/Harmony), AVR, and dsPIC. Two flavors: MCC Classic and the newer MCC Melody. |
| esp-gpio-tool | Espressif | Official YAML-based GPIO/pin-constraint validator for ESP32 designs (data also powers the community reference site esp32pin.com). |

## 4. Digital Systems / FPGA

| Tool | Vendor | What it does |
| --- | --- | --- |
| Xilinx Power Estimator (XPE) / Power Design Manager | AMD | Spreadsheet/GUI tool for pre-design FPGA power budgeting (static + dynamic) across logic, I/O, and transceivers; PDM is the newer platform for Versal/Kria devices. Pairs with Vivado. |
| Early Power Estimator (EPE) / Power and Thermal Calculator (PTC) | Altera (Intel) | Same role for Altera FPGAs — early-stage power/thermal estimation before or during Quartus Prime implementation. |

## 5. Clocks & Timing

| Tool | Vendor | What it does |
| --- | --- | --- |
| ClockBuilder Pro (CBPro) | Skyworks (formerly Silicon Labs' timing line) | Step-by-step clock-tree design for clock generators, jitter attenuators, and buffers; validates feasibility, recommends devices, generates register maps and production files. |
| Clock Tree Architect | Texas Instruments | Clock-tree synthesis tool that generates solutions from system requirements (listed under TI's design/simulation tools hub). |

## 6. Communications Protocols / RF & Interfaces

| Tool | Vendor | What it does |
| --- | --- | --- |
| CAN Bus Bit Timing Calculator (+ CAN FD version) | Kvaser | Calculates valid CAN/CAN-FD bit-timing register sets (prescaler, segments, SJW, sample point) for a given clock and bus speed. |
| MatchCalc | Qorvo | RF/microwave impedance-matching calculator from S1P/S2P files, with live Smith Chart tuning. |
| RF Impedance Matching Calculator | Qorvo | Quick L-match network calculator (C and L values) for matching amplifier stages. |

### Notes

- Most of these require a free account only for saving/exporting designs — browsing and basic use is generally open.

- Several vendors merged or rebranded recently: Maxim → Analog Devices (shared toolset); Silicon Labs' timing business → Skyworks (ClockBuilder Pro).

- TI's WEBENCH Amplifier/Sensor Designer tools were discontinued in Jan 2020 (Adobe Flash EOL) — only the Power Designer/Circuit Designer side of WEBENCH survives, and it's still being updated.

- REDEXPERT and MatchCalc/RF tools pair well with the power/analog tools above — use them to select the actual magnetics or RF matching components once a controller IC is chosen.