PCB DESIGN

COMPREHENSIVE CHEAT SHEET

Schematic Design  •  PCB Layout  •  Design Workflow  •  KiCAD Tricks & Techniques  •  Signal Integrity  •  EMC  •  Manufacturing

Professional Engineering Reference  |  26 Comprehensive Sections

# 1. PCB DESIGN WORKFLOW — FULL PROCESS

| COMPLETE PCB DESIGN FLOW (SCHEMATIC → ASSEMBLED BOARD) |  |  |  |
| --- | --- | --- | --- |
| PHASE 1 — REQUIREMENTS & PLANNING |  |  |  |
| Define requirements | Voltage, current, signal freq, temp range, size, cost | Standards | IPC-2221 (generic), IPC-2222 (rigid), IPC-6012 (qual) |
| Layer count decision | 2-layer (simple), 4-layer (mixed), 6+ (high-speed/RF) | Board size | Verify with mechanical team before starting layout |
| Component selection | Availability, footprint, package thermal, cost, lead time | BOM planning | Check JLCPCB/LCSC/Mouser stock early in design |
| PHASE 2 — SCHEMATIC DESIGN |  |  |  |
| Create symbols | Draw or import — match datasheet pin names exactly | Annotate | R1,R2... C1,C2... U1,U2... — consistent prefixes |
| Hierarchical design | Top-level sheet calls sub-sheets per functional block | Flat design | Single sheet — OK for simple boards (<50 parts) |
| ERC (Electrical RC) | Run before layout — fix all errors, review warnings | Net naming | Name critical nets: VCC_3V3, GND_AGND, CLK_48MHz |
| BOM generation | Export CSV with MPN, value, footprint, quantity | Cross-check | Verify every component has a valid footprint assigned |
| PHASE 3 — PCB LAYOUT |  |  |  |
| Import netlist | Sync schematic → PCB (KiCAD: Update PCB from Schematic) | Board outline | Set board edge (User.Cuts or Edge.Cuts layer) |
| Place components | Group by function, power first, then signal | Constraint zones | Define keepout, copper pour, via restrictions |
| Route power | Wide traces first: VCC, GND, PWR rails | Route signal | Short, direct, avoid acute angles (use 45° or curves) |
| Pour GND plane | Fill remaining copper with GND on all layers | Check DRC | Fix all errors before sending to fab |
| PHASE 4 — VERIFICATION & OUTPUT |  |  |  |
| DRC | Design Rule Check — clearance, short, unconnected | 3D check | Verify 3D model for mechanical fit + collision |
| Gerber export | One file per layer + drill file + drill map | Fab review | Use fab's DFM checker (JLC, PCBWay, OSHPark) |
| PCBA output | Centroid (pick & place) + BOM for assembly | Silkscreen | Verify all ref designators visible, not under parts |
| PHASE 5 — PROTOTYPE & VALIDATION |  |  |  |
| Bare board test | Continuity, shorts, impedance if controlled | Assembly | Reflow (SMD) / wave (THT) / hand solder |
| Bring-up sequence | Power rails first → core → peripherals → firmware | Oscilloscope | Check clock, power ripple, signal integrity |
| ECO (Eng Change) | Document every change: what, why, who, when | Revision | Rev A (proto), Rev B (fix), Rev C (production) |
| NOTE: Follow IPC-2221B for generic design requirements. Every design decision should be traceable to a requirement. Never skip ERC/DRC — fix all errors before proceeding. |  |  |  |

# 2. SCHEMATIC DESIGN FUNDAMENTALS

| SCHEMATIC DRAWING STANDARDS & CONVENTIONS |  |  |  |
| --- | --- | --- | --- |
| ORGANISATION & READABILITY |  |  |  |
| Signal flow | Left to right, top to bottom — inputs left, outputs right | Sheet border | Include title block: project, rev, date, author, sheet |
| Net labels vs wires | Use net labels for signals crossing far distances | Power symbols | Use VCC/GND power symbols — not wired to rails |
| Off-sheet refs | Hierarchical pins or global labels for multi-sheet | Net ties | Connect different GND domains (AGND/DGND) with net tie |
| No connect (X) | Mark unused pins explicitly — prevents ERC errors | Dangling wire | Never leave wire ends unconnected — always terminate |
| Component spacing | Leave space around ICs for annotations and readability | Wire angles | Horizontal/vertical only — no diagonal wires |
| REFERENCE DESIGNATORS (IPC-2612) |  |  |  |
| R — Resistor | R1, R2, R3... | C — Capacitor | C1, C2, C3... |
| L — Inductor | L1, L2... | U — IC/Module | U1, U2... (integrated circuit) |
| Q — Transistor | Q1, Q2... (BJT/FET/IGBT) | D — Diode | D1, D2... (diodes, LEDs, Zener) |
| J — Connector | J1, J2... (jacks, headers, terminals) | SW — Switch | SW1, SW2... |
| F — Fuse | F1, F2... | TP — Test Point | TP1, TP2... (probe points) |
| FB — Ferrite Bead | FB1, FB2... | Y — Crystal | Y1, Y2... (oscillators, resonators) |
| POWER & GROUND HIERARCHY |  |  |  |
| VBAT | Battery voltage (unregulated, high impedance source) | VIN / VMAIN | Primary input supply (before regulator) |
| VCC / VBUS | Main regulated supply (e.g. 5V, 3.3V) | VCORE | Digital core supply (1.0–1.8V for MCU/FPGA) |
| VDDA / AVDD | Analogue supply — separate from digital VCC | VREF | Precision reference for ADC/DAC |
| AGND | Analogue ground — connect to DGND at single star point | PGND | Power ground — high-current return paths |
| DGND / GND | Digital ground — main logic return | Shield GND | Chassis/screen ground — connect at one point |
| DECOUPLING CAPACITORS IN SCHEMATIC |  |  |  |
| 100nF ceramic | Place on every VCC pin of every IC (mandatory) | 10µF bulk | Per power domain: near voltage regulator output |
| 1µF ceramic | For ICs with multiple VCC pins (bridge 100nF & 10µF) | VCORE decoup. | 100nF + 1µF per VCORE pin on MCU/FPGA |
| Value annotation | Always label value + voltage rating: 100nF/10V X7R | Symbol | Use generic cap symbol — assign value in properties |
| SCHEMATIC ANNOTATION & BOM DISCIPLINE |  |  |  |
| MPN field | Manufacturer Part Number in component properties — e.g. MPN = RC0402FR-0710KL (Yageo 10k 1% 0402) |  |  |
| Value format | Resistors: 10k, 4.7k, 100R ¦ Capacitors: 100nF, 10µF ¦ Inductors: 10µH ¦ Always include tolerance |  |  |
| Footprint | Assign EARLY: Resistor_SMD:R_0402 ¦ Capacitor_SMD:C_0603 ¦ Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |  |  |
| NOTE: Golden rule: schematic is the engineering intent — it must be correct, readable, and complete BEFORE layout begins. A bad schematic creates a bad PCB. |  |  |  |

# 3. COMPONENT FOOTPRINTS & LAND PATTERNS

| SMD PACKAGE SIZES & DIMENSIONS |  |  |  |
| --- | --- | --- | --- |
| RESISTORS & CAPACITORS — IMPERIAL / METRIC |  |  |  |
| 0201 (0603 metric) | 0.6×0.3mm — reflow only, no hand solder | 0402 (1005) | 1.0×0.5mm — small, standard for modern designs |
| 0603 (1608) | 1.6×0.8mm — hand-solderable, beginner-friendly | 0805 (2012) | 2.0×1.25mm — easy to solder, good for high voltage |
| 1206 (3216) | 3.2×1.6mm — high power, high voltage caps | 1210 (3225) | 3.2×2.5mm — large capacitors (10µF+) |
| 2512 (6432) | 6.4×3.2mm — power resistors (1W+) | Minimum pitch | 0402 for production; 0201 for space-critical |
| IC PACKAGES |  |  |  |
| SOT-23-3 | 3-pin, 0.95mm pitch — BJT, small regulators | SOT-23-5/6 | 5/6-pin, 0.95mm — dual gates, small ICs |
| SOIC-8 | 1.27mm pitch, 3.9 or 5.3mm wide — op-amps, drivers | SOIC-16 | 1.27mm pitch, 16 pins — logic, comms ICs |
| TSSOP-20/24 | 0.65mm pitch — dense logic, DACs, drivers | TQFP-32/48/64 | 0.8mm pitch — MCUs, CPLDs |
| LQFP-100/144 | 0.5mm pitch — STM32, larger MCUs | QFN-20..64 | No-lead bottom pad — good thermal, needs reflow |
| BGA packages | 0.5/0.8/1.0mm pitch — FPGAs, processors, DDR | BGA via-in-pad | Requires ECIA pad plating for reliability |
| THROUGH-HOLE (THT) PACKAGES |  |  |  |
| DIP-8/14/16/28/40 | 2.54mm (100mil) pitch — ICs, legacy | TO-220 | Power transistors, regulators — bolt to heatsink |
| TO-92 | Plastic transistor — BC547, 2N2222 | TO-247 | High-power FETs/IGBTs > 10A |
| Axial R/C | 1.27mm min lead pitch for 100mil grid | Radial C | Electrolytic — 2.54mm or 5.08mm pitch |
| LAND PATTERN DESIGN RULES (IPC-7351) |  |  |  |
| Courtyard | Minimum clearance envelope around part (0.25mm min) | Fab layer | Shows actual component body outline |
| Courtyard clearance | 0.25mm (IPC-7351B nominal), 0.5mm for hand assembly | Silkscreen | Do NOT overlap pads — move to fab layer if needed |
| Paste aperture | Paste area = 88% of pad area (default), adjust for QFN | Center pad | QFN/DFN thermal pad: cross-hatch paste, 50–70% coverage |
| Pad extensions | Extend pads beyond body for inspection / hand soldering | SOD-123 heel | Add 0.3mm heel extension beyond body line |
| CONNECTOR FOOTPRINTS |  |  |  |
| 2.54mm header | Standard Arduino-style, hand-solderable | 1.27mm header | Compact, surface mount available |
| JST-PH 2.0mm | Common battery/sensor connector | JST-XH 2.5mm | Standard in many modules |
| USB Type-C | Use official footprint — check pad polarity carefully | RJ-45 | Ethernet — usually shielded, check keepout |
| NOTE: Always verify footprints against actual component datasheet dimensions. Manufacturer footprints are NOT always correct in KiCAD libraries — measure a physical part or use IPC-7351 calculator. |  |  |  |

# 4. LAYER STACKUP & CONTROLLED IMPEDANCE

| COMMON LAYER STACKUPS |  |  |  |
| --- | --- | --- | --- |
| 2-LAYER STACKUP (1.6mm total) |  |  |  |
| Layer 1 (Top) | Signal + Components + Power traces | Layer 2 (Bot) | GND plane (pour) + secondary signals |
| Core thickness | 1.6mm FR4, εr = 4.3–4.7 | Copper weight | 1oz (35µm) standard; 2oz for power |
| Trace impedance 2L | 50Ω microstrip: W ≈ 2.8mm on 1.6mm board (1oz) | Limitation | No continuous GND return — avoid for high-speed |
| 4-LAYER STACKUP (1.6mm total) — JLCPCB JLC04161H-3313 |  |  |  |
| Layer 1 (Top) | Signal + Components | Thickness Top | 0.035mm Cu + 0.1mm prepreg |
| Layer 2 (In1) | GND plane (solid — reference for Top signals) | Thickness In1 | 0.035mm Cu + 1.065mm core |
| Layer 3 (In2) | Power planes (VCC, VCORE, split zones) | Thickness In2 | 0.035mm Cu + 0.1mm prepreg |
| Layer 4 (Bot) | Signal + secondary components | Thickness Bot | 0.035mm Cu |
| 50Ω microstrip (top) | W ≈ 0.19mm on 0.1mm prepreg to GND (JLC standard) | 50Ω stripline | W ≈ 0.14mm embedded between GND layers |
| 6-LAYER STACKUP (1.6mm) — HIGH-SPEED / RF |  |  |  |
| L1 Top | Signal (microstrip, ref to L2 GND) | L2 | GND (solid plane) |
| L3 | Signal (stripline, ref L2/L4) | L4 | Power planes |
| L5 | Signal (stripline, ref L4/L6) | L6 Bottom | Signal + GND pour |
| IMPEDANCE FORMULAE |  |  |  |
| Microstrip Z0 | Z0 = 87/√(εr+1.41) × ln(5.98H/(0.8W+T)) | Stripline Z0 | Z0 = 60/√εr × ln(4B/(0.67π(0.8W+T))) |
| W = trace width | H = dielectric height, T = copper thickness | εr FR4 | 4.3–4.7 @ 1GHz (use fab's exact Dk) |
| Differential pair | Zdiff = 2×Z0×(1 - 0.347×e^(-2.9×S/H)) | S = gap | Gap between differential pair traces |
| Common mode Zcc | Zcc = Z0/2 × (1 + 0.347×e^(-2.9×S/H)) | Intra-pair skew | < 5mil mismatch for USB/HDMI/LVDS |
| JLCPCB 4-LAYER CONTROLLED IMPEDANCE (JLC04161H-3313) |  |  |  |
| 50Ω microstrip L1 | W = 0.19mm (7.5 mil) | 50Ω microstrip L4 | W = 0.19mm |
| 50Ω stripline L2 | W = 0.13mm (5.1 mil) | 50Ω stripline L3 | W = 0.13mm |
| 100Ω diff micro L1 | W = 0.19mm, S = 0.2mm | 90Ω diff micro | W = 0.16mm, S = 0.13mm (USB 2.0) |
| NOTE: Always use your fab's impedance calculator with their specific stackup. JLC, PCBWay, AISLER all publish their layer stackup Dk and thickness values. Request impedance test coupon on panel. |  |  |  |

# 5. PCB LAYOUT — COMPONENT PLACEMENT STRATEGY

| PLACEMENT ORDER & HIERARCHY |  |  |  |
| --- | --- | --- | --- |
| PLACEMENT SEQUENCE (always follow this order) |  |  |  |
| 1. Connectors | Fix first — constrained by mechanical enclosure / mating | 2. Crystals/OSC | Near MCU clock pins, away from switching noise |
| 3. Power ICs | Regulators, converters — near input connector | 4. MCU / FPGA | Central, decoupling caps immediately adjacent |
| 5. Signal ICs | ADC, DAC, amplifiers — near their sensor/load | 6. Passives | Fill in near their associated active component |
| 7. Bulk capacitors | Near power entry and regulator outputs | 8. Test points | Place LAST — fill spare space, accessible locations |
| CRITICAL PLACEMENT RULES |  |  |  |
| Decoupling caps | < 0.5mm from VCC pin, on SAME side as IC if possible | Crystal | < 10mm from MCU pins, symmetric layout, GND guard ring |
| Diff pair components | Router, USB, HDMI — place so pairs can route parallel | Reset filter | R+C near MCU reset pin — not at connector |
| Ferrite beads | In series on PWR traces between digital and analogue | ESD protection | At connector entry — first thing signal sees |
| TVS / protection | < 5mm from connector pin to TVS to GND | Fuse placement | On positive rail, before regulator, near input |
| THERMAL PLACEMENT |  |  |  |
| Hot components | Away from temperature-sensitive parts (oscillators, ADC) | Airflow | Orient components along airflow direction (forced) |
| Power dissipation | Pd = (Vin-Vout) × Iout for LDO — use switcher if Pd>1W | Thermal relief | Use spoke thermal connects on THT pads in GND plane |
| Heatsink clearance | ≥ 3mm to adjacent components (or per heatsink spec) | GND via array | Below thermal pad of QFN/DFN — 0.3mm vias, 0.8mm grid |
| MECHANICAL CONSTRAINTS |  |  |  |
| Mounting holes | Keep clearance: 3mm from hole edge to nearest trace | Board edge | Keep 0.2mm min (JLC), prefer 0.5mm from traces/pads |
| SMD near edge | ≥ 0.5mm from board edge for SMD pads | THT near edge | ≥ 1mm from board edge for THT pads |
| Assembly clearance | Leave 5mm around board edge for conveyor grip | Panelisation | V-score: 0.5mm groove each side; break-away 0.3mm holes |
| NOTE: Placement drives 80% of the board quality. Good placement = easy routing. Bad placement = impossible routing or EMC failures. Spend at least 50% of layout time on placement. |  |  |  |

# 6. ROUTING TECHNIQUES & TRACE GEOMETRY

| TRACE WIDTH vs CURRENT CAPACITY (IPC-2221) |  |  |  |
| --- | --- | --- | --- |
| EXTERNAL TRACES (1oz copper, 10°C rise) |  |  |  |
| 100 mA | 0.10mm (4mil) minimum | 500 mA | 0.25mm (10mil) |
| 1 A | 0.40mm (16mil) | 2 A | 0.80mm (31mil) |
| 3 A | 1.20mm (47mil) | 5 A | 2.00mm (79mil) |
| 10 A | 4.00mm (157mil) | IPC formula | W = (I / (k × ΔT^0.44))^(1/0.725) / (1.378×T^0.5) |
| TRACE WIDTH QUICK RULES |  |  |  |
| Signal traces | 0.15mm (6mil) min for production, 0.2mm preferred | Power traces | Calculate per IPC-2221, + 50% safety margin |
| GND fill | Full copper pour — no width limit | High voltage | 1mm clearance per 100V (conservative, check standards) |
| USB D+/D- (90Ω) | W = 0.16mm, Gap = 0.13mm (check fab stackup!) | Ethernet diff | 100Ω: W depends on stackup — use fab calculator |
| ROUTING RULES & GEOMETRY |  |  |  |
| 45° rule | Use 45° bends, not 90° — reduces EMI/reflections | Curved routing | Smooth arcs — best for RF and high-speed signals |
| Avoid acute angles | < 45° bends trap acid in etch — design rule violation | Return current | Route signal and its return path close together |
| No daisy chains | Star topology for sensitive signals (CLK distribution) | T-junctions | Avoid for high-speed — cause reflections |
| Stub avoidance | Unterminated stubs create resonant reflections | Via placement | Via changes layer — breaks return path continuity |
| VIA TYPES & SPECIFICATIONS |  |  |  |
| Through-via | Drills entire board — cheapest, most common | Blind via | From outer layer to inner layer only (expensive) |
| Buried via | Inner layers only — highest density, expensive | Micro-via | 0.1mm drill, laser — for HDI/BGA escape routing |
| Via size (JLC std) | Drill: 0.2mm min (0.3mm preferred), pad: 0.4mm | Via-in-pad | Fill + plate for BGA/QFN thermal vias |
| Via aspect ratio | Board thickness / drill diameter < 10:1 for mechanical | Tented via | Covered with solder mask — prevents solder wicking |
| RETURN CURRENT & GROUND PLANE |  |  |  |
| Return current rule | Return current follows path of LEAST IMPEDANCE (at DC: least resistance; at AC: least inductance = directly under signal trace) |  |  |
| Plane slot avoidance | Never route signal traces across splits or slots in GND plane — return current must detour → creates EMI loop |  |  |
| Layer change via | When changing layers: place GND via adjacent to signal via to maintain return path continuity |  |  |
| Moat around crystal | Flood GND around crystal on all layers, stitch with vias every 1mm, no other traces through crystal keepout |  |  |
| NOTE: Route from most critical signals first: clocks, diff pairs, high-frequency signals. Leave power and low-speed signals for last. Never compromise signal integrity for routing convenience. |  |  |  |

# 7. SIGNAL INTEGRITY & HIGH-SPEED DESIGN

| SIGNAL INTEGRITY FUNDAMENTALS |  |  |  |
| --- | --- | --- | --- |
| Propagation velocity | v = c / √εr ≈ 0.6c on FR4 (εr≈4.4) | Delay per inch | ≈ 150 ps/inch on FR4 (6 ps/mm) |
| Critical length | Lcrit = Tr / (2 × Tpd) — where Tr = rise time | Termination | Required when trace length > Lcrit |
| Reflection coeff Γ | Γ = (ZL - Z0) / (ZL + Z0) | Series term. | Rs = Z0 - Rdriver (place near source) |
| Parallel term. | Rterm = Z0 (place at load end) | AC termination | R+C: R=Z0, C=100pF (blocks DC, terminates AC) |
| Crosstalk NEXT | Near-end: aggressor energy couples backward | FEXT | Far-end: couples to victim at load end |
| 3W rule | Space traces 3× trace width (edge-to-edge) to reduce XT | H rule | Keep trace height H above reference plane constant |
| DIFFERENTIAL PAIR ROUTING |  |  |  |
| Match length | Intra-pair length mismatch < 5mil (0.127mm) for USB | Routing style | Route as pair — keep together from source to load |
| Coupling length | Pairs must be coupled (close) for full differential benefit | Symmetry | Symmetric layout — same vias, same layer changes |
| USB 2.0 | 90Ω ±15%, trace length matched ±100ps | USB 3.0 | 90Ω, trace matched ±5ps, additional SSC consideration |
| LVDS pair | 100Ω differential, couple ≥ 80% of trace length | HDMI pair | 100Ω, 7 differential pairs + HPD + DDC (I2C) + 5V |
| DECOUPLING STRATEGY (DETAIL) |  |  |  |
| Rule 1 | 100nF on EVERY VCC pin of EVERY IC — no exceptions | Rule 2 | Value: 100nF for logic, 1µF for analogue/power ICs |
| Rule 3 | Place as close to VCC pin as possible — direct connection | Rule 4 | GND side: via directly to GND plane (not long trace) |
| Rule 5 | 10µF–100µF bulk: near regulator output or board entry | Rule 6 | Use X7R/X5R dielectric — NOT Y5V/Z5U (capacitance drops) |
| PDN impedance | Target < 10mΩ across frequency band of interest | Resonance | Place caps with different values to fill frequency gaps |
| CLOCK SIGNAL ROUTING |  |  |  |
| Series resistor | 22–33Ω series R at source — reduces ringing and EMI | Load cap | Each receiver ≈ 5–15pF — keep total load low |
| Point-to-point | Single source to single load — no branches | Daisy chain clk | Only if using differential LVDS/LVPECL |
| Clock buffer | Use for >2 loads — fan-out buffer (e.g. SY100EL16) | Clock distribution | Star topology from buffer to all receivers |
| Guard traces | GND traces adjacent to clock trace on same layer | GND vias | Stitch vias alongside clock trace every 5mm |
| NOTE: Signal integrity is physics — it cannot be fixed by firmware. Always simulate critical nets with HyperLynx/QUCS/Sigrity or use rule-based checking (rise time vs trace length) before layout is finalized. |  |  |  |

# 8. POWER DISTRIBUTION NETWORK (PDN) & PLANES

| POWER PLANE DESIGN |  |  |  |
| --- | --- | --- | --- |
| PLANE vs TRACE POWER DISTRIBUTION |  |  |  |
| Use plane when | Supply current > 3A, or many loads distributed across board | Use trace when | < 1A, localized load, thin board or 2-layer |
| Split planes | Split inner plane into power domains (VCC/VCORE/VDDA) | Split rule | Never route signal traces across a plane split |
| Plane capacitance | Cpln = ε0 × εr × A / d — inner layer pair is a capacitor | Benefit | PDN capacitance: ~100pF/cm² for 0.1mm dielectric |
| Plane inductance | Very low inductance — much better than wide traces | GND plane | Dedicate at least ONE full layer to solid GND |
| VOLTAGE REGULATOR PLACEMENT & LAYOUT |  |  |  |
| LDO layout | Vin cap → LDO → Vout cap → load (linear path) | LDO feedback R | Feedback resistors: short traces, close to VOUT pin |
| Buck converter | Input cap → Switch → Inductor → Output cap (hot loop small! | Hot loop | Vin cap, switch FETs, inductor in tight triangle |
| Boost converter | GND→Inductor→Switch→Diode→Output cap → load | CCM vs DCM | CCM: lower ripple; DCM: enters at light load |
| Switcher GND | Separate switcher GND to star point — away from analogue | EMI reduction | Snubber RC across diode; shield inductor; spread spectrum |
| CURRENT PATH & HOT LOOP MINIMISATION |  |  |  |
| Hot loop | The loop formed by high dI/dt switching — minimize its area: Input Cap → MOSFET High → MOSFET Low → Input Cap GND. Keep all three as tight triangle. |  |  |
| L = µ0 × A / h | Loop inductance proportional to area A — halve loop area → halve inductance → halve switching voltage spike |  |  |
| Via inductance | Each via ≈ 0.5–1nH — use multiple vias in parallel for high-current paths (I > 1A per via is excessive) |  |  |
| PLANE STITCHING VIAS |  |  |  |
| GND stitch spacing | Every λ/20 at highest frequency (< 5mm for 1GHz) | Around signals | Row of GND vias alongside differential pairs |
| Layer transition | GND via next to every signal via — maintain return path | Board edge | Ring of GND vias 1mm inside board perimeter |
| Power stitching | Multiple vias for power layer connections (≥ 4 vias/A) | Thermal vias | 0.3mm drill, 0.2mm wall, array under thermal pads |
| NOTE: PDN design: target < 10mΩ impedance at the IC's power pins from DC to 1GHz. Use a PDN analyzer tool (Altium PDN Analyzer, SIwave) for complex designs. Rule of thumb: 1 decap per 100MHz of bandwidth. |  |  |  |

# 9. EMC / EMI PCB DESIGN RULES

| EMC DESIGN FUNDAMENTALS |  |  |  |
| --- | --- | --- | --- |
| EMI source | Any fast-switching signal (clock, PWM, switcher) | EMI coupling | Radiated (electric/magnetic loop) or conducted |
| Electric field (E) | Driven by high dV/dt — reduce by termination, shielding | Magnetic (H) | Loop area × current — minimize switching loop area |
| Rise time vs freq | Highest harmonic ≈ 0.35 / Tr (Tr = 10-90% rise time) | 3ns rise time | ≈ 117MHz content → needs SI/EMC treatment above 100MHz |
| EMC LAYOUT TECHNIQUES |  |  |  |
| GND plane | Solid GND plane is the single biggest EMC improvement | No slot in GND | Every slot or gap disrupts return currents → radiates |
| Partition board | Analogue section ¦ Digital section ¦ Power section — separate | Single star GND | Connect partitions at single low-impedance point |
| Filter at entry | Every cable/connector: ferrite + TVS + CM choke at entry | Pi-filter | C → L → C on power entry — handles both CM and DM |
| Spread spectrum | Enable in MCU/switcher — spreads EMI over wider frequency | Shielding can | Metal shield over RF section, gasketed if needed |
| Guard ring | Driven GND ring around sensitive analogue section | Keepout zone | Exclude switchers/clock from analogue section |
| COMMON MODE & DIFFERENTIAL MODE FILTERING |  |  |  |
| Common mode noise | Both conductors swing together relative to GND | DM noise | Conductors swing opposite — normal signal mode |
| CM choke | High impedance to CM, transparent to DM | CM cap (Cy) | Line-to-GND: 1–10nF (safety rated Y1/Y2 for mains) |
| DM inductor | Series on each line — with Cx cap for pi-filter | Cx cap | Line-to-line capacitor: safety rated X1/X2 for mains |
| Ferrite bead | Resistive at high frequency — not a pure inductor | Ferrite select | ZF at 100MHz > 600Ω for digital; plot impedance vs freq |
| PCB SHIELDING & GROUNDING FOR EMC |  |  |  |
| Chassis GND | Connect PCB GND to chassis at ONE point (star GND) | Strap bond | Low-impedance strap < λ/20 at highest frequency |
| Slot antenna | Exposed signals on board edge radiate like slot antennas | Mitigation | Keep signals 3mm from board edge; add GND pour |
| Cable shield | Terminate cable shield at board entry via low-Z cap/direct | Pigtail | < 25mm pigtail — longer pigtail = antenna above 100MHz |
| EMC STANDARDS (SA / International) |  |  |  |
| CISPR 32 | Multimedia equipment emissions (replaces EN 55032) | CISPR 35 | Multimedia equipment immunity |
| IEC 61000-4-2 | ESD immunity: ±2kV contact, ±4kV air | IEC 61000-4-4 | EFT (fast transient): ±2kV on power lines |
| IEC 61000-4-5 | Surge: ±2kV L-L, ±4kV L-GND | ICASA / SABS | SA: ICASA type approval required for wireless products |
| NOTE: Get EMC right in layout — post-layout EMC fixes (ferrites, caps) add cost and complexity. A solid GND plane + proper partitioning + decoupling solves 80% of EMC issues before testing. |  |  |  |

# 10. DRC / DFM / MANUFACTURING RULES

| DESIGN RULE CHECK (DRC) PARAMETERS |  |  |  |
| --- | --- | --- | --- |
| JLCPCB STANDARD PROCESS MINIMUMS |  |  |  |
| Min trace width | 0.09mm (3.5mil) — prefer 0.127mm (5mil) | Min clearance | 0.09mm (3.5mil) trace-to-trace; prefer 0.127mm |
| Min drill (mech) | 0.2mm drill, 0.4mm pad — prefer 0.3mm drill | Min drill (laser) | 0.1mm micro-via |
| Min annular ring | 0.13mm (5mil) — outer layer | Min hole/pad | Pad = drill + 2×annular ring + 2×fab tolerance |
| Soldermask web | 0.1mm between adjacent SMD pads (preferred 0.15mm) | Silkscreen width | 0.1mm min line width; 0.8mm min text height |
| Board edge clearance | 0.2mm min (trace/copper to board edge) | V-score | Material remaining: 0.4mm (JLC standard V-score) |
| DESIGN FOR MANUFACTURE (DFM) CHECKLIST |  |  |  |
| Fiducial marks | 3× minimum on PCB for pick & place alignment | Fiducial size | 1mm Cu circle, 3mm solder-mask clearance, on top copper |
| Tooling holes | 2× 2.0mm or 3.0mm (NPTH) for PCB holding during reflow | Paste coverage | Stencil thickness 0.12mm for 0402, 0.15mm for 0603+ |
| Assembly polarity | Polarity marker on every polarised part in silkscreen | Pin 1 marker | Triangle or dot on IC silkscreen — must be visible |
| Part clearance | ≥ 0.3mm between adjacent SMD parts (reflow) | THT clearance | ≥ 1mm between THT parts; solder side clearance for wave |
| Solder bridge risk | Fine-pitch (<0.65mm) needs solder mask between pads | Tomb-stoning | Match pad size/length exactly for small passives |
| DESIGN FOR ASSEMBLY (DFA) BEST PRACTICES |  |  |  |
| One-sided SMD | Reduce cost — all SMD on top, THT on bottom if possible | Consistent orient | Orient all passives same direction — speeds visual check |
| Avoid bottom SMD | Saves second reflow pass — significant cost saving | Large parts last | Tall parts (caps, connectors) placed after small SMD |
| Thermal relief | THT pads in GND pour — use 4-spoke thermal relief | SMD in pour | Remove thermal relief for SMD pads in pour (better solder) |
| GERBER OUTPUT CHECKLIST |  |  |  |
| Gerber layers | F.Cu (GTL) ¦ B.Cu (GBL) ¦ In1.Cu (G1) ¦ In2.Cu (G2) ¦ F.Paste (GTP) ¦ B.Paste (GBP) ¦ F.Silkscreen (GTO) ¦ B.Silkscreen (GBO) ¦ F.Mask (GTS) ¦ B.Mask (GBS) ¦ Edge.Cuts (GKO) |  |  |
| Drill files | PTH drill (DRL) + NPTH drill (DRL) + Drill map (PDF or DXF) — use Excellon format |  |  |
| Assembly files | Centroid/CPL (CSV: reference, x, y, rotation, side) + BOM (CSV: ref, qty, value, MPN, footprint) |  |  |
| NOTE: Always visually inspect Gerber files in a viewer (KiCAD Gerber viewer, Gerber Panelizer, GerbView) before ordering. Check drill alignment, silkscreen completeness, and mask openings. |  |  |  |

# 11. KICAD — INTERFACE & GLOBAL WORKFLOW

| KICAD 7/8 PROJECT STRUCTURE |  |  |  |
| --- | --- | --- | --- |
| *.kicad_pro | Project file — links all files together | *.kicad_sch | Schematic (main sheet or top-level hierarchy) |
| *.kicad_pcb | PCB layout file | *.kicad_sym | Symbol library file |
| *.kicad_mod | Footprint library file (folder = .pretty) | fp-info-cache | Footprint info cache — regenerate if corrupt |
| sym-lib-table | Symbol library table — maps library names to paths | fp-lib-table | Footprint library table — same for footprints |
| Backups/ | Auto-backup folder — KiCAD saves every N minutes | *.kicad_wks | Page layout (title block) worksheet file |
| PROJECT WORKFLOW IN KICAD |  |  |  |
| 1. New project | File → New Project → creates .kicad_pro + blank .kicad_sch + blank .kicad_pcb |  |  |
| 2. Draw schematic | Open Eeschema (schematic editor) → add symbols → draw wires → assign footprints → run ERC |  |  |
| 3. Sync to PCB | In Eeschema: Tools → Update PCB from Schematic (F8) → Review changes → Update |  |  |
| 4. Layout | Open Pcbnew → set board outline → place components → set design rules → route → fill zones → DRC |  |  |
| 5. Output | File → Fabrication Outputs → Gerbers, Drill Files, BOM, PnP (centroid) |  |  |
| KICAD SETTINGS TO CONFIGURE FIRST |  |  |  |
| Preferences → Libraries | Set global symbol & footprint library paths | Scripting console | View → Scripting Console — Python 3 REPL for automation |
| Origin point | Place → Grid Origin at corner of board — sets (0,0) | Grid settings | Fine: 0.1mm or 0.05mm; Coarse: 1mm; use G key to toggle |
| Design rules | File → Board Setup → Design Rules — set ALL clearances | Net classes | Board Setup → Net Classes — assign width/clearance per net |
| Colour scheme | Preferences → Manage Colour Themes — import community theme | Interactive router | Settings → Interactive Router: Highlight Mode, Push & Shove |
| NOTE: Always use a project-local library (MyProject.kicad_sym / MyProject.pretty) for custom symbols and footprints. Never edit global libraries — updates will overwrite your changes. |  |  |  |

# 12. KICAD SCHEMATIC EDITOR — SHORTCUTS & TRICKS

| Shortcut | Action | Context |
| --- | --- | --- |
| NAVIGATION |  |  |
| Scroll wheel | Zoom in/out | Schematic |
| Middle click drag | Pan view | Schematic |
| Ctrl+Home | Fit entire schematic in view | Schematic |
| F | Zoom to fit selection | Schematic |
| ADDING & EDITING COMPONENTS |  |  |
| A | Add symbol (opens symbol chooser) | Schematic |
| P | Add power symbol (VCC, GND, etc.) | Schematic |
| W | Draw wire | Schematic |
| Q | End wire / end drawing action | Schematic |
| L | Add net label | Schematic |
| E | Edit properties of selected item | Schematic |
| R | Rotate selected item (90° CCW) | Schematic |
| X | Mirror horizontally (flip X) | Schematic |
| Y | Mirror vertically (flip Y) | Schematic |
| G | Grab item (drag while maintaining connections) | Schematic |
| M | Move item (disconnect from wires) | Schematic |
| Del | Delete selected item | Schematic |
| Ctrl+D | Duplicate selected item | Schematic |
| Ctrl+Z / Ctrl+Y | Undo / Redo | Schematic |
| ANNOTATION & TOOLS |  |  |
| T | Add text / label annotation | Schematic |
| K | No Connect flag (X marker on pin) | Schematic |
| J | Add junction (dot where wires cross and connect) | Schematic |
| Ctrl+A | Annotate schematic (auto-number all components) | Tools menu |
| Ctrl+Shift+A | Clear annotation (reset all reference designators) | Tools menu |
| Ctrl+L | Edit symbol fields (mass edit all component properties) | Tools menu |
| Ctrl+F | Find / search for component or net | Schematic |
| F8 | Update PCB from Schematic (sync to layout) | Tools menu |
| Ctrl+R | Run ERC (Electrical Rules Check) | Tools menu |
| HIERARCHY & SHEETS |  |  |
| S | Add hierarchical sheet (sub-schematic block) | Schematic |
| I | Add hierarchical label (port on sub-sheet) | Schematic |
| Ctrl+click | Navigate into/out of hierarchical sheet | Schematic |
| Ctrl+Shift+U | Highlight net (highlight all connected wires & pins) | Schematic |

KiCAD Schematic — Pro Tips & Tricks

// TIP 1: Net Inspector — Inspect → Net Inspector

// Shows every net, its connections, and flags disconnected nets

// TIP 2: Symbol from CSV — Use KiCAD's symbol generator scripts

// Tools → Edit Symbol → Import from Datasheet PIN table (KiCAD 8)

// TIP 3: Bus notation for multi-bit signals

// Draw bus line, label: DATA[0..7] — connects DATA0..DATA7 auto

// TIP 4: Global vs Local labels

// Net Label (L) = local to current sheet

// Global Label = visible across ALL sheets in project

// Power Symbol (P) = special global — always use for VCC/GND

// TIP 5: Assign footprints BEFORE layout — use footprint browser

// Ctrl+click on component → Properties → Footprint → Browse

// Or use Assign Footprints tool (Tools → Assign Footprints)

// TIP 6: ERC customisation — Tools → ERC → ERC Violations

// Right-click violation → Ignore rule to suppress false positives

// Document WHY you ignored — add schematic note nearby

// TIP 7: Highlight net across PCB and Schematic simultaneously

// Click net name/label → Inspect → Highlight Net

// Works cross-application (both editors open simultaneously)

# 13. KICAD PCB EDITOR (Pcbnew) — SHORTCUTS & TRICKS

| Shortcut | Action | Context |
| --- | --- | --- |
| NAVIGATION |  |  |
| Scroll wheel | Zoom in/out | PCB |
| Middle drag | Pan view | PCB |
| Ctrl+Home | Zoom to fit board | PCB |
| ` (Backtick key) | Toggle ratsnest visibility | PCB |
| Ctrl+backtick | Toggle track and via visibility | PCB |
| ROUTING |  |  |
| X | Start routing trace from pad/via (interactive router) | PCB |
| V | Add via while routing (changes layer + adds via) | PCB |
| / | Switch to next allowed copper layer while routing | PCB |
| W | Change trace width while routing (cycles Net Classes) | PCB |
| U | Expand selection to entire connected track segment | PCB |
| I | Select all tracks in net | PCB |
| Q | End routing / exit routing mode | PCB |
| Backspace | Undo last routing step (shorten trace) | PCB while routing |
| D | Interactive router: drag track (extends/reshapes) | PCB |
| PLACEMENT & EDITING |  |  |
| E | Edit properties of selected item | PCB |
| R | Rotate selected item (use in properties for exact angle) | PCB |
| F | Flip selected item to other side of board | PCB |
| G | Drag item while maintaining connections | PCB |
| M | Move item | PCB |
| T | Add text | PCB |
| Ctrl+D | Duplicate selected item | PCB |
| Ctrl+G | Group selected items | PCB |
| Ctrl+Shift+G | Ungroup items | PCB |
| ZONES (COPPER POUR) |  |  |
| Z | Draw filled zone (copper pour area) | PCB |
| Ctrl+B | Fill ALL zones (copper pour fill/update) | PCB |
| Ctrl+Shift+B | Un-fill / remove copper pour fill from all zones | PCB |
| E on zone | Edit zone properties (net, clearance, fill, priority) | PCB |
| INSPECTION & TOOLS |  |  |
| Ctrl+Shift+U | Highlight net across PCB and schematic | PCB |
| Ctrl+R | Run DRC (Design Rules Check) | PCB |
| Inspect → Net Inspector | See all nets, lengths, connected components | Menu |
| Inspect → Board Statistics | Track count, via count, pad count summary | Menu |
| Alt+3 | Open 3D viewer (with live sync to PCB changes) | PCB |
| Ctrl+Shift+M | Measure distance between two points | PCB |
| LAYER MANAGEMENT |  |  |
| Layer selector | Top toolbar dropdown — select active layer | PCB |
| + / - | Cycle to next/previous copper layer | PCB |
| Ctrl+Click layer | Set active layer in layer panel | PCB |
| ; (semicolon) | Switch to component layer (auto: top or bottom based on footprint) | PCB |

# 14. KICAD — EFFICIENCY TRICKS & ADVANCED FEATURES

| KICAD POWER USER TECHNIQUES |  |  |  |
| --- | --- | --- | --- |
| INTERACTIVE ROUTER MODES |  |  |  |
| Push & Shove | Router pushes existing traces out of way (best mode) | Walk Around | Router avoids existing traces — does not move them |
| Highlight Collision | Shows DRC errors in red as you route — visual feedback | Enable mode | Route → Interactive Router Settings → Route Mode |
| Lock track | Lock track → router cannot move it in Push & Shove | Track locking | Select track → right-click → Lock |
| Diff pair router | Route → Route Differential Pair (D+D-) as matched pair | Dp skew tuning | Route → Tune Differential Pair Skew (length matching) |
| Length tuning | Route → Tune Track Length → add meanders to match length | Accordion | Meander style: adds serpentine section for length matching |
| DESIGN RULE MANAGER |  |  |  |
| Net class rules | Board Setup → Net Classes → Add Class → set Width/Clearance/Via Size per net class (assign nets in class) |  |  |
| Custom DRC rule | (rule high_speed_clearance (condition "A.NetClass == 'HS'") (constraint clearance (min 0.3mm))) |  |  |
| Diff pair rule | (rule diff_pair_gap (condition "A.isDiffPair()") (constraint diff_pair_gap (min 0.13mm) (max 0.13mm))) |  |  |
| Via fence rule | (rule no_vias_in_keepout (condition "A.Type == 'Via'") (constraint disallow via)) |  |  |
| FOOTPRINT EDITOR TRICKS |  |  |  |
| Pad numbering | Select multiple pads → right-click → Renumber Pads | Pad properties | Set paste/mask expansion per pad (override global) |
| 3D model | Footprint Properties → 3D Models → Add .step/.wrl file | 3D offset | Adjust X/Y/Z offset and rotation to align model |
| Courtyard check | Tools → Check Footprints → reports missing courtyard | Copy from lib | Footprint Editor → Load Footprint from Library → edit |
| Pattern from array | Edit → Create Array (Ctrl+T) — creates pad/via grid | Mirror pads | Select pads → right-click → Mirror X / Mirror Y |
| SCRIPTING & AUTOMATION |  |  |  |
| Open console | View → Scripting Console → Python 3 REPL (or pcbnew → open from command line) |  |  |
| Enumerate nets | import pcbnew; b=pcbnew.GetBoard(); [print(n.GetNetname()) for n in b.GetNets().values()] |  |  |
| Move component | ref = b.FindFootprintByReference('U1'); ref.SetPosition(pcbnew.FromMM(50, 30)) |  |  |
| Run DRC in script | import pcbnew; drc=pcbnew.WriteDRCReport(b, '/tmp/drc.txt', pcbnew.EDA_UNITS_MILLIMETRES, True) |  |  |
| Set trace width | for t in b.GetTracks(): t.SetWidth(pcbnew.FromMM(0.2)) if t.GetNetname()=='GND' else None |  |  |
| PLUGINS — MUST-HAVE KICAD PLUGINS |  |  |  |
| KiBuzzard | Draw complex labels/logos in PCB silkscreen | Footprint Wizard | Built-in: SMD pads, connectors, QFN patterns |
| Interactive HTML BOM | Clickable BOM linked to board view — easier assembly | PCBWay plugin | Direct Gerber export + ordering from PCBWay |
| KiKit | Panel generation (array boards with mouse bites/V-score) | Freerouter | External Java auto-router (import/export DSN) |
| PCBDraw | Renders PCB to SVG/PNG for documentation | fab_outputs | Automates Gerber + drill + BOM export in one click |
| NOTE: Install plugins via: PCB Editor → Plugin and Content Manager (PCM). KiKit and Interactive HTML BOM are essential for professional workflows. Bookmark kicad.org/download/plugins/ for the full catalogue. |  |  |  |

# 15. KICAD — SYMBOL & FOOTPRINT LIBRARIES

| LIBRARY MANAGEMENT BEST PRACTICES |  |  |  |
| --- | --- | --- | --- |
| LIBRARY TYPES |  |  |  |
| Global library | Available to ALL projects — set in Preferences → Manage Libraries | Project library | Stored in project folder — portable, recommended |
| KiCAD std libs | kicad-symbols (GitHub) — regularly updated | Community libs | SnapEDA, UltraLibrarian, Component Search Engine |
| samacsys plugin | Imports symbols+footprints from SamacSys directly in KiCAD | LCSC | EasyEDA parts → convert via easyeda2kicad tool |
| CREATING A PROJECT LIBRARY |  |  |  |
| Symbol lib | Symbol Editor → File → New Library → Save as MyProject.kicad_sym → add to project lib table |  |  |
| Footprint lib | Footprint Editor → File → New Library → Save as MyProject.pretty → add to fp-lib-table |  |  |
| sym-lib-table | (lib (name "MyProject")(type "KiCad")(uri "${KIPRJMOD}/MyProject.kicad_sym")(options "")(descr "")) |  |  |
| fp-lib-table | (lib (name "MyProject")(type "KiCad")(uri "${KIPRJMOD}/MyProject.pretty")(options "")(descr "")) |  |  |
| SYMBOL CREATION CHECKLIST |  |  |  |
| Pin names | Match EXACTLY to datasheet — case sensitive | Pin numbers | Match datasheet pin numbering — critical for footprint mapping |
| Pin types | Input/Output/Bidirectional/Power In/Power Out/Passive | Hidden pins | VCC/GND supply pins: hide + assign to power net |
| Reference prefix | U for IC, R for resistor, C for cap, etc. | Pin length | 100 mil (2.54mm) standard pin length |
| Body units | Multi-unit for dual/quad op-amps — share power pins | Datasheet link | Add datasheet URL in component properties field |
| FOOTPRINT CREATION CHECKLIST |  |  |  |
| IPC calculator | Use KiCAD built-in IPC-7351 footprint wizard | Courtyard | 0.25mm outside body (production) / 0.5mm (hand solder) |
| Fab layer | Exact body outline from datasheet drawing | Silkscreen | Simplified body + pin 1 dot — must not overlap pads |
| 3D model | Download STEP from manufacturer, align precisely | Check | Footprint Editor → Tools → Check Footprints |
| EASYEDA → KICAD IMPORT |  |  |  |
| easyeda2kicad | pip install easyeda2kicad → easyeda2kicad --lcsc-id C14663 --output MyLib (imports by LCSC part number) |  |  |
| LCSC to KiCAD | Search LCSC part → click EasyEDA symbol → in KiCAD Plugin: EasyEDA Pro Import → converts symbol+footprint+3D |  |  |
| NOTE: ALWAYS verify imported symbols and footprints against actual datasheets. Automated imports often have wrong pin types, missing courtyard, or incorrect 3D orientation. Never use unverified parts in production designs. |  |  |  |

# 16. KICAD — PYTHON SCRIPTING RECIPES

Useful Python Scripts for KiCAD Automation

import pcbnew, os

# ── Get board object ─────────────────────────────────────────────────

b = pcbnew.GetBoard() # from within KiCAD console

b = pcbnew.LoadBoard('/path/to/file.kicad_pcb') # from external script

# ── List all footprints ──────────────────────────────────────────────

for fp in b.GetFootprints():

print(fp.GetReference(), fp.GetValue(), fp.GetPosition())

# ── Find a specific footprint ────────────────────────────────────────

u1 = b.FindFootprintByReference('U1')

print(u1.GetPosition()) # returns (x, y) in IU

# ── Move component to specific XY (mm) ──────────────────────────────

u1.SetPosition(pcbnew.VECTOR2I(pcbnew.FromMM(50.0), pcbnew.FromMM(30.0)))

# ── Set component orientation (degrees) ─────────────────────────────

u1.SetOrientationDegrees(90) # 0, 90, 180, 270

# ── Set trace width for all GND traces ──────────────────────────────

for track in b.GetTracks():

if track.GetNetname() == 'GND':

track.SetWidth(pcbnew.FromMM(0.5))

# ── Add via at position ──────────────────────────────────────────────

via = pcbnew.PCB_VIA(b)

via.SetPosition(pcbnew.VECTOR2I(pcbnew.FromMM(25), pcbnew.FromMM(25)))

via.SetDrill(pcbnew.FromMM(0.3))

via.SetWidth(pcbnew.FromMM(0.6))

via.SetNet(b.FindNet('GND'))

b.Add(via)

# ── Generate Gerbers from script ─────────────────────────────────────

plot_ctrl = pcbnew.PLOT_CONTROLLER(b)

plot_opts = plot_ctrl.GetPlotOptions()

plot_opts.SetOutputDirectory('/output/gerbers/')

plot_opts.SetPlotFrameRef(False)

plot_opts.SetPlotValue(True)

plot_opts.SetPlotReference(True)

layers = [(pcbnew.F_Cu,'F_Cu'),(pcbnew.B_Cu,'B_Cu'),

(pcbnew.F_SilkS,'F_Silkscreen'),(pcbnew.Edge_Cuts,'Edge_Cuts')]

for layer, name in layers:

plot_ctrl.SetLayer(layer)

plot_ctrl.OpenPlotfile(name, pcbnew.PLOT_FORMAT_GERBER, name)

plot_ctrl.PlotLayer()

plot_ctrl.ClosePlot()

# ── Export drill file ────────────────────────────────────────────────

drill_writer = pcbnew.EXCELLON_WRITER(b)

drill_writer.SetOptions(False, True, b.GetDesignSettings().GetAuxOrigin(), True)

drill_writer.CreateDrillandMapFilesSet('/output/gerbers/', True, False)

# ── Save board ───────────────────────────────────────────────────────

b.Save(b.GetFileName())

pcbnew.Refresh() # refresh PCB editor view

KiCAD Custom DRC Rules File (.kicad_dru)

# Place in project root as: project_name.kicad_dru

# Board Setup → Design Rules → Load Rules File

(version 1)

# ── High-speed net class clearance ──────────────────────────────────

(rule hs_clearance

(condition "A.NetClass == 'HighSpeed' && B.NetClass == 'HighSpeed'")

(constraint clearance (min 0.3mm)))

# ── Differential pair gap constraint ─────────────────────────────────

(rule diff_pair

(condition "A.isDiffPair()")

(constraint diff_pair_gap (min 0.127mm) (max 0.127mm))

(constraint diff_pair_uncoupled (max 5mm)))

# ── Crystal keepout — no vias within 3mm ─────────────────────────────

(rule crystal_keepout

(condition "A.Type == 'Via' && B.Reference == 'Y1'")

(constraint clearance (min 3mm)))

# ── Power trace minimum width ─────────────────────────────────────────

(rule power_trace_width

(condition "A.NetClass == 'Power'")

(constraint track_width (min 0.5mm)))

# ── No traces in courtyard of U1 ──────────────────────────────────────

(rule u1_courtyard_keepout

(condition "A.Type == 'Track' && B.Reference == 'U1'")

(constraint courtyard_clearance (min 0.5mm)))

# ── Minimum annular ring ─────────────────────────────────────────────

(rule annular_ring

(constraint annular_width (min 0.15mm)))

# 17. HIGH-SPEED & RF PCB DESIGN RULES

| HIGH-SPEED INTERFACE ROUTING RULES |  |  |  |
| --- | --- | --- | --- |
| USB 2.0 (480Mbps) |  |  |  |
| Impedance | 90Ω ±15% differential | Trace width | W = 0.16mm, Gap S = 0.13mm (verify with fab stackup) |
| Max length | 18 inches (457mm) on PCB from connector to IC | Series resistor | 22–33Ω on each line at source — dampen reflections |
| Routing | Route as tightly coupled pair, no vias if possible | ESD protection | PRTR5V0U2X or similar near connector before series R |
| Crystal | 12MHz or 48MHz — within 10mm of USB controller | Ferrite | 600Ω@100MHz ferrite on VBUS before 100nF bypass cap |
| USB 3.0 / USB 3.1 (5/10 Gbps) |  |  |  |
| Impedance | 85Ω ±10% differential SuperSpeed pairs | Length matching | SuperSpeed pair match: ±1mm intra-pair, ±5mm inter-pair |
| No vias | Vias add capacitive discontinuity — avoid on SS pairs | Layer changes | If via needed: back-drill to remove stub; compensate spacing |
| SMA connector | 50Ω SMA to coax → 100Ω diff pair transition carefully | Crosstalk | ≥ 20mil spacing between TX and RX pair groups |
| DDR SDRAM (DDR3/DDR4) |  |  |  |
| Topology | Fly-by topology (DDR3/4) — not star (DDR2 and earlier) | Termination | On-die termination (ODT) in DDR3/4 — no external R |
| Length match (DQ) | ±25mil within byte lane (DQ to DQS) | Address match | ±50mil for address/command lines |
| Via rule DDR4 | Avoid vias on data lines — back-drill if unavoidable | Impedance DDR3 | 50Ω single-ended, 100Ω differential (DQS pairs) |
| Decoupling DDR | 100nF per VDD/VDDQ pin + 10µF bulk per power domain | Guard vias | Row of GND vias between DDR bus and other circuits |
| RF PCB RULES (Sub-GHz to 6GHz) |  |  |  |
| Microstrip 50Ω | Calc W from εr, H, T using fab stackup data | Ground plane | Solid GND on layer directly below RF trace — no splits |
| RF keepout | No traces, vias, or fills within λ/10 of antenna | Antenna isolation | 20dB min isolation between TX and RX antenna ports |
| Component placement | RF IC → matching network → antenna — linear path | Balun | Place between unbalanced IC and balanced antenna |
| SMA edge launch | Tab on SMA: must connect to GND plane immediately | Via fence | Row of GND vias alongside 50Ω trace every 1/10 λ |
| Component lead length | Keep leads < λ/20 for RF passives (use 0402 or 0201) | NF connector | U.FL/IPEX for internal RF — straight vertical mount |
| ETHERNET (10/100/1000BASE-T) |  |  |  |
| Isolation transformer | Magnetics mandatory — check if built into RJ-45 or separate | Common mode | 1500V isolation in magnetics per IEEE 802.3 |
| PoE protection | TVS + CM choke on data pairs before transformer | Impedance | 100Ω differential for 1000BASE-T pairs |
| Centre tap | Magnetics centre tap connects to 75Ω to chassis GND | Bob Smith term | 75Ω to GND on each pair at ICM — reduces EMI |
| NOTE: For RF designs: always run 3D EM simulation (EMPro, HFSS, Sonnet, or free Scikit-RF+OpenEMS) before taping out. Rules of thumb fail at GHz frequencies — simulation is mandatory. |  |  |  |

# 18. THERMAL MANAGEMENT IN PCB DESIGN

| THERMAL CALCULATIONS |  |  |  |
| --- | --- | --- | --- |
| Junction temp Tj | Tj = Ta + Pd × (θja) [°C] | With heatsink | Tj = Ta + Pd × (θjc + θcs + θsa) |
| θja (junction-air) | IC datasheet — no heatsink, free air | θjc (jctn-case) | IC datasheet — best case (with heatsink) |
| θcs (case-sink) | Thermal interface material: 0.5–2°C/W | θsa (sink-air) | Heatsink spec — function of airflow |
| PCB copper θ | Cu layer: θ ≈ 0.003°C/W per mm² per layer | FR4 θ | FR4: 0.3 W/m·K (poor) — copper spreads heat |
| TIM (thermal paste) | 3–8 W/m·K typical; phase-change 3–5 W/m·K | Thermal pad | Graphite 5–10 W/m·K; indium foil 80 W/m·K |
| PCB THERMAL DESIGN TECHNIQUES |  |  |  |
| Thermal vias | Array of 0.3mm vias under QFN/DFN thermal pad | Via spacing | 0.8–1.0mm grid — fills with solder during reflow |
| Copper pour | Large GND pour on bottom layer — spreads heat laterally | Heavy copper | 2oz (70µm) or 3oz for power stages — doubles Cu thickness |
| Component spacing | Hot components away from oscillators, sensors | Airflow orient | Long axis of hot components parallel to airflow |
| Edge connectors | Connector bodies block airflow — leave gap beside them | Fan placement | Exhaust away from sensitive analogue section |
| QFN / DFN THERMAL PAD DESIGN |  |  |  |
| Exposed pad area | Match PCB pad to IC package thermal pad exactly (from datasheet) — do NOT extend beyond package outline |  |  |
| Paste coverage | Cover 50–70% of thermal pad area with stencil apertures (cross-hatch or 9-circle pattern) |  |  |
| Via fill | Via-in-pad: fill with copper fill, plate over — prevents solder wicking into vias (required for small pads) |  |  |
| Thermal resistance | Rthermal_vias = Rthermal_FR4 / N_vias — each via is parallel thermal path through board |  |  |
| NOTE: Use thermal simulation (Ansys Icepak, SolidWorks Flow, or free tools like OpenFOAM) for high-power designs. For quick checks: online calculators at Analog Devices or TI thermal design tools. |  |  |  |

# 19. KICAD — COMPLETE DESIGN RULES SETUP

| BOARD SETUP → DESIGN RULES (File → Board Setup) |  |  |  |
| --- | --- | --- | --- |
| CONSTRAINTS (minimum values — for JLCPCB standard process) |  |  |  |
| Min clearance | 0.127mm (5mil) — preferred, use 0.09mm absolute minimum | Min track width | 0.127mm (5mil) — use 0.09mm for escape routing only |
| Min via hole | 0.3mm drill (0.6mm pad) for standard; 0.2mm absolute | Min via annular | 0.13mm (5mil) |
| Min uVia hole | 0.1mm (laser) — only for HDI stackup | Min uVia pad | 0.2mm |
| Min hole spacing | 0.254mm drill-to-drill spacing | Min silkscreen | 0.1mm line width; 0.8mm text height |
| Copper to edge | 0.2mm absolute; prefer 0.5mm | Drill to edge | 0.3mm absolute; prefer 0.5mm |
| NET CLASSES SETUP |  |  |  |
| Default class | Width: 0.2mm ¦ Clearance: 0.2mm ¦ Via drill: 0.3mm ¦ Via pad: 0.6mm ¦ µVia drill: 0.1mm ¦ µVia pad: 0.2mm |  |  |
| Power class | Width: 0.5mm ¦ Clearance: 0.3mm ¦ Via drill: 0.4mm ¦ Via pad: 0.8mm — assign VCC, GND, 3V3, 5V nets |  |  |
| HighSpeed class | Width: 0.15mm ¦ Clearance: 0.25mm ¦ Via drill: 0.2mm ¦ Via pad: 0.4mm — assign USB, CLK, SPI nets |  |  |
| DiffPair class | Width: 0.15mm ¦ Clearance: 0.25mm ¦ Gap: 0.127mm — assign USB_D+/D-, ETH pairs |  |  |
| LAYER SETUP |  |  |  |
| Copper layers | 2, 4, 6, 8... — set in Board Setup → Board | Board thickness | 1.6mm standard; 0.8mm, 1.0mm, 2.0mm available at JLC |
| Copper weight | 1oz (35µm) default; 2oz (70µm) for power layers | Dielectric | FR4 standard; Rogers/Taconic for RF > 3GHz |
| Layer naming | F.Cu=Top, B.Cu=Bottom, In1.Cu..InN.Cu for inner layers | Layer purpose | Define in layer name: In1.Cu_GND, In2.Cu_Power |
| SOLDER MASK & PASTE SETTINGS |  |  |  |
| Solder mask clearance | 0.05mm global — increased locally for fine-pitch via tenting | Paste margin | 0.00mm global (match pad exactly) — adjust in footprint |
| Mask min web width | 0.1mm between adjacent pads (JLC minimum) | Via tenting | Tented vias: mask over via (no opening) — check with fab |
| HASL vs ENIG | HASL: cheap, uneven; ENIG: flat, expensive, needed for QFN | OSP | Organic Solderability Preservative — flat, low cost |
| NOTE: Import your fab's design rule file if available (JLC provides .kicad_dru files). Validate with their online DFM checker after Gerber export. Set up net classes BEFORE routing — changing width mid-route is painful. |  |  |  |

# 20. KICAD — GERBER EXPORT & FAB ORDERING

| GERBER EXPORT SETTINGS (KiCAD 7/8) |  |  |  |
| --- | --- | --- | --- |
| PLOT SETTINGS (File → Fabrication Outputs → Gerbers) |  |  |  |
| Format | Gerber X2 (preferred) or RS-274X | Coordinate format | 4:6 (mm) — maximum precision |
| Plot footprint values | ON — for assembly reference | Plot ref desig. | ON — for assembly reference |
| Force plotter units | mm (metric) | Check zones | Enable 'Check zone fills before plotting' |
| Layers to plot | All copper + mask + paste + silkscreen + edge cuts + courtyard | Separate files | One Gerber file per layer (standard practice) |
| LAYERS → FILE MAPPING (JLC naming preference) |  |  |  |
| F.Cu → GTL | Front copper (top layer) | B.Cu → GBL | Back copper (bottom layer) |
| In1.Cu → G1L | Inner copper layer 1 | In2.Cu → G2L | Inner copper layer 2 |
| F.Mask → GTS | Front solder mask | B.Mask → GBS | Back solder mask |
| F.Paste → GTP | Front paste (stencil) | B.Paste → GBP | Back paste |
| F.Silkscreen → GTO | Front silkscreen | B.Silkscreen → GBO | Back silkscreen |
| Edge.Cuts → GKO | Board outline (mechanical) | Drill → DRL | Excellon drill file (PTH + NPTH separate) |
| DRILL FILE EXPORT (File → Fabrication Outputs → Drill Files) |  |  |  |
| Format | Excellon — universally accepted | Units | Millimetres; 4:4 coordinate format |
| Zeros format | Suppress leading zeros | Separate files | PTH (plated) and NPTH (non-plated) separate files |
| Map file | Export drill map as PDF or DXF for visual check | Oval holes | Use Alternate Drilling mode for oval slots |
| ASSEMBLY (PNP) EXPORT |  |  |  |
| Centroid file | File → Fabrication Outputs → Component Placement → CSV → includes: Reference, Value, Footprint, X, Y, Rotation, Side |  |  |
| BOM export | File → Fabrication Outputs → BOM → use kibom plugin or interactive HTML BOM for richer output |  |  |
| JLC rotation fix | Many JLC footprints have 90° or 180° offset — use 'jlcpcb-kicad-footprints' library or correct rotation in centroid CSV |  |  |
| ORDERING CHECKLIST |  |  |  |
| Board dimensions | Verify board size in mm in fab order form | Layer count | Match ordered layers to your design |
| Surface finish | HASL-Pb free (cheapest) / ENIG (for fine-pitch) | Solder mask | Green/Red/Blue/Black — green cheapest and fastest |
| Silkscreen | White on green/black; black on white | Controlled imp. | Request only if design needs it (adds cost) |
| Quantity & stencil | Order stencil with first prototype run | Fab DFM check | Upload Gerbers to JLC/PCBWay DFM tool before confirming |
| NOTE: Always order a 3D print or laser-cut mechanical mockup before ordering PCBs for enclosure-constrained designs. Verify connector positions and mounting hole positions match mechanical drawings. |  |  |  |

# 21. PCB DESIGN GOLDEN RULES & BEST PRACTICES

| THE GOLDEN RULES OF PCB DESIGN |  |  |  |
| --- | --- | --- | --- |
| SCHEMATIC RULES |  |  |  |
| Rule S1 | Schematic is the engineering document. If it is wrong, the PCB is wrong. NEVER modify netlist manually to fix schematic errors. |  |  |
| Rule S2 | Name EVERY signal net. Unnamed nets (Net-(R3-Pad1)) are impossible to debug. Name at minimum: clocks, power rails, data buses, interrupt lines. |  |  |
| Rule S3 | Run ERC and fix ALL errors before opening PCB editor. Warnings must be reviewed and documented — never silently ignored. |  |  |
| Rule S4 | Add decoupling caps in schematic, not as an afterthought. Every IC VCC pin needs one. Include in BOM and assign footprint before layout. |  |  |
| LAYOUT RULES |  |  |  |
| Rule L1 | Current needs a return path. Every signal current comes from somewhere and returns to source. Ensure that path is short and has low impedance. |  |  |
| Rule L2 | Never split a ground plane under a signal trace. The return current MUST have a direct path — splits force long detours and create EMI loops. |  |  |
| Rule L3 | Decoupling capacitors: the shorter the path from cap to VCC pin, the better. Via on GND side of cap goes DIRECTLY to GND plane, not a long trace. |  |  |
| Rule L4 | Switching regulator layout: make the hot loop (Vin cap → high-side FET → low-side FET → input cap GND) as small as physically possible. |  |  |
| Rule L5 | Run DRC and fix ALL errors before generating Gerbers. DRC warnings must be reviewed. A board with DRC errors will almost certainly have a problem. |  |  |
| HIGH-SPEED RULES |  |  |  |
| Rule H1 | When trace length > Tr/(2×Tpd), the trace is a transmission line. Apply termination. On FR4: traces > 25mm at 1Gbps need impedance control. |  |  |
| Rule H2 | Differential pairs: route as a pair from source to load. Never split them. Match intra-pair length to within 5mil. Maintain coupling (close spacing) for full benefit. |  |  |
| Rule H3 | Crystal: place as close as possible to IC pins. GND guard ring on all sides. No other traces through crystal area. Dedicated GND pour, stitched to main GND. |  |  |
| PROCESS RULES |  |  |  |
| Rule P1 | Use version control (Git) for PCB designs. Commit at every milestone: after schematic, after placement, after routing, before Gerber export. |  |  |
| Rule P2 | Document design decisions. Add notes to schematic (text blocks) explaining why: why this value, why this topology, why this layout choice. |  |  |
| Rule P3 | Review with fresh eyes: wait 24h, print the schematic and review on paper, have a colleague review. Errors are invisible when you created them. |  |  |
| Rule P4 | Prototype before production: order 5 prototype boards, assemble and test one. Fix all issues before ordering production quantity. |  |  |
| NOTE: The most expensive PCB mistake is the one that passes your review. The second most expensive is the one that doesn't get reviewed. Always review. Always test. Always document. |  |  |  |

# 22. PCB TESTING, BRING-UP & DEBUGGING

| BOARD BRING-UP SEQUENCE |  |  |  |
| --- | --- | --- | --- |
| BARE BOARD TESTS (before assembly) |  |  |  |
| Visual inspection | Check for physical damage, delamination, mask voids | Continuity test | Every net: no opens, no shorts between different nets |
| Impedance test | TDR or VNA on controlled impedance traces + coupon | Hi-pot test | High-voltage isolation test for mains-connected boards |
| BRING-UP SEQUENCE (after assembly) |  |  |  |
| 1. Current limit | Set bench PSU to expected Imax + 20mA — watch for smoke | 2. Power rails | Check VCC, VCORE, VDDA with DMM before powering MCU |
| 3. Clock verify | Probe crystal / oscillator output with oscilloscope | 4. Reset release | Monitor RESET line — verify it deasserts correctly |
| 5. Comms test | UART loopback → SPI flash ID → I2C scan → CAN/ETH | 6. Peripherals | GPIO toggle → ADC reading → PWM output → ISR |
| 7. Load testing | Full operational load — measure power, check temps | 8. Stress test | Thermal cycling, vibration, EMC pre-compliance test |
| DEBUG TOOLS & TECHNIQUES |  |  |  |
| Oscilloscope probing | Use X10 probe; ground clip as short as possible to TP | Current probe | Hall-effect clamp for supply current measurement |
| Logic analyser | Saleae Logic / PulseView — 8/16ch digital capture | Spectrum analyser | EMI pre-compliance: near-field probe + SA (RSA306B) |
| Thermal camera | FLIR/Seek — identify hot spots during load test | De-solder station | Hot air + tweezers — swap suspect components safely |
| Oscilloscope power | Measure PDN: scope probe across VCC/GND cap during load | Ripple target | < 10mV for digital logic; < 1mV for precision analogue |
| COMMON ASSEMBLY FAULTS & FIXES |  |  |  |
| Solder bridge | Flux + solder wick — wick from pad, not wire | Cold joint | Reflow with hot air + flux — glossy finish = good solder |
| Tomb-stoning | 0402/0603 standing on end — reflow profile / pad mismatch | Missing paste | Reflow with paste manually applied + hot air |
| Part polarity | Check cap (stripe = negative), LED, diode orientation | Wrong part | Remove with hot air, verify MPN, replace and reflow |
| NOTE: Keep a bring-up log: date, what you tested, result, scope screenshots. This is invaluable when debugging issues weeks later or when someone else inherits the board. |  |  |  |

# 23. SPECIAL PCB TYPES — FLEX, RIGID-FLEX, HDI

| FLEX PCB DESIGN RULES |  |  |  |
| --- | --- | --- | --- |
| FLEX PCB MATERIALS & MINIMUMS (IPC-2223) |  |  |  |
| Base material | Polyimide (Kapton): 12.5µm–125µm thick | Copper | 0.5oz (17µm) for dynamic flex; 1oz for static flex |
| Min trace width | 0.1mm (dynamic flex); 0.075mm (static) | Min clearance | 0.1mm minimum trace-to-trace |
| Bend radius | Dynamic: 10× total thickness minimum | Static bend | 6× total thickness minimum |
| No vias in bend | Keep vias and pads ≥ 1mm from bend zone | Teardrop pads | Add teardrops at trace-to-pad transitions — reduces stress |
| Trace orientation | Route traces perpendicular to bend axis in bend zone | Avoid crossing | No trace crossings in bend area — use hatched fill |
| RIGID-FLEX DESIGN (IPC-2223C) |  |  |  |
| Stiffener | FR4 or polyimide stiffener in connector area for strength | Coverlay | Adhesive polyimide overlay — replaces solder mask on flex |
| Via-in-pad | Plated flex vias must be encapsulated — no bare vias | Layer transition | Stepped board edge where rigid meets flex |
| ZIF connector | ZIF (Zero Insertion Force) on flex tail — spec insertion cycles | Tail thickness | Match ZIF connector specification exactly |
| HDI (HIGH DENSITY INTERCONNECT) |  |  |  |
| Micro-vias | 0.1mm laser drill — only connects adjacent layers | Via stacking | Stacked micro-vias (costly) vs staggered (cheaper) |
| Capture pad | 0.2mm pad for 0.1mm micro-via — very tight | ELIC | Every Layer Interconnect — micro-vias on every layer |
| BGA escape | Use micro-vias for fine-pitch BGA (< 0.8mm pitch) escape | Dog-bone | Via behind pad with trace to via — for 1mm+ BGA pitch |
| ALUMINIUM / METAL CORE PCB (MCPCB) |  |  |  |
| Use case | LED lighting, high-power modules — better thermal | Thermal cond. | 1–3 W/m·K for MCPCB vs 0.3 W/m·K for FR4 |
| Dielectric layer | Thin dielectric between copper and Al — 50–150µm | Single layer | Usually 1-layer copper on top of Al substrate |
| Design rules | Min 0.2mm clearance; no plated holes to Al core | Cutouts | Punched, not drilled — check with manufacturer |
| NOTE: Flex PCB design requires flex-experienced fabricator. Always confirm bend radius, dynamic vs static flex, and stiffener requirements with your fab BEFORE starting layout. Mistakes in flex design are expensive. |  |  |  |

# 24. PCB COST REDUCTION STRATEGIES

| PCB FABRICATION COST REDUCTION |  |  |  |
| --- | --- | --- | --- |
| BOARD DESIGN FOR COST |  |  |  |
| Fewer layers | 2-layer < 4-layer < 6-layer — halve layers where possible | Standard size | JLC cheapest: 100×100mm or smaller; use full panel |
| Standard trace | Avoid 0.09mm traces — use 0.127mm+ to stay in std process | Standard via | 0.3mm drill / 0.6mm pad — avoid micro-via premium |
| HASL finish | Cheaper than ENIG — use only if fine-pitch parts absent | 2-layer for GND | Pour GND on bottom layer — avoids 4-layer premium |
| Board panelisation | Fit multiple designs on one panel — amortise setup cost | V-score panel | JLC panels: max 500×500mm — fit as many as possible |
| ASSEMBLY COST REDUCTION |  |  |  |
| Single-side SMD | All SMD on top — one reflow pass, cheaper PCBA | Avoid BGA | BGAs require X-ray inspection — adds assembly cost |
| Standard pitch | ≥ 0.65mm pitch parts: cheaper to assemble than 0.4mm | Larger packages | SOIC > TSSOP > QFN for assembly cost, ease |
| JLCPCB Basic parts | Use 'Basic' parts from LCSC — no extended part handling fee | Extended parts | +$3/type for non-basic parts — minimise unique extended |
| Combine BOM | Same value/footprint = same part — reduces part types | Standard values | E12 or E24 series values — always in stock |
| COMPONENT COST REDUCTION |  |  |  |
| LCSC / JLCPCB | Chinese marketplace — fraction of Mouser/Digi-Key price | JLCPCB SMT | Best value for small volumes < 100 boards |
| Octopart search | Price comparison across all distributors | EOL avoidance | Check last-buy date + stock level during design |
| Alternative MPN | List 2 alternative MPNs per critical component | DNP parts | Design Now, Place Never — include in BOM, skip in order |
| NOTE: Cheapest design = fewest layers, all-top-side SMD, 0402/0603 standard packages, LCSC basic parts, board size ≤ 100×100mm. Cost optimise AFTER design is functionally correct, not before. |  |  |  |

# 25. QUICK REFERENCE — UNITS, STANDARDS & CONSTANTS

| PCB UNITS & CONVERSION |  |  |  |
| --- | --- | --- | --- |
| 1 mil (thou) | 0.0254 mm (1/1000 inch) | 1 mm | 39.37 mil |
| 1 inch | 25.4 mm = 1000 mil | IU (internal) | KiCAD: 1 IU = 1 nm (10⁻⁶ mm) |
| pcbnew.FromMM(x) | Converts mm to KiCAD IU | pcbnew.ToMM(x) | Converts KiCAD IU to mm |
| STANDARD TRACE WIDTHS (reference) |  |  |  |
| Signal (default) | 0.15–0.2mm (6–8mil) | Power (1A) | 0.4mm (16mil) minimum |
| Power (2A) | 0.8mm (31mil) | Power (5A) | 2.0mm (79mil) |
| USB D+/D- (90Ω) | 0.16mm — check fab stackup | 50Ω microstrip | Depends on stackup — use calculator |
| KEY IPC STANDARDS REFERENCE |  |  |  |
| IPC-2221B | Generic PCB design standard | IPC-2222 | Rigid organic PCB design |
| IPC-2223C | Flexible/Rigid-flex design | IPC-7351C | Land pattern and footprint standard |
| IPC-A-610 | Acceptability of electronic assemblies | IPC-6012F | Qualification of rigid PCBs |
| IPC-2141C | Controlled impedance design | J-STD-001 | Soldering requirements |
| MATERIAL PROPERTIES (FR4) |  |  |  |
| Dielectric constant | εr = 4.3–4.7 @ 1GHz (varies with frequency) | Loss tangent | tanδ = 0.018–0.020 @ 1GHz |
| Tg (glass transition) | Tg = 130°C (standard); 170°C (high-Tg) | Td (decomp.) | Td > 300°C (FR4) |
| Thermal cond. | 0.3 W/m·K (poor) — use MCPCB or vias for heat spreading | CTE | 17 ppm/°C in-plane; 55 ppm/°C Z-axis |
| ELECTROMAGNETIC CONSTANTS (PCB) |  |  |  |
| Speed in FR4 | v = c/√εr ≈ 1.45×10⁸ m/s ≈ 0.48c | Delay per mm | ≈ 6.9 ps/mm on FR4 (εr=4.4) |
| λ at 1GHz in FR4 | λ = v/f ≈ 145mm (λ/10 = 14.5mm) | λ at 2.4GHz | λ ≈ 60mm in FR4 (λ/10 = 6mm antenna keepout) |
| Skin depth in Cu | δ = 1/√(πfµσ) — 2.1µm at 1GHz; 0.66µm at 10GHz | 1oz Cu @ 1GHz | 35µm >> skin depth → full 1oz participates at GHz |
| NOTE: For SA fabrication: PCBWay, JLCPCB, and Elmatica all ship to South Africa. JLCPCB offers DDP (Delivered Duty Paid) — no import surprises. Factor 2–3 week lead time + shipping for prototypes. |  |  |  |

# 26. STM32H755ZIT6 — POWER WIRING, DECOUPLING & PROGRAMMING

| STM32H755ZIT6 — LQFP144 OVERVIEW |  |  |  |
| --- | --- | --- | --- |
| Package | LQFP144 — 144-pin, 0.5mm pitch, 20×20mm body | Cores | Cortex-M7 @ 480MHz + Cortex-M4 @ 240MHz |
| Flash | 2MB dual-bank (Bank1 + Bank2, 1MB each) | RAM | 1MB SRAM: DTCM(128K)+ITCM(64K)+AXI(512K)+D2/D3 |
| Supply range | VDD: 1.71–3.6V ¦ VDDA: 1.62–3.6V | Package marking | STM32H755ZIT6 = LQFP144, Temp -40°C to +85°C (I grade) |
| VDD POWER PINS (connect all to +3.3V with 100nF per pin) |  |  |  |
| VBAT — Pin 8 | Backup domain supply: RTC, backup SRAM, LSE osc. | Recommended | Connect to 3.3V with 100nF. Add coin cell via diode if backup needed |
| VDD — Pin 7 | Digital supply (I/O Bank A, partial) | VDD — Pin 13 | Digital supply (I/O Bank A/B) |
| VDD — Pin 19 | Digital supply (I/O Bank B) | VDD — Pin 32 | Digital supply (I/O Bank C) |
| VDD — Pin 42 | Digital supply (I/O Bank C) | VDD — Pin 56 | Digital supply (I/O Bank D) |
| VDD — Pin 71 | Digital supply (I/O Bank E) | VDD — Pin 79 | Digital supply (I/O Bank E/F) |
| VDD — Pin 92 | Digital supply (I/O Bank F/G) | VDD — Pin 106 | Digital supply (I/O Bank G/H, SWD region) |
| VDD — Pin 119 | Digital supply (I/O Bank H/I) | VDD — Pin 129 | Digital supply (I/O Bank I/J) |
| VDD — Pin 144 | Digital supply (I/O Bank J/K) | Total VDD pins | 13× VDD pins — each MUST have 100nF cap to GND |
| USB & ANALOGUE SUPPLY PINS |  |  |  |
| VDD33_USB — Pin 91 | USB 3.3V supply — connects to VDD (+3.3V) rail | Decoupling | 1µF + 100nF directly at pin |
| VDD50_USB — Pin 90 | USB 5V supply — connects to +5V_USB (VBUS/D+ ref) | Decoupling | 1µF + 100nF. Needed even if not using USB |
| VDDA — Pin 36 | Analogue supply for ADC, DAC, PLL, RNG, temperature sensor | Decoupling | 1µF tantalum/electrolytic + 10nF ceramic. Ferrite bead from VDD to VDDA (600R@100MHz) |
| INTERNAL VOLTAGE REGULATOR PINS (VDDCORE system) |  |  |  |
| VDDSMPS — Pin 16 | Input to internal SMPS regulator — connects to +3.3V | Decoupling | 4.7µF + 100nF at pin. SMPS output feeds VDDCORE (1.2V) |
| VDDLDO — Pin 70 | Input to internal LDO — connects to VDDCORE output | VDDLDO — Pin 105 | Same: input to internal LDO, connect to VDDCORE rail |
| VDDLDO — Pin 143 | Input to internal LDO — connects to VDDCORE | VDDCORE rail | ~1.2V generated internally — route to VDDLDO pins only |
| NOTE: VDDSMPS generates VDDCORE (1.2V internal). Route VDDCORE net (from SMPS output) ONLY to VDDLDO pins. Never connect VDDLDO to 3.3V. The SMPS output is NOT exposed as a PCB pin — it is internal to the IC. Use BYPASS mode if SMPS is disabled. |  |  |  |
| VCAP PINS — INTERNAL LDO OUTPUT BYPASS |  |  |  |
| VCAP — Pin 68 | Internal core voltage regulator output — DO NOT connect to external supply | Decoupling | 2.2µF ceramic (X5R/X7R) to GND — exactly as shown in Image 2 |
| VCAP — Pin 103 | Internal core voltage regulator output — bypass cap to GND | Decoupling | 2.2µF ceramic to GND |
| VCAP — Pin 140 | Internal core voltage regulator output — bypass cap to GND | VCAP rule | Each VCAP pin MUST have its own 2.2µF cap. Never share. Never connect together via trace. |
| NOTE: VCAP is the output of the internal LDO/SMPS core regulator (~1.2V). It MUST have 2.2µF ±20% (X5R or better) per pin. DO NOT connect VCAP pins together. DO NOT load VCAP with external circuitry. ST AN4879 is the reference application note for H7 power supply. |  |  |  |
| VSS / GND PINS — ALL MUST CONNECT TO GND |  |  |  |
| VSS — Pin 6 | Digital ground return — primary VSS | VSS — Pin 11 | Digital ground (also referenced as VSS) |
| VSS — Pin 17 | Digital ground | VSS — Pin 27 | Digital ground |
| VSS — Pin 38 | Digital ground | VSS — Pin 50 | Digital ground |
| VSS — Pin 65 | Digital ground | VSS — Pin 76 | Digital ground |
| VSS — Pin 89 | Digital ground | VSS — Pin 100 | Digital ground |
| VSS — Pin 116 | Digital ground | VSS — Pin 127 | Digital ground |
| VSS — Pin 138 | Digital ground | VSSA — Pin 34 | Analogue ground — star-connect to GND at single point |
| VSSMPS — Pin 14 | SMPS ground — connect to GND (same as VSS) | GND rule | All VSS, VSSA, VSSMPS connect to the same GND plane |
| NOTE: VSSA (analogue ground) must connect to the main GND plane but ideally at a single star point away from noisy digital switching. Route VDDA supply through ferrite bead (600Ω@100MHz) from VDD. Route VSSA back to GND via a 0Ω resistor or ferrite bead for the star connection. |  |  |  |

| STM32H755ZIT6 — COMPLETE DECOUPLING TABLE |  |  |  |
| --- | --- | --- | --- |
| DECOUPLING CAPACITOR PLACEMENT RULES |  |  |  |
| Rule 1 — VDD | 100nF X7R 0402 on EVERY VDD pin (13 caps total). Place on same side as IC, as close to pin as physically possible. GND via goes directly to GND plane. |  |  |
| Rule 2 — Bulk | 10µF X5R 0603 per power domain: one near the VDD entry to the MCU section. This handles lower-frequency transients that 100nF cannot. |  |  |
| Rule 3 — VDDA | 10nF + 1µF (tantalum or X5R electrolytic) + ferrite bead (600Ω@100MHz, 500mA) in series from VDD to VDDA. VSSA connect with 0Ω resistor to GND. |  |  |
| Rule 4 — VCAP | 2.2µF ±20% X5R/X7R 0402 per VCAP pin (3 caps total: pins 68, 103, 140). Each cap isolated — no shared traces between VCAP pins. |  |  |
| Rule 5 — VBAT | 100nF + 1µF on VBAT pin 8. If using backup battery: Schottky diode from VDD → VBAT anode, battery to VBAT directly (diode OR topology). |  |  |
| Rule 6 — VDDSMPS | 4.7µF + 100nF on VDDSMPS (pin 16). Add 100nF on each VDDLDO pin (70, 105, 143). SMPS output (VDDCORE) needs 4.7µF near the SMPS inductor (internal — but near the IC). |  |  |
| POWER SUPPLY ARCHITECTURE DECISION |  |  |  |
| SMPS mode | Recommended: lower power, VDDCORE generated by SMPS | LDO mode | Simpler: bypass SMPS, LDO provides VDDCORE. More heat. |
| BYPASS mode | External VDDCORE: supply 1.2V externally to VDDLDO pins | PWR_CR3 reg | Configure SMPS/LDO/BYPASS in firmware: PWR_CR3.SMPSEN |
| SMPS external L | SMPS mode requires 2.2µH inductor between VDDSMPS and VDDCORE (external). Use Murata LQH series or equivalent low-DCR. Current rating ≥ 500mA. |  |  |
| SMPS output cap | VDDCORE: 10µF + 1µF ceramic at SMPS inductor output. Route VDDCORE only to VDDLDO pins — not to anything else. |  |  |
| NOTE: See ST datasheet DS12930 and application note AN4938 (STM32H7 power supply) and AN4879 (hardware guidelines). Use ST's STM32CubeH7 power_supply example project to configure PWR_CR3 for your chosen supply mode. |  |  |  |

| STM32H755ZIT6 — PROGRAMMING & DEBUG PINS (LQFP144) |  |  |  |
| --- | --- | --- | --- |
| SWD (SERIAL WIRE DEBUG) — PRIMARY DEBUG INTERFACE (2 pins) |  |  |  |
| SWDIO — PA13 / Pin 105 | Serial Wire Data I/O — bidirectional data line | Pull | Internal pull-up (50kΩ). External 10kΩ pull-up to VDD recommended |
| SWDCLK — PA14 / Pin 106 | Serial Wire Clock — from programmer to MCU | Pull | Internal pull-down. External 10kΩ pull-down to GND recommended |
| NRST — Pin 25 | Active-low reset input. 100nF cap to GND, 10kΩ to VDD | SWD connector | 10-pin Cortex Debug (Tag-Connect TC2030) or 0.1in header |
| JTAG — FULL DEBUG INTERFACE (5 pins, subset of SWD) |  |  |  |
| JTMS/SWDIO — PA13/105 | Test Mode Select (JTAG) / Data (SWD) | JTCK/SWDCLK — PA14/106 | Test Clock |
| JTDI — PA15 / Pin 107 | Test Data In — JTAG only (not needed for SWD) | JTDO/SWO — PB3/133 | Test Data Out / Trace SWO (printf without UART) |
| NJTRST — PB4 / Pin 134 | JTAG TAP reset — pull up to VDD if not used | SWO / TRACESWO | PB3: connect to programmer SWO pin for ITM printf output |
| BOOT CONFIGURATION PINS |  |  |  |
| BOOT0 — Pin 60 | Boot mode selection: 0=Flash, 1=System memory (bootloader) | Pull-down | 10kΩ pull-down to GND for normal boot from Flash |
| BOOT pin control | In STM32H7: BOOT0 only + BOOT_ADD0/1 option bytes | Bootloader mode | BOOT0 HIGH at reset → enter ROM bootloader (UART/USB DFU/SPI) |
| BOOT0 circuit | BOOT0(pin60)──10kΩ──GND AND BOOT0──Jumper/switch──VDD (for field bootloader access). Alternatively use dedicated BOOT0 pad. |  |  |
| STM32H7 DEBUG CONNECTOR STANDARDS |  |  |  |
| 10-pin Cortex Debug | 2×5, 1.27mm pitch — standard ARM Cortex SWD | Pins | 1:VCC_Target 2:SWDIO 3:GND 4:SWDCLK 5:GND 6:SWO 7:NC 8:NC 9:GND 10:nRESET |
| 20-pin JTAG | 2×10, 2.54mm pitch — full JTAG + SWD | Tag-Connect | TC2030-CTX-NL: no PCB connector needed — uses spring-pin clip |
| 4-pin SWD minimal | VCC, SWDIO, SWDCLK, GND — absolute minimum for debug | TC2030-IDC-NL | Tag-Connect 6-pin: SWD + nRESET + SWO — recommended for production boards |
| PROGRAMMING INTERFACE PULL RESISTORS (always include on PCB) |  |  |  |
| SWDIO pull-up | 10kΩ from SWDIO (PA13/Pin105) to VDD — prevents floating when no programmer attached. DNP if debug only during development. |  |  |
| SWDCLK pull-dn | 10kΩ from SWDCLK (PA14/Pin106) to GND — holds clock low when no programmer attached. |  |  |
| NRST filter | 100nF from NRST (Pin25) to GND — filters ESD/noise on reset line. 10kΩ pull-up to VDD already internal but add external too. |  |  |
| SWO optional | Connect PB3 (Pin133) to programmer SWO pin via 33Ω series resistor. Enables ITM/SWO trace output (printf-style debug without UART). |  |  |
| NOTE: SWD (SWDIO + SWDCLK + GND + VCC) is the minimum viable debug interface. Always include a Tag-Connect TC2030-IDC-NL landing on the PCB — it adds zero cost (6 pads + 3 mounting holes) and eliminates need for a connector in production. |  |  |  |

| RECOMMENDED PROGRAMMERS — STM32H755ZIT6 |  |  |  |
| --- | --- | --- | --- |
| DEVELOPMENT PROGRAMMERS / DEBUGGERS |  |  |  |
| ST-LINK/V3 MINIE | ST official: SWD+JTAG+SWO, USB-C, UART bridge. Best value. | Cost | ~$12 USD — highly recommended for all development work |
| ST-LINK/V3E | Enhanced: SWD+JTAG+SWO+UART, 1.65–5V VCC auto-detect | Cost | On STM32H7 Nucleo boards (detachable) — free with eval board |
| ST-LINK/V3 SET | Full kit: STLINK/V3 + STDC14 cable + adapters | Protocol | SWD 50MHz, JTAG 33MHz, SWO 24MHz, VCP UART 16Mbps |
| J-Link BASE | SEGGER: gold standard, fastest, all protocols | Cost | ~$500 USD (full), ~$18 (EDU — for non-commercial only) |
| J-Link EDU Mini | SEGGER educational licence: SWD+JTAG+SWO, limited commercial | Speed | Up to 50MHz SWD. Best for open-source/student projects |
| CMSIS-DAP / DAPLink | Open standard: supported by PlatformIO, pyOCD, OpenOCD | Cost | < $5 USD clones available. Slower but universal |
| BlackMagicProbe v2.3 | Open-source JTAG/SWD probe with built-in GDB server | Benefit | No OpenOCD needed — GDB connects directly to probe |
| Nucleo-H755ZI-Q | STM32H755 eval board: has ST-LINK/V3E built-in detachable | Benefit | Powers and programs your target via SWD cable — ideal for first bring-up |
| MASS PRODUCTION PROGRAMMERS |  |  |  |
| ST-LINK/V3 MODS | ST module for integration into custom test jigs (SWD) | Interface | SWD + UART. Can be automated via STM32CubeProgrammer CLI |
| STLINK-V3PWR | ST-LINK with power measurement: SWD+power monitoring | Use case | Production test + current consumption verification in one tool |
| Segger J-Link PRO | High-speed production: 25MHz SWD, multi-target, licensing | Automation | Command-line: JLink.exe -CommandFile flash_cmd.txt (scriptable) |
| Segger FlashPro | Gang programmer: programs up to 32 targets simultaneously | Use case | High-volume production line (>10K units) |
| ISSI Cyclone FX | P&E Micro gang programmer: standalone, no PC required | Storage | Stores encrypted image locally — operator just presses button |
| STM32CubeProgrammer | ST official GUI+CLI: SWD/JTAG/UART/USB DFU/SPI bootloader | CLI syntax | STM32_Programmer_CLI -c port=SWD -d firmware.hex 0x08000000 -rst |
| UART Bootloader DFU | STM32H7 ROM bootloader: USART1/3, USB, SPI, I2C, CAN | BOOT0 high | Set BOOT0=1, reset MCU → enters factory ROM bootloader automatically |
| USB DFU mode | USB Full-Speed DFU: PA11/PA12 USB FS pins — no UART needed | Tool | dfu-util (open source) or STM32CubeProgrammer USB DFU mode |
| PROGRAMMER COMPARISON SUMMARY |  |  |  |
| Dev + Debug | ST-LINK/V3 MINIE (best value) or J-Link EDU Mini (fastest) | Bring-up | Use Nucleo board as debugger + power source for first test |
| Low volume prod. | ST-LINK/V3 MODS + STM32CubeProgrammer CLI in test jig | High volume | Segger FlashPro gang programmer or ISSI Cyclone FX standalone |
| Field firmware update | UART DFU (bootloader) or USB DFU — no JTAG connector needed | Security | Enable RDP Level 1 after production flash — prevents readback |
| NOTE: For SA production: ST-LINK/V3 MINIE is the best value development programmer (available from Micro Robotics / RS Components SA). For automated production flashing, use STM32CubeProgrammer CLI in a Python/batch script triggered by test jig GPIO. Enable read protection (RDP1) after programming to protect firmware IP. |  |  |  |

STM32H755ZIT6 — KiCAD Schematic Wiring Checklist (per screenshots)

// ── VDD PINS (Image 1) — all connect to +3.3V net ─────────────────────

// Pin 8 VBAT → +3.3V (100nF to GND, add 1µF if using RTC/backup)

// Pin 7 VDD → +3.3V (100nF to GND)

// Pin 13 VDD → +3.3V (100nF to GND)

// Pin 19 VDD → +3.3V (100nF to GND)

// Pin 32 VDD → +3.3V (100nF to GND)

// Pin 42 VDD → +3.3V (100nF to GND)

// Pin 56 VDD → +3.3V (100nF to GND)

// Pin 71 VDD → +3.3V (100nF to GND)

// Pin 79 VDD → +3.3V (100nF to GND)

// Pin 92 VDD → +3.3V (100nF to GND)

// Pin 106 VDD → +3.3V (100nF to GND) ← NOTE: same pin group as SWDCLK

// Pin 119 VDD → +3.3V (100nF to GND)

// Pin 129 VDD → +3.3V (100nF to GND)

// Pin 144 VDD → +3.3V (100nF to GND)

// ── USB SUPPLY PINS ───────────────────────────────────────────────────

// Pin 91 VDD33_USB → +3.3V (1µF + 100nF to GND)

// Pin 90 VDD50_USB → +5V_USB (1µF + 100nF to GND) ← needs 5V even if USB unused

// ── ANALOGUE SUPPLY ───────────────────────────────────────────────────

// Pin 36 VDDA → +3.3V via 600R@100MHz ferrite bead (1µF + 10nF to GND)

// Pin 34 VSSA → GND via 0Ω resistor (star point connection)

// ── INTERNAL REGULATOR PINS ───────────────────────────────────────────

// Pin 16 VDDSMPS → +3.3V (4.7µF + 100nF to GND)

// External 2.2µH inductor: VDDSMPS → L1 → VDDCORE net

// Pin 70 VDDLDO → VDDCORE net (100nF to GND)

// Pin 105 VDDLDO → VDDCORE net (100nF to GND)

// Pin 143 VDDLDO → VDDCORE net (100nF to GND)

// VDDCORE → 10µF + 1µF to GND (near inductor output)

// ── VCAP PINS (Image 2) — internal LDO output bypass ─────────────────

// Pin 68 VCAP → 2.2µF X5R 0402 to GND (DO NOT connect to supply!)

// Pin 103 VCAP → 2.2µF X5R 0402 to GND (separate cap, no shared net)

// Pin 140 VCAP → 2.2µF X5R 0402 to GND (separate cap, no shared net)

// ── VSS PINS (Image 3) — all connect to GND ───────────────────────────

// Pin 6 VSS → GND

// Pin 11 VSS → GND

// Pin 14 VSSMPS → GND (SMPS return — same GND plane)

// Pin 17 VSS → GND

// Pin 27 VSS → GND

// Pin 34 VSSA → GND (via 0Ω/ferrite star connection — separate from DGND trace)

// Pin 38 VSS → GND

// Pin 50 VSS → GND

// Pin 65 VSS → GND

// Pin 76 VSS → GND

// Pin 89 VSS → GND

// Pin 100 VSS → GND

// Pin 116 VSS → GND

// Pin 127 VSS → GND

// Pin 138 VSS → GND

// ── PROGRAMMING / DEBUG PINS ──────────────────────────────────────────

// Pin 105 PA13/SWDIO → 10-pin debug header pin 2 (10kΩ pull-up to VDD)

// Pin 106 PA14/SWDCLK → 10-pin debug header pin 4 (10kΩ pull-down to GND)

// Pin 25 NRST → 10-pin debug header pin 10 (100nF to GND, 10kΩ to VDD)

// Pin 133 PB3/SWO → 10-pin debug header pin 6 (33Ω series) — optional SWO trace

// Pin 60 BOOT0 → 10kΩ to GND + solder jumper/test point to VDD

// ── 10-PIN CORTEX DEBUG HEADER (1.27mm pitch, 2×5) ──────────────────

// Pin 1: VCC_TARGET (connect to 3.3V via 10Ω series resistor)

// Pin 2: SWDIO (PA13) Pin 3: GND

// Pin 4: SWDCLK (PA14) Pin 5: GND

// Pin 6: SWO (PB3) Pin 7: KEY (no pin)

// Pin 8: NC Pin 9: GND

// Pin 10: nRESET (NRST)

| STM32H755ZIT6 — POWER SUPPLY SCHEMATIC BLOCKS |  |  |  |
| --- | --- | --- | --- |
| BLOCK 1 — 3.3V DIGITAL SUPPLY TO MCU (main VDD) |  |  |  |
| Supply source | 3.3V LDO or buck converter output → bulk cap (10µF) → ferrite bead (optional) → 3.3V rail → fan out to all VDD pins |  |  |
| Per VDD cap | Each VDD pin: 100nF X7R 0402 placed within 0.5mm of pin, GND via straight down to GND plane |  |  |
| KiCAD net name | Use net label '+3.3V' on all VDD pins — matches Image 1. One power symbol per sub-sheet is cleaner. |  |  |
| BLOCK 2 — ANALOGUE SUPPLY (VDDA) |  |  |  |
| VDDA filter | +3.3V ──[FB1: 600Ω@100MHz / 500mA]── VDDA node ──[1µF + 10nF to VSSA] |  |  |
| VSSA connection | VSSA ──[R_STAR: 0Ω]── GND (place at star ground point, far from digital switching) |  |  |
| KiCAD net name | Use 'VDDA' and 'VSSA' as separate net names in KiCAD — assign VSSA to analogue GND power symbol |  |  |
| BLOCK 3 — INTERNAL SMPS (VDDCORE) |  |  |  |
| SMPS circuit | +3.3V ──[4.7µF to GND]── VDDSMPS(Pin16) ¦ VDDSMPS ──[L1: 2.2µH]── VDDCORE ──[10µF + 1µF to GND] |  |  |
| VDDCORE to LDO | VDDCORE ──[100nF each]── VDDLDO(Pin70) ¦ VDDLDO(Pin105) ¦ VDDLDO(Pin143) |  |  |
| KiCAD net name | Use 'VDDCORE' net label on VDDCORE rail. This net only connects to: L1 output, VDDLDO pins, and their bypass caps. |  |  |
| BLOCK 4 — USB SUPPLY |  |  |  |
| USB 3.3V | +3.3V ──[1µF + 100nF to GND]── VDD33_USB(Pin91) — always powered from 3.3V rail |  |  |
| USB 5V | +5V_USB(VBUS) ──[1µF + 100nF to GND]── VDD50_USB(Pin90) — required even if only USB FS is used |  |  |
| USB protection | VBUS ──[F1: 500mA PTC resettable fuse]──[D_TVS: USBLC6-2SC6 or PRTR5V0U2X]── USB connector D+/D- |  |  |
| BLOCK 5 — BACKUP SUPPLY (VBAT) |  |  |  |
| Simple VBAT | +3.3V ──[100nF + 1µF to GND]── VBAT(Pin8) — sufficient if no battery backup needed |  |  |
| With battery | +3.3V ──[D1: BAT54C Schottky]──┐ ¦ CR2032 ──[D2: BAT54C Schottky]──┘ → VBAT(Pin8) ──[1µF to GND] |  |  |
| VBAT function | Maintains RTC, backup SRAM (4KB), LSE oscillator, BOR on VBAT domain even when VDD is off |  |  |
| NOTE: Reference: ST DS12930 (STM32H755 datasheet), AN4938 (STM32H7 power supply), AN4879 (STM32H7 hardware dev guidelines), RM0399 (STM32H745/755 reference manual). Always verify pin numbers against the SPECIFIC package variant datasheet. |  |  |  |

PCB DESIGN QUICK REFERENCE

| DESIGN CHECKLIST — BEFORE SENDING TO FAB |  |  |  |
| --- | --- | --- | --- |
| ERC passed | Zero errors in Electrical Rules Check | DRC passed | Zero errors in Design Rules Check |
| Footprints verified | Every footprint checked against datasheet | 3D check | 3D viewer shows no collisions or missing parts |
| Silkscreen clean | No text under pads; all ref designators visible | Courtyard clean | No overlapping courtyards |
| Gerbers verified | Inspected in Gerber viewer — all layers correct | Drill check | Drill map shows all holes in correct positions |
| BOM complete | Every component has MPN, value, footprint, quantity | Assembly files | Centroid CSV exported and verified |
| Stackup confirmed | Layer stackup matches fab's available options | Impedance | Controlled impedance traces confirmed with fab calculator |
| Design rule file | Loaded fab's .kicad_dru file and DRC clean | Version control | Design committed to Git with meaningful commit message |

PCB Design Cheat Sheet  •  26 Sections  •  Workflow | Schematic | Footprints | Stackup | Placement | Routing | SI | PDN | EMC | DRC/DFM | KiCAD | Shortcuts | Python | Libraries | High-Speed | RF | Thermal | Testing | Flex | Cost