COMPREHENSIVE EE & ECE

FORMULA & REFERENCE CHEAT SHEET

Electronics  •  BJT  •  MOSFET  •  SCR  •  Op-Amp  •  Filters  •  Digital  •  Analogue  •  Embedded  •  MCU  •  Memory  •  Industrial Comms  •  Metering  •  WebSockets  •  REST API  •  PoE

Final Year / Professional Engineering Reference  |  17 Comprehensive Sections

# 1. SEMICONDUCTOR DIODES

| DIODE FUNDAMENTALS |  |  |  |
| --- | --- | --- | --- |
| Shockley Eqn | I = I₀(e^(V/nVT) - 1) | Thermal Voltage | VT = kT/q ≈ 26 mV @ 300K |
| Ideality Factor n | 1 (ideal), 1-2 (real) | Forward V (Si) | VF ≈ 0.7 V |
| Forward V (Ge) | VF ≈ 0.3 V | Forward V (Schottky) | VF ≈ 0.2-0.4 V |
| Reverse Sat Current | I₀ ≈ 10⁻¹² A (Si) | Knee Voltage | Vγ ≈ 0.5 V (Si) |
| RECTIFIER CIRCUITS |  |  |  |
| Half-Wave Vdc | Vdc = Vm / π | HW Ripple Factor | γ = 1.21 |
| Full-Wave Vdc | Vdc = 2Vm / π | FW Ripple Factor | γ = 0.482 |
| Bridge Vdc | Vdc = 2Vm / π | Bridge PIV | PIV = Vm |
| Ripple Factor | γ = Vrms(ac) / Vdc | Capacitor Filter C | C = IL / (2fVr) |
| ZENER & SPECIAL DIODES |  |  |  |
| Zener Regulation | Vout = VZ (constant) | Zener Resistance | rz = ΔVZ / ΔIZ |
| Zener < 5V | Tunneling (Zener) breakdown | Zener > 5V | Avalanche breakdown |
| LED Wavelength | λ = hc / Eg (nm) | Photodiode | IL = S × Φ (A/W × W) |
| Varactor Cap | Cj = C0 / (1+VR/V0)^n | Schottky | Metal-semiconductor junction |

# 2. BJT TRANSISTORS (Bipolar Junction)

| BJT BASICS & CURRENT RELATIONS |  |  |  |
| --- | --- | --- | --- |
| Current gain β (hFE) | β = IC / IB | Alpha α | α = IC / IE = β/(β+1) |
| KCL | IE = IC + IB | β from α | β = α / (1 - α) |
| IC (active) | IC = β × IB + (1+β)ICBO | ICEO | ICEO = (1+β) × ICBO |
| OPERATING REGIONS |  |  |  |
| Cut-off | VBE < 0.5V (switch OFF) | Active (Amplify) | VBE ≈ 0.7V, VCE > VCE(sat) |
| Saturation (ON) | VCE(sat) ≈ 0.2V, both junctions fwd | Reverse Active | BE reverse, BC forward (rare) |
| SMALL-SIGNAL MODEL (h-parameters) |  |  |  |
| Transconductance gm | gm = IC / VT = IC / 0.026 | Input resistance rπ | rπ = β / gm |
| Output resistance ro | ro = VA / IC | Early Voltage VA | VA ≈ 50-300 V |
| CONFIGURATIONS |  |  |  |
| CE Voltage Gain | Av = -gm × RC (¦¦ro) | CE Input Z | Zin = RB ¦¦ rπ |
| CE Output Z | Zout ≈ RC ¦¦ ro | CE Current Gain | AI = -β (phase inv) |
| CB Voltage Gain | Av ≈ gm × RC (>0) | CB Current Gain | AI = α < 1 |
| CB Input Z | Zin = 1/gm (low) | CB Output Z | Zout ≈ RC (high) |
| CC (Emitter Flwr) | Av ≈ 1 (no phase inv) | CC Input Z | Zin = β × RE (high) |
| CC Output Z | Zout ≈ 1/gm (low, ~25Ω) | CC Current Gain | AI = β+1 |
| BIASING |  |  |  |
| Fixed Bias IB | IB = (VCC - VBE) / RB | Self-Bias ICQ | ICQ = (VCC - VBE) / (RB/β + RE) |
| Voltage Divider VB | VB = VCC × R2/(R1+R2) | Q-point stability | S = ΔIC / ΔICO |
| Thermal Runaway | High IC→Temp↑→IC↑ (instable) | Stability Factor S | S = (1+β)/(1-β × dIB/dIC) |
| POWER & SWITCHING |  |  |  |
| Power Dissipation | PD = VCE × IC | Max Power Hyperbola | VCE × IC = PD(max) |
| Switching ton | ton = tr + td | Switching toff | toff = ts + tf |
| NOTE: Always check β range; real β varies 2:1 across samples. Use voltage-divider bias for stability. |  |  |  |

| BJT LOGIC-LEVEL SWITCHING — DUAL RESISTOR NETWORK |  |  |  |
| --- | --- | --- | --- |
| WHY TWO RESISTORS? RB (base series) + R_BE (base-to-emitter pull-down) |  |  |  |
| RB — base resistor | Limits base current: RB = (Vlogic - VBE) / IB_min | Typical RB | 1kΩ – 10kΩ for 3.3V/5V logic |
| R_BE — pull-down | Pulls base to GND when driver is Hi-Z / off | Typical R_BE | 10kΩ – 47kΩ (10× RB recommended) |
| IB required | IB = IC / β_min (use worst-case β) | Overdrive x10 | IB = 10 × IC / β for hard saturation |
| VBE threshold | NPN: VBE ≈ 0.7V ¦ PNP: VEB ≈ 0.7V | Logic low leak | R_BE holds base < 0.5V → device stays OFF |
| NPN SWITCH (LOW-SIDE) — CIRCUIT & CALCULATIONS |  |  |  |
| Circuit (NPN) | VCC──RLOAD──Collector ¦ Base──RB──GPIO(3.3V) ¦ R_BE: Base──GND ¦ Emitter──GND |  |  |
| RB formula | RB = (Vgpio - VBE) / IB = (3.3V - 0.7V) / (IC / β_min) [Ω] |  |  |
| R_BE formula | R_BE = 10 × RB (prevents false turn-on from leakage / noise) |  |  |
| Example 100mA | IC=100mA, β=100, Vgpio=3.3V → IB=1mA → RB=(2.6V/1mA)=2.6kΩ → use 2.7kΩ; R_BE=27kΩ |  |  |
| Saturation check | VCE(sat)=0.2V must be < VCC-VRLOAD ¦ IC(sat) = (VCC-0.2V)/RLOAD |  |  |
| PNP SWITCH (HIGH-SIDE) — CIRCUIT & CALCULATIONS |  |  |  |
| Circuit (PNP) | VCC──Emitter ¦ Base──RB──GPIO ¦ R_BE: Base──VCC(via pull-up) ¦ Collector──RLOAD──GND |  |  |
| PNP ON condition | GPIO = LOW (0V) → VEB = VCC - 0 > 0.7V → transistor conducts |  |  |
| PNP OFF | GPIO = HIGH (VCC) → VEB ≈ 0V → transistor OFF |  |  |
| RB (PNP) | RB = (VCC - Vgpio_low - VEB) / IB = (VCC - VEB) / IB when GPIO pulls to GND |  |  |
| SWITCHING SPEED CONSIDERATIONS |  |  |  |
| Turn-ON delay td | Time for base charge to build (ns–µs) | Turn-OFF ts | Storage time: base charge must clear |
| Speedup capacitor | C parallel with RB — boosts base drive on edge | Value Cspeed | C ≈ 10–100pF typical (trial with scope) |
| Baker clamp | Schottky diode: base–collector prevents deep sat. | Benefit | Eliminates storage time ts → fast switch |
| NOTE: Always use R_BE pull-down with GPIO-driven BJT switches. Without it, floating base or leakage current can partially turn on the transistor. Ratio rule: R_BE = 10× RB. |  |  |  |

# 3. MOSFET & FET

| MOSFET REGIONS & EQUATIONS |  |  |  |
| --- | --- | --- | --- |
| Enhancement NMOS ON | VGS > Vth (typically 1-4V) | Depletion NMOS ON | VGS = 0 (normally ON) |
| Triode (Linear) ID | ID = k[(VGS-Vth)VDS - VDS²/2] | Saturation ID | ID = (kn/2)(VGS - Vth)² |
| Process Transcond. k | k = µn × Cox × W/L | Cox | Cox = εox / tox |
| Transconductance gm | gm = 2ID/(VGS-Vth) = √(2k×ID) | Output cond. gds | gds = ID / VA |
| Voltage Gain (sat) | Av = -gm × RD | Drain-Source sat. | VDS(sat) = VGS - Vth |
| JFET |  |  |  |
| Shockley (JFET) | ID = IDSS(1 - VGS/VP)² | Pinch-off VP | VP < 0 for N-JFET |
| gm (JFET) | gm = -2IDSS/VP × (1-VGS/VP) | IDSS | ID at VGS = 0 |
| CMOS & LOGIC |  |  |  |
| CMOS Inverter | NMOS pulls down, PMOS pulls up | Static Power | P ≈ 0 (only leakage) |
| Dynamic Power | P = α × C × VDD² × f | Propagation Delay | tpd = 0.69RC |
| CMOS Noise Margin | NMH = VOH - VIH | NML | NML = VIL - VOL |
| POWER MOSFET (switching) |  |  |  |
| RDS(on) | On-resistance (mΩ range) | Gate Charge QG | QG = IG × tg |
| Switching Loss | Psw = 0.5 × VDS × ID × (tr+tf) × f | Conduction Loss | Pcond = ID² × RDS(on) |
| Gate Drive Vgs | Vgs(th) + 10V typ for full ON | Safe Operating Area | Check SOA curve in datasheet |
| NOTE: Higher W/L → lower Vth, higher gm. Use PMOS for high-side, NMOS for low-side switching. |  |  |  |

| MOSFET LOGIC-LEVEL SWITCHING — GATE RESISTOR NETWORK |  |  |  |
| --- | --- | --- | --- |
| WHY TWO RESISTORS? RG (gate series) + RGS (gate-to-source pull-down) |  |  |  |
| RG — gate resistor | Damps ringing on gate trace (RC with Ciss) | Typical RG | 10Ω – 100Ω for discrete MOSFETs |
| RGS — pull-down | Holds gate at 0V when driver Hi-Z / power-up | Typical RGS | 10kΩ – 100kΩ (must not load driver) |
| Gate threshold Vth | VGS(th): 1–4V logic-level ¦ 2–6V standard | Drive Vgs | Logic-level MOSFET: fully ON at 5V |
| RGS prevents | Floating gate → dV/dt turn-on / ESD latch-up | Gate capacitor | Ciss = Cgd + Cgs (pF–nF range) |
| N-CHANNEL LOW-SIDE SWITCH — STANDARD CONFIGURATION |  |  |  |
| Circuit (NMOS) | VDD──RLOAD──Drain ¦ Gate──RG──GPIO ¦ RGS: Gate──Source(GND) ¦ Source──GND |  |  |
| Turn-ON time | t_on ≈ RG × Ciss (time constant for gate to charge through RG) |  |  |
| Turn-OFF time | t_off ≈ (RG + Rdriver_pull-down) × Ciss |  |  |
| Gate current pk | Ig_peak = (Vdriver - Vgs_plateau) / RG [A] (occurs during Miller plateau) |  |  |
| Example | Vgpio=3.3V, Ciss=1nF, RG=47Ω → t_on≈47ns; RGS=100kΩ holds gate<0.1V when OFF |  |  |
| P-CHANNEL HIGH-SIDE SWITCH |  |  |  |
| Circuit (PMOS) | VDD──Source ¦ Gate──RG──GPIO ¦ RGS: Gate──Source(VDD via pull-up) ¦ Drain──RLOAD──GND |  |  |
| PMOS ON | GPIO=LOW → VGS = 0 - VDD = -VDD → ¦VGS¦ > ¦Vth¦ → PMOS conducts |  |  |
| PMOS OFF | GPIO=HIGH (VDD) → VGS = VDD - VDD = 0V → PMOS OFF |  |  |
| RGS (PMOS) | Pull-up resistor: Gate──RGS──VDD (holds gate=VDD=OFF when GPIO Hi-Z) |  |  |
| GATE DRIVER IC (recommended for >1A switching) |  |  |  |
| Gate driver function | Provides high peak current (1–10A) for fast switching | Example ICs | UCC27524, TC4427, IR2101, IRS2003 |
| Bootstrap (half-brdg) | High-side NMOS gate drive above VDD via bootstrap C | Dead time | Both switches OFF briefly to prevent shoot-through |
| Shoot-through | Both high-side & low-side ON simultaneously → short! | Protection | Dead-time + cross-conduction interlock |
| PARASITIC EFFECTS & MITIGATION |  |  |  |
| Miller effect | Cdg amplifies effective input capacitance | dV/dt turn-on | Fast drain edge charges Cgd → raises Vgs |
| Gate ringing | L_trace × Ciss resonance → overshoot/EMI | Fix | Increase RG or add ferrite bead on gate |
| Negative VGS spike | Source inductance creates -VGS on turn-off | Fix | Minimize source path inductance; use Kelvin source |
| NOTE: RGS is mandatory — never leave a MOSFET gate floating. For fast switching: use logic-level FET (Vth <2V) with gate driver IC. Increase RG to reduce EMI at the cost of switching loss. |  |  |  |

# 4. SCR & THYRISTORS

| SCR BASICS |  |  |  |
| --- | --- | --- | --- |
| Structure | PNPN (4-layer) — Anode, Gate, Cathode | Turn-ON | Gate pulse (IG > IGT) while VAK > 0 |
| Holding Current IH | Min IA to stay latched (gate removed) | Latching Current IL | Min IA to turn ON after gate pulse |
| Turn-OFF | Reduce IA < IH (commutation) | Forward Breakover | VBO: turns ON without gate (avoid) |
| 2-transistor model | PNP + NPN regenerative pair | α1 + α2 = 1 | Condition for latching |
| FIRING ANGLE & POWER CONTROL |  |  |  |
| Half-wave avg Vdc | Vdc = (Vm/2π)(1+cosα) | Full-wave avg Vdc | Vdc = (Vm/π)(1+cosα) |
| Firing angle α | Delay from voltage zero-crossing (°) | Conduction angle | γ = π - α (half-wave) |
| Power delivered | P = Vrms² / R (Vrms varies with α) | RMS output | Vrms = Vm√[(π-α+sin2α/2)/2π] |
| TRIAC & DIAC |  |  |  |
| TRIAC | Bidirectional SCR (AC control) | DIAC | Bilateral trigger diode (breakover ~32V) |
| TRIAC trigger | Gate pulse in any quadrant (Q1-Q4) | Snubber RC | Prevents dV/dt false triggering |
| COMMUTATION METHODS |  |  |  |
| Natural (line) | AC supply reversal turns OFF SCR | Forced | External LC circuit forces IA=0 |
| Class A | Self commutated by LC resonance | Class B | Separate commutation capacitor |
| NOTE: SCRs cannot be turned OFF by gate. Use IGBT or power MOSFET if active turn-off is needed. |  |  |  |

# 5. OPERATIONAL AMPLIFIERS

| IDEAL OP-AMP RULES & SPECS |  |  |  |
| --- | --- | --- | --- |
| Virtual Short | V+ = V- (with neg. feedback) | No Input Current | I+ = I- = 0 |
| Ideal AOL | AOL = ∞ | Ideal Zin | Zin = ∞ |
| Ideal Zout | Zout = 0 | Ideal BW | BW = ∞, CMRR = ∞ |
| GBW Product | GBW = Av × BW = constant | Slew Rate SR | SR = dVout/dt¦max [V/µs] |
| CMRR | CMRR = 20log(Ad/Acm) [dB] | PSRR | 20log(ΔVs/ΔVout) [dB] |
| Input Offset Vos | Differential error voltage | Bias Current IB | IB = (I+ + I-)/2 |
| AMPLIFIER CONFIGURATIONS |  |  |  |
| Inverting Amp | Av = -Rf / Rin | Non-Inverting Amp | Av = 1 + Rf/R1 |
| Voltage Follower | Av = 1, Zin = ∞ | Difference Amp | Vout = (R2/R1)(V2-V1) |
| Summing Amp | Vout = -Rf(V1/R1 + V2/R2 + ...) | Instrumentation Amp | Av = (1+2R/RG) × R2/R1 |
| INTEGRATORS & DIFFERENTIATORS |  |  |  |
| Integrator Vout | Vout = -(1/RC)∫Vin dt | Differentiator Vout | Vout = -RC × dVin/dt |
| Integrator f_lower | f_low = 1/(2πRfC) (reset R) | Diff. stability | Add R series with C input |
| COMPARATORS & OSCILLATORS |  |  |  |
| Comparator | Vout = ±Vsat (no feedback) | Hysteresis VH | VH = 2VsatR1/(R1+R2) |
| Schmitt Trigger + | V+ = VsatR1/(R1+R2) | Schmitt Trigger - | V- = -VsatR1/(R1+R2) |
| Wien Bridge f0 | f0 = 1/(2πRC) | Phase Shift Osc f0 | f0 = 1/(2π√6 RC) |
| 555 Astable f | f = 1.44/((R1+2R2)C) | 555 Duty Cycle | D = (R1+R2)/(R1+2R2) |
| ACTIVE FILTERS (see Section 6 for complete filter reference) |  |  |  |
| 1st-order LPF fc | fc = 1/(2πRC) | 1st-order HPF fc | fc = 1/(2πRC) |
| Sallen-Key BPF f0 | f0 = 1/(2π√(R1R2C1C2)) | MFB Gain | Av = -R2/R1 @ dc |

| OP-AMP CURRENT SOURCE & CURRENT SINK CIRCUITS |  |  |  |
| --- | --- | --- | --- |
| VOLTAGE-CONTROLLED CURRENT SOURCE (Howland Pump) |  |  |  |
| Howland Iout | Iout = (Vin2 - Vin1) / R [A] — requires matched resistors R1=R2=R3=R4=R |  |  |
| Compliance | Max Vload = Vcc - Vsat (op-amp output swing limited) |  |  |
| Accuracy tip | Use 0.1% matched R; unmatched R creates load-dependent error |  |  |
| SIMPLE VCCS — GROUNDED LOAD CURRENT SOURCE (NPN + Op-Amp) |  |  |  |
| Circuit | Vref──(+)OpAmp(-)──Base of NPN transistor ¦ Emitter──Rset──GND ¦ Emitter feedback to (-)OpAmp ¦ Load: VCC──Load──Collector |  |  |
| Output current | Iout = Vref / Rset (op-amp forces V- = V+ = Vref across Rset) |  |  |
| Example 4-20mA | Iout=4mA: Vref=0.4V,Rset=100Ω ¦ Iout=20mA: Vref=2.0V,Rset=100Ω |  |  |
| Accuracy | Error ≈ Vos/Rset + IB → use low Vos op-amp (OPA2333, LT1013) |  |  |
| CURRENT SINK — GROUNDED LOAD (Op-Amp + N-MOSFET) |  |  |  |
| Circuit | VCC──Load──Drain(NMOS)──Source──Rset──GND ¦ Rset mid-point → OpAmp(-) ¦ Vctrl → OpAmp(+) ¦ OpAmp out → Gate |  |  |
| Sink current | Isink = Vctrl / Rset (loop forces Vsource = Vctrl via feedback) |  |  |
| MOSFET choice | Use logic-level N-MOSFET (Vgs(th) < 2.5V) for 3.3V/5V control |  |  |
| Max Isink | Limited by MOSFET Id(max) and Rset power: P = Isink² × Rset |  |  |
| 4-20mA TRANSMITTER (2-WIRE LOOP-POWERED) |  |  |  |
| Loop equation | Iloop = Vsensor_scaled / Rset range 4mA (0%) to 20mA (100%) |  |  |
| Loop voltage | Vloop = 24V (typical) ¦ Vdrop_wire + Vdrop_receiver + VCCS compliance |  |  |
| Receiver | Vout = Iloop × Rload (Rload = 250Ω → 1-5V output; 500Ω → 2-10V) |  |  |
| Power budget | Device must operate on 4mA × (24V - losses) ≈ 80mW max |  |  |
| PRECISION CURRENT REFERENCE (Op-Amp + Zener + BJT) |  |  |  |
| Zener reference | Vzener → non-inv input → op-amp → BJT base | Output Iref | Iref = (Vzener - VBE) / Rset |
| Temperature drift | Match Zener TC with VBE TC for stability | Better option | Use bandgap reference IC (LM4040, REF02) |
| Current mirror | IC1 = IC2 when VBE1 = VBE2 (matched BJTs) | Wilson mirror | Higher output impedance (3rd BJT added) |
| CURRENT MEASUREMENT WITH OP-AMP (SHUNT AMPLIFIER) |  |  |  |
| High-side sense | Vdiff = I × Rshunt ¦ Use diff-amp: Vout = (R2/R1) × I × Rshunt |  |  |
| Low-side sense | Shunt between load and GND ¦ Single-ended amp: Vout = I × Rshunt × Av |  |  |
| INA219 / INA226 | Integrated shunt + 12/16-bit ADC + I2C — for 0-3.2A / ±40V |  |  |
| NOTE: For 4-20mA industrial loops: use dedicated ICs (XTR115, AD693). Always protect op-amp inputs with back-to-back Schottky clamps on industrial wiring. |  |  |  |

# 6. FILTERS (Passive & Active)

| PASSIVE FILTERS |  |  |  |
| --- | --- | --- | --- |
| RC LPF -3dB | fc = 1/(2πRC) | RC HPF -3dB | fc = 1/(2πRC) |
| RC LPF ¦H(jω)¦ | ¦H¦ = 1/√(1+(f/fc)²) | RC HPF ¦H(jω)¦ | ¦H¦ = (f/fc)/√(1+(f/fc)²) |
| LC LPF fc | fc = 1/(2π√LC) | LC BPF f0 | f0 = 1/(2π√LC) |
| RLC BW | BW = R/L = f0/Q | Q factor | Q = f0/BW = (1/R)√(L/C) |
| Notch (Twin-T) f0 | f0 = 1/(2πRC) | Notch depth (ideal) | Infinite (infinite Q) |
| ACTIVE FILTER TYPES |  |  |  |
| Butterworth | Maximally flat passband, -20n dB/dec rolloff | Chebyshev | Faster rolloff, passband ripple |
| Bessel | Maximally flat group delay (best phase linearity) | Elliptic (Cauer) | Steepest rolloff, both ripples |
| SALLEN-KEY (2nd ORDER) |  |  |  |
| S-K LPF f0 | f0 = 1/(2π√(R1R2C1C2)) | S-K LPF Q | Q = √(R1R2C1C2)/(C2(R1+R2)) |
| S-K HPF f0 | f0 = 1/(2π√(R1R2C1C2)) | S-K HPF Q | Q = √(R1R2C1C2)/(R1(C1+C2)) |
| Unity-gain S-K | Av = 1 (Butterworth: Q=0.707) | Gain >1 S-K | Av = 1 + Rb/Ra |
| BODE PLOT RULES |  |  |  |
| Single pole slope | -20 dB/decade below fc | Single zero slope | +20 dB/decade above fz |
| 2nd-order rolloff | -40 dB/decade | Phase at fc (1P) | -45° |
| Phase total 1P | 0° to -90° | Phase total 2P | 0° to -180° |
| GROUP DELAY & PHASE |  |  |  |
| Group Delay τg | τg = -dφ/dω [s] | Phase Linearity | Best: Bessel, Worst: Elliptic |
| Decibels | dB = 20log¦Vout/Vin¦ | Half-power point | -3.01 dB = ¦H¦ = 0.707 |

# 7. DIGITAL ELECTRONICS

| BOOLEAN ALGEBRA & GATES |  |  |  |
| --- | --- | --- | --- |
| AND | Y = A · B | OR | Y = A + B |
| NOT | Y = A' | NAND (Universal) | Y = (A·B)' |
| NOR (Universal) | Y = (A+B)' | XOR | Y = A⊕B = A'B + AB' |
| XNOR | Y = (A⊕B)' = AB + A'B' | De Morgan 1 | (AB)' = A' + B' |
| De Morgan 2 | (A+B)' = A'·B' | Consensus | AB + A'C + BC = AB + A'C |
| COMBINATIONAL CIRCUITS |  |  |  |
| Half Adder S | S = A⊕B | Half Adder Cout | Cout = AB |
| Full Adder S | S = A⊕B⊕Cin | Full Adder Cout | Cout = AB+Cin(A+B) |
| MUX 4:1 out | Y = I0s̄1s̄0 + I1s̄1s0 + I2s1s̄0 + I3s1s0 | DEMUX 1:4 | Reverse of MUX |
| Encoder | 2ⁿ inputs → n outputs | Decoder | n inputs → 2ⁿ outputs |
| Comparator 1-bit | A=B: XNOR; A>B: AB'; A<B: A'B | Priority Encoder | Highest input wins |
| FLIP-FLOPS & REGISTERS |  |  |  |
| D Flip-Flop | Qnext = D (edge-triggered) | JK FF toggle | J=K=1: toggle; J=K=0: hold |
| SR FF Set | S=1,R=0: Q=1 | SR FF Reset | S=0,R=1: Q=0 |
| T Flip-Flop | Q toggles when T=1 at clock edge | Metastability | Occurs when setup/hold violated |
| Setup time tsu | Data must be stable before clk edge | Hold time th | Data stable after clk edge |
| Max clock freq | fmax = 1/(tpd + tsu + tskew) | Propagation delay | tpd = output delay from clk edge |
| COUNTERS & STATE MACHINES |  |  |  |
| Mod-N counter | Divides by N, needs log2N FFs | Ring counter | N FFs for Mod-N |
| Johnson counter | 2N states with N FFs | Gray code counter | 1-bit change per step |
| Moore FSM | Output = f(state only) | Mealy FSM | Output = f(state, input) |
| NUMBER SYSTEMS & ARITHMETIC |  |  |  |
| Binary to Decimal | Σ bit × 2^position | 2's Complement | Invert all bits, add 1 |
| Hex digit | = 4 binary bits (nibble) | BCD | 4 bits per decimal digit (0-9) |
| Signed range n-bit | -(2^(n-1)) to 2^(n-1)-1 | Unsigned range | 0 to 2^n - 1 |
| LOGIC FAMILIES |  |  |  |
| TTL VOH | > 2.4 V | TTL VOL | < 0.4 V |
| CMOS logic swing | Rail-to-rail (0 to VDD) | CMOS power | Dynamic only: P=CfV² |
| Fan-out TTL | 10 loads max (std TTL) | Noise Margin | NMH = VOH - VIH |

# 8. ANALOGUE ELECTRONICS

| SIGNAL ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| RMS General | Vrms = √(1/T ∫v²dt) | Average (full-wave) | Vavg = 2Vm/π |
| Form Factor | FF = Vrms/Vavg = 1.11 (sine) | Crest Factor | CF = Vpeak/Vrms = √2 (sine) |
| THD | THD = √(V2²+V3²+...)/V1 × 100% | SINAD | Signal+Noise+Distortion ratio |
| SNR | SNR = 20log(Vsignal/Vnoise) dB | Dynamic Range | DR = 20log(Vmax/Vmin) |
| FEEDBACK AMPLIFIERS |  |  |  |
| Closed-loop gain | Acl = Aol / (1 + Aol×β) | Loop gain T | T = Aol × β |
| Bandwidth increase | BWcl = BWol × (1 + T) | Distortion reduce | Dcl = Dol / (1 + T) |
| Series-series FB | Increases Rin, increases Rout | Shunt-shunt FB | Decreases Rin, decreases Rout |
| Stability criterion | Gain Margin > 6 dB, Phase Margin > 45° | Phase Margin | φm = 180° - ¦φ @ unity gain¦ |
| SIGNAL GENERATORS |  |  |  |
| Barkhausen Criterion | ¦Aβ¦ = 1 and ∠Aβ = 0° | Colpitts f0 | f0 = 1/(2π√(LC_eq)) |
| Hartley f0 | f0 = 1/(2π√(LT×C)) | Crystal Oscillator | Q = 10,000–100,000 (stable) |
| NOISE IN CIRCUITS |  |  |  |
| Thermal Noise Vn | Vn = √(4kTRΔf) [V/√Hz] | Shot Noise In | In = √(2qIDC×Δf) |
| 1/f Noise | Dominates at low frequency | Noise Figure NF | NF = 10log(Fout/Fin) dB |
| Friis Formula | Ftotal = F1+(F2-1)/G1+(F3-1)/(G1G2) | Noise Temp | Te = T0(F-1) |
| POWER AMPLIFIERS |  |  |  |
| Class A η_max | η = 25% (resistive), 50% (transformer) | Class B η_max | η = 78.5% (π/4 × 100%) |
| Class AB | Small bias, crossover distortion fix | Class C | η > 90%, used in RF/tuned |
| Class D (switching) | η > 90%, PWM + LC filter output | Class E/F | Resonant switching, RF PAs |
| Thermal resistance | Tj = Ta + Pd×(θjc+θcs+θsa) | Heat sink calc | θsa = (Tj-Ta)/Pd - θjc - θcs |

| ANALOGUE CIRCUIT DESIGN — EXTENDED REFERENCE |  |  |  |
| --- | --- | --- | --- |
| SIGNAL CONDITIONING CHAIN |  |  |  |
| Typical chain | Sensor → Anti-alias LPF → Instrumentation Amp → PGA → ADC → DSP/MCU |  |  |
| Input protection | TVS diode + series R clamp to supply rails | ESD rating | IEC 61000-4-2: ±2kV contact, ±4kV air |
| Anti-alias LPF | fc < fs/2 (Nyquist); set fc = 0.4 × fs for margin | Order choice | 2nd order Bessel for min phase distortion |
| Inst. Amp gain | Av = 1 + 2R/Rg (INA128: Rg = 50kΩ/Av) | CMRR typical | 80-120 dB at 60Hz |
| PGA (prog gain amp) | Digitally set gain via SPI/I2C — ADS8688, PGA112 | Gain ranging | Auto-range: start high gain, reduce on clip |
| IMPEDANCE MATCHING |  |  |  |
| Max power transfer | ZSource = ZLoad* (conjugate match) | Voltage match | ZSource << ZLoad (buffer between stages) |
| Input buffer | Voltage follower: Zin=∞, Zout≈1/gm | Output buffer | Emitter follower: Zout ≈ 25Ω |
| Resistive divider load | Vout = Vin × R2/(R1+R2) only valid if Zload >> R2 | Loading error | Error% = R2¦¦(Zload) vs R2 |
| PRECISION RECTIFIER & PEAK DETECTOR |  |  |  |
| Precision HWR | D in feedback: Vout = Vin for Vin>0, Vout=0 for Vin<0 (no 0.7V drop) |  |  |
| Full-wave prec. | Two-stage: HWR + summer ¦ Vout = ¦Vin¦ for all Vin |  |  |
| Peak detector | D + C: Vcap = Vpeak ¦ Decay: Vcap decays at τ = R_leak × C |  |  |
| RMS converter | AD736/AD737: true RMS IC ¦ Vout = Vrms for any waveform |  |  |
| SAMPLE & HOLD (S/H) |  |  |  |
| Acquisition time | Time for output to settle to ±0.5LSB after sample cmd | Droop rate | ΔV/Δt = Ileak / Chold |
| Aperture jitter | Uncertainty in sample instant → amplitude error | Clock jitter | σV = σt × dV/dt¦max |
| S/H formula | Vhold = Vin(t_sample) (frozen for ADC conversion) | Hold cap | C = I_ADC / (dV/dt_max) |
| VOLTAGE REFERENCES |  |  |  |
| Bandgap reference | 1.25V (LM385) or 2.5V (REF02) ± 0.1% | Temp coeff | Bandgap: 5-50 ppm/°C; Zener: 50-200 ppm/°C |
| Buried Zener | Sub-surface Zener: < 5 ppm/°C | Low noise ref | LT6655, ADR4520: < 1µV p-p noise |
| Shunt vs series ref | Shunt: 2-terminal, constant V (TL431) | Series ref | 3-terminal, lower dropout (LM4040) |
| ANALOGUE MULTIPLICATION & MODULATION |  |  |  |
| Analogue multiplier | Vout = (V1 × V2) / Vref (AD633, MPY634) | Applications | Power metering, AM modulation, AGC |
| AM modulation | Vout = Vc(1 + m×Vm)sinωct ¦ m = mod. depth | DSB-SC | Balanced modulator — suppressed carrier |
| Lock-in amplifier | Extracts signal at known freq from noisy background | PSRR use | Multiply × ref, LPF → rejects all other f |
| GROUNDING & SHIELDING BEST PRACTICES |  |  |  |
| Star ground | All grounds meet at single point (low frequency) | Ground plane | Solid copper plane (RF / switching circuits) |
| Guard ring | Driven shield at same potential as hi-Z node | Eliminates | Leakage current across PCB surface |
| Twisted pair | Differential signal — cancels common-mode noise | Shield drain | Connect shield at one end only (to signal GND) |
| Decoupling caps | 100nF ceramic + 10µF electrolytic per power pin | Placement | < 2mm from IC VCC pin; via direct to GND plane |
| NOTE: Analogue design rule: keep signal traces short, away from switching nodes, and always decouple power locally. A poor ground plane ruins even the best circuit design. |  |  |  |

# 9. EMBEDDED SYSTEMS

| TIMING & CLOCK |  |  |  |
| --- | --- | --- | --- |
| Instruction Time | T_inst = CPI / f_clk | MIPS | MIPS = f_clk / (CPI × 10⁶) |
| Timer Period | T = (PRE × (TOP+1)) / f_clk | PWM Duty Cycle | D% = (OCR / TOP) × 100 |
| PWM freq | f_PWM = f_clk / (PRE × (TOP+1)) | Baud rate error% | err = (actual-desired)/desired×100 |
| ADC & DAC |  |  |  |
| ADC Resolution | LSB = Vref / 2ⁿ | ADC Output code | D = (Vin × 2ⁿ) / Vref |
| ADC Accuracy | ±0.5 LSB ideal | SNR (n-bit ADC) | SNR = 6.02n + 1.76 dB |
| DAC Output V | Vout = (D / 2ⁿ) × Vref | Settling time | Time for Vout to reach ±0.5LSB |
| Nyquist Rate | fs ≥ 2 × fmax (signal) | Anti-alias filter | fc < fs/2 |
| UART / SERIAL |  |  |  |
| UART frame | Start(1) + Data(8) + Parity(0/1) + Stop(1-2) | Baud = bit rate | Bits per second |
| SPI speed | Up to f_clk/2 (master clock/2) | SPI modes | CPOL/CPHA: 0,0 ¦ 0,1 ¦ 1,0 ¦ 1,1 |
| I2C speed | 100kbps / 400kbps / 1Mbps / 3.4Mbps | I2C address | 7-bit (128 nodes) or 10-bit |
| MEMORY MAP & ADDRESSING |  |  |  |
| Address decode | n address bits → 2ⁿ locations | Word width | Data bus width (8/16/32 bits) |
| Memory size | Size = 2^(address_bits) × word_width/8 | Base+Offset | Effective addr = base + offset |
| RTOS CONCEPTS |  |  |  |
| Task priority | Higher number = higher priority (common) | Context switch | Save/restore CPU state (registers) |
| Semaphore | Counting sync primitive (binary=mutex) | Deadlock | 2+ tasks wait on each other's resource |
| Jitter | Variation in task execution timing | Latency | Time from event to response |
| Stack overflow | Task stack exceeds allocated RAM | Watchdog timer | WDT resets if not kicked in time |
| POWER MANAGEMENT |  |  |  |
| Dynamic power | P = CL × VDD² × f × α | Static power | P = VDD × Ileak |
| Sleep modes | Idle→ADC Noise Red.→Power-save→Standby→Power-down | Wakeup sources | INT, WDT, Timer, USART, TWI |
| NOTE: Always enable watchdog in production firmware. Use volatile for shared vars between ISR and main loop. |  |  |  |

# 10. MICROCONTROLLERS (MCU)

| MCU ARCHITECTURE & PERIPHERALS |  |  |  |
| --- | --- | --- | --- |
| AVR (ATmega) | 8-bit Harvard, 1 MIPS/MHz, 2-16KB SRAM | ARM Cortex-M0+ | 32-bit, 1.77 DMIPS/MHz, low power |
| ARM Cortex-M4 | 32-bit+FPU, DSP, 3.4 DMIPS/MHz | ESP32 | Dual-core Xtensa 240MHz, WiFi+BT |
| STM32 families | F0/G0=M0, F1/F3=M3, F4=M4, H7=M7 | PIC16/18 | 8-bit RISC, simple Harvard |
| TIMERS |  |  |  |
| Timer overflow IRQ | Every (2^n - start_val) counts | Input Capture | Records timer count on edge |
| Output Compare | Action on match (PWM, GPIO toggle) | Encoder mode | Quadrature pulse counting |
| GPIO & INTERRUPTS |  |  |  |
| GPIO output current | Typ 8-40 mA (check datasheet) | GPIO input levels | VIH ≥ 0.7VDD, VIL ≤ 0.3VDD |
| External INT | EXTI on rising/falling/both edge | NVIC priority | 0=highest, check MCU bits (4-8bit) |
| ISR golden rules | Keep short, no blocking, volatile vars | Debounce | SW delay or RC+Schmitt HW |
| DMA (Direct Memory Access) |  |  |  |
| DMA purpose | Data transfer without CPU (mem↔periph) | Circular mode | Buffer wraps, continuous transfer |
| Half-transfer IRQ | Process first half while second fills | DMA priority | VHigh > High > Med > Low |
| BOOTLOADER & PROGRAMMING |  |  |  |
| ISP (In-System Prog) | Program via SPI/UART without removal | JTAG/SWD | Debug + program interface |
| Bootloader address | Start of flash (0x00000 or 0x08000000) | Application address | After bootloader region |
| Flash endurance | 10k-100k write cycles typical | EEPROM endurance | 100k-1M write cycles |
| NOTE: Use DMA for UART/SPI/ADC to avoid CPU stalls. ARM SWD only needs 2 pins vs JTAG 4 pins. |  |  |  |

| MICROCONTROLLER — EXTENDED REFERENCE |  |  |  |
| --- | --- | --- | --- |
| POPULAR MCU FAMILIES COMPARISON |  |  |  |
| ATmega4809 (AVR) | 8-bit, 20MHz, 48KB Flash, 6KB SRAM, UPDI debug | ATtiny3217 | 8-bit, 20MHz, 32KB Flash, 2KB SRAM |
| PIC16F877A | 8-bit, 20MHz, 14KB Flash, 368B SRAM, ICSP | PIC18F4550 | 8-bit, 48MHz, 32KB Flash, USB FS OTG |
| STM32F103C8 (Blue Pill) | 32-bit M3, 72MHz, 64KB Flash, 20KB SRAM, 37 GPIO | STM32G0B1 | 32-bit M0+, 64MHz, 512KB Flash, FDCAN |
| STM32H755ZI | Dual M7+M4, 480/240MHz, 2MB Flash, 1MB SRAM | RP2040 | Dual M0+, 133MHz, 264KB SRAM, PIO blocks |
| ESP32-S3 | Dual Xtensa 240MHz, WiFi+BT5, AI accelerator | nRF52840 | M4F, 64MHz, BLE5, USB, 1MB Flash, 256KB SRAM |
| DSPIC33EP | 16-bit DSC, 70 MIPS, hardware QEI, PWM, CAN | TMS320F28379D | 32-bit C28x DSP, 200MHz, dual-core, CLA coprocessor |
| CLOCK CONFIGURATION |  |  |  |
| PLL multiply | Fsys = Fxtal × PLL_M / PLL_N / PLL_P | Flash wait states | Add wait states when Fsys > threshold voltage |
| Clock security CSS | Detects HSE failure, falls back to HSI | HSI accuracy | ±1% trimmed factory (STM32) vs crystal ±20ppm |
| Clock gating | Disable unused peripheral clocks to save power | RCC enable | RCC->APB1ENR ¦= RCC_APB1ENR_TIM2EN; |
| ADC — ADVANCED CONFIGURATION |  |  |  |
| ADC sampling time | Tsamp ≥ (Rin + Radc) × Cadc × ln(2^(n+1)) | Min sample time | Drives Cadc to required accuracy |
| Differential mode | Measures V+ − V− (rejects common mode noise) | Pseudo-diff | V− tied to stable ref, V+ is signal |
| Oversampling | Average N samples → +log2(N)/2 extra bits | 16× oversample | 12-bit ADC → effective 14-bit resolution |
| ADC triggering | Timer CC event starts ADC — precise periodic sampling | DMA + ADC | Continuous DMA fill → no CPU needed for samples |
| PWM ADVANCED FEATURES |  |  |  |
| Complementary PWM | High + low side with dead time insertion | Dead time calc | DT = DT_count / Fclk (e.g. 100ns = 10 counts @ 100MHz) |
| Centre-aligned PWM | Counter counts up then down — lower harmonics | Phase shift PWM | Multiple timers synced at phase offsets |
| SPWM (sine PWM) | Sine modulation: compare sin table vs triangle carrier | THD SPWM | < 5% THD with LC output filter |
| COMMUNICATION PERIPHERAL DETAILS |  |  |  |
| UART flow control | RTS/CTS hardware flow control for high-speed data | UART FIFO | Hardware FIFO prevents overrun (16-64 bytes) |
| SPI DMA transfer | Set up DMA for TX+RX simultaneously | SPI CS control | Manual CS (GPIO) or hardware NSS managed |
| I2C clock stretching | Slave holds SCL low to stall master | I2C timeout | Set TIMEOUT register to detect bus lockup |
| CAN init sequence | Set bit timing, enable, wait INAK=0, set normal mode | CAN bus off | TEC > 255 → auto bus-off, requires reset |
| DEBUGGING & DIAGNOSTICS |  |  |  |
| SWD interface | SWDIO + SWDCLK + GND (+Vref optional) | JTAG | TDI+TDO+TMS+TCK+TRST — 5 pins, supports boundary scan |
| ITM / SWO printf | printf via SWO pin — no UART needed, zero overhead | Live watch | OpenOCD + GDB: watch variables in real-time |
| Fault handlers | HardFault, MemManage, BusFault, UsageFault — decode CFSR | CFSR register | Precise error: divide by 0, unaligned, invalid PC |
| Stack watermark | Fill stack with 0xDEADBEEF, check how far it depleted | FreeRTOS | uxTaskGetStackHighWaterMark() returns min free words |
| PRODUCTION & RELIABILITY |  |  |  |
| Watchdog types | IWDG (independent, LSI clock) ¦ WWDG (windowed, APB clock) | IWDG timeout | IWDG_PR + IWDG_RLR: 1ms–32s range |
| Brown-out reset BOR | Resets if VDD falls below threshold | BOR levels | STM32: BOR0-3 (1.8-2.9V configurable) |
| Option bytes | Configure BOR, WDG, RDP, nBOOT0 in Flash option area | RDP levels | 0=open, 1=no debug read, 2=permanent lock |
| CRC hardware | Built-in CRC32 unit — verify firmware images | Reset cause | RCC->CSR: PINRSTF, BORRSTF, IWDGRSTF, WWDGRSTF |
| NOTE: Enable BOR, IWDG, and stack watermark monitoring in all production firmware. Use ITM/SWO during development — remove for release. Always decode HardFault CFSR in the fault handler. |  |  |  |

# 11. SEMICONDUCTOR MEMORIES

| MEMORY TYPES COMPARISON |  |  |  |
| --- | --- | --- | --- |
| SRAM | Static, 6T cell, fast (ns), volatile, costly | DRAM | Dynamic, 1T+1C, needs refresh, dense |
| SDRAM | Synchronous DRAM, burst access, DDR/DDR4 | EEPROM | Electrically erasable, byte-write, slow |
| NOR Flash | Random access, fast read, slow write/erase | NAND Flash | Page/block access, dense, fast write |
| FRAM (FeRAM) | Non-volatile, 10^12 writes, fast | MRAM | Magnetic, non-volatile, unlimited writes |
| KEY PARAMETERS |  |  |  |
| Access time ta | Address valid → data valid | Cycle time tc | Min time between accesses (tc ≥ ta) |
| Read latency CL | Clock cycles to first data (DRAM) | Burst length BL | Words transferred per command |
| Endurance | Erase/write cycles before wear-out | Retention | Years data retained without power |
| Write amplification | Physical writes / logical writes | Wear leveling | Distribute writes across flash cells |
| MEMORY HIERARCHY |  |  |  |
| L1 Cache | On-die, ~1ns, 16-64 KB per core | L2 Cache | On/near die, ~5ns, 256KB-4MB |
| L3 Cache | Shared, ~15ns, 4-64 MB | Main RAM | Off-chip DRAM, ~60ns, 4-64 GB |
| Cache hit rate H | H = hits / total accesses | AMAT | H×Tc + (1-H)×Tm |
| Direct-mapped | 1-way, simple, conflict misses | N-way set-assoc | N lines per set, LRU eviction |
| NAND FLASH OPERATIONS |  |  |  |
| Page size | 512B – 16KB typical | Block = N pages | Erase unit (64-256 pages typical) |
| ECC | Error correction required (BCH/LDPC) | Bad block mgmt | Mark bad at factory + runtime |
| FTL | Flash Translation Layer: logical→physical | JEDEC standards | eMMC, UFS, SD card interfaces |

| PARALLEL MEMORY INTERFACES |  |  |  |
| --- | --- | --- | --- |
| PARALLEL BUS SIGNALS |  |  |  |
| Address bus A[n:0] | Selects memory location: 2^n total locations | Data bus D[m:0] | Width m+1 bits (8/16/32 bit wide) |
| Chip Enable /CE | Activates device (active low) | Output Enable /OE | Enables data output onto bus (active low) |
| Write Enable /WE | Enables write cycle (active low) | Byte Enable /BEx | Selects byte lane on 16/32-bit wide bus |
| PARALLEL SRAM (e.g. IS62WV25616BLL — 256Kx16) |  |  |  |
| Access time tAA | Address stable → valid data: 45-70ns typical | CE access tACS | CE low → valid data: same as tAA |
| Cycle time tRC | Min time between read starts (≥ tAA) | Write cycle tWC | Min write cycle time |
| Read cycle | /CE low + /OE low → data valid after tAA | Write cycle | /CE low + /WE pulse → data latched on /WE rise |
| Density example | IS62WV25616: 256K addr × 16-bit = 512KB | Bus interface | STM32 FMC/FSMC — auto generates timing signals |
| PARALLEL NOR FLASH (e.g. S29GL064S — 64Mbit) |  |  |  |
| Read access | Same as SRAM read — direct random access (XIP) | Sector erase | Cmd sequence: 0xAA→0x55→0x80→0xAA→0x55→0x30 |
| Word program | Cmd sequence: 0xAA→0x55→0xA0→data | Program time | ≈ 9µs per word typical |
| CFI query | Common Flash Interface: ID geometry + timing params | Erase time | ≈ 500ms per sector (64KB) |
| XIP (execute) | CPU fetches instructions directly from NOR (no copy) | Bus width | x8 (byte wide) or x16 (word wide) |
| PARALLEL DRAM / SDRAM (e.g. W9825G6KH — 256Mb) |  |  |  |
| RAS (Row Address Str) | Latches row address on CAS pin bundle | CAS (Col Address) | Latches column address after RAS |
| Refresh requirement | Each row must be refreshed every 64ms (4096 rows) | Auto-refresh | SDRAM controller sends AUTO REFRESH cmd |
| CAS latency CL | Clocks from READ cmd to first data: CL=2 or 3 | Burst mode | Up to 8 words per command cycle |
| RAS to CAS delay tRCD | Min time between RAS and CAS assertion | Row precharge tRP | Time to deactivate row before next activate |
| SDRAM Init seq | Power-on → 200µs wait → PRECHARGE ALL → 8× AUTO-REF → LOAD MODE REG | Mode reg | Set CL, burst length, burst type |
| STM32 FMC (Flexible Memory Controller) CONFIGURATION |  |  |  |
| FMC banks | Bank1: NOR/SRAM ¦ Bank2: NAND ¦ Bank3: SDRAM | Address mapping | NE1: 0x60000000, NE2: 0x64000000... |
| SRAM timing regs | ADDSET, ADDHLD, DATAST, BUSTURN in BCR/BTR | SDRAM timing | TMRD, TXSR, TRAS, TRC, TWR, TRP, TRCD in SDTR |
| FMC clock | SDRAM: FMC_CLK = HCLK/2 or /3 (max 100MHz) | Refresh rate | Set FMC_SDRTR: Count = (tREFI × FMC_CLK) - 20 |
| PARALLEL vs SERIAL MEMORY COMPARISON |  |  |  |
| Parallel SRAM | Fast (≤70ns), simple, many pins (26-32 signal lines) | Serial SRAM (SPI) | Slower, 4-6 pins, up to 104MHz SPI |
| Parallel NOR | XIP capable, fast read, many pins | Serial NOR (QSPI) | 4-bit QSPI: execute-in-place via memory-mapped |
| Use parallel when | < 100ns latency required, large bandwidth (LCD, DSP) | Use serial when | Pin-count constrained, medium speed OK |
| NOTE: Most modern MCUs use QSPI/OctoSPI for external Flash (fewer pins, DMA, memory-mapped XIP). Use parallel SRAM only when access time < 70ns is critical or when existing PCB dictates it. |  |  |  |

# 12. INDUSTRIAL COMMUNICATION PROTOCOLS

| MODBUS |  |  |  |
| --- | --- | --- | --- |
| Modbus RTU | Serial (RS-232/485), binary, CRC-16 | Modbus TCP | Ethernet, port 502, no CRC (TCP handles) |
| Modbus ASCII | Serial, ASCII chars, LRC checksum | Max slaves RTU | 247 addresses (1-247) |
| Function 01 | Read Coils (discrete output) | Function 02 | Read Discrete Inputs |
| Function 03 | Read Holding Registers (16-bit) | Function 04 | Read Input Registers |
| Function 05 | Write Single Coil | Function 06 | Write Single Register |
| Function 15 | Write Multiple Coils | Function 16 | Write Multiple Registers |
| RTU timeout | 3.5 char silence = frame boundary | Modbus frame | Addr(1)+FC(1)+Data(N)+CRC(2) |
| CAN BUS |  |  |  |
| CAN 2.0A (std) | 11-bit identifier, up to 1 Mbps | CAN 2.0B (ext) | 29-bit identifier |
| CAN FD | Up to 8 Mbps data phase, 64-byte payload | Dominant bit | Logic 0 (bus driven low) |
| Arbitration | CSMA/CA: lowest ID wins (non-destructive) | ACK bit | Receiver pulls dominant |
| Max nodes | ~127 (practical, bus load dependent) | Max cable (1Mbps) | ~40 m |
| CRC | CAN: 15-bit CRC, FD: 17/21-bit | Termination | 120Ω at each end of bus |
| PROFIBUS & PROFINET |  |  |  |
| PROFIBUS DP | RS-485, up to 12 Mbps, 126 nodes | PROFIBUS PA | MBP, intrinsically safe, 31.25 kbps |
| PROFINET | Industrial Ethernet (100M/1G), IRT/RT/NRT | IRT cycle time | < 1 ms (sync'd Ethernet) |
| GSD file | Device description for config tool | Slot/Module | PROFIBUS modular addressing |
| OTHER FIELDBUS PROTOCOLS |  |  |  |
| DeviceNet | CAN-based, 500kbps, 64 nodes, ODVA | EtherNet/IP | CIP over Ethernet, ODVA |
| EtherCAT | Ethernet slave-to-slave, <100µs cycle | Modbus RTU baud | 9.6k / 19.2k / 38.4k / 115.2k |
| RS-232 levels | ±3 to ±15V, point-to-point, <15m | RS-485 levels | ±1.5V diff, 32-256 nodes, 1200m |
| RS-422 | Differential, full-duplex, 1200m @ 100kbps | LIN bus | Single-wire, 20kbps, automotive |
| NOTE: Modbus is the most widely used in Africa/SA industry. CAN is dominant in automotive. PROFIBUS/PROFINET in Siemens PLC environments. |  |  |  |

# 13. DC METERING

| DC MEASUREMENT BASICS |  |  |  |
| --- | --- | --- | --- |
| Voltmeter Rin | Rin → ∞ ideal (typ >10MΩ) | Ammeter Rin | Rin → 0 ideal (typ <1Ω) |
| Loading error % | err = Rin_meter/(Rin_meter + Rcircuit) - 1 | 4-wire (Kelvin) | Eliminates lead resistance in R meas. |
| SHUNTS & MULTIPLIERS |  |  |  |
| Ammeter shunt Rs | Rs = Ifs × Rm / (I - Ifs) | Voltmeter Rm_series | Rseries = (V/Ifs) - Rm |
| Shunt power | Ps = I² × Rs | Multiplier factor | m = I_full_scale / Ifs |
| BRIDGE CIRCUITS |  |  |  |
| Wheatstone balance | R1/R2 = R3/Rx → Rx = R2R3/R1 | Sensitivity S | S = ΔVout / ΔR/R |
| Kelvin bridge | Low resistance measurement (<1Ω) | Maxwell bridge | Measures inductance L |
| Schering bridge | Measures capacitance & tan δ | Wien bridge | Frequency measurement |
| CURRENT SENSING |  |  |  |
| Shunt resistor | V = I × Rshunt (typ 1-100 mΩ) | Hall-effect sensor | Isolated, I = B×d/(µ0×Hall const) |
| CT (Current Transf) | Is = Ip × Np/Ns (ideal) | Rogowski coil | di/dt measurement, no core sat. |
| DC POWER METERS |  |  |  |
| DC Power | P = V × I | Energy meter count | E = P × t [Wh or kWh] |
| Battery SOC | Coulomb counting: SOC = SOC0 + ∫I dt / Qrated | Battery SoH | SoH = Qactual/Qnominal × 100% |
| Resistance (4W) | R = V_sense / I_force | DMM accuracy | % reading + % full scale + digits |
| NOTE: Always use 4-wire Kelvin connection for R < 1Ω. CT secondary must NEVER be open-circuited (dangerous HV). |  |  |  |

# 14. AC METERING

| AC MEASUREMENT FUNDAMENTALS |  |  |  |
| --- | --- | --- | --- |
| True RMS V | Vrms = √(1/T ∫v²dt) | True RMS I | Irms = √(1/T ∫i²dt) |
| True Power P | P = (1/T) ∫v×i dt [W] | Reactive Power Q | Q = √(S²-P²) [VAR] |
| Apparent Power S | S = Vrms × Irms [VA] | Power Factor PF | PF = P/S = cosφ |
| Displacement PF | DPF = cos(φ₁) (fundamental only) | True PF (non-linear) | PF = DPF / √(1+THD²) |
| ENERGY METERING (IEC 62053 / SANS 474) |  |  |  |
| Active energy | Wh = P × t (kWh for billing) | Reactive energy | VARh = Q × t |
| Class 0.5S | ±0.5% accuracy (utility meters) | Class 1 | ±1% (industrial/commercial) |
| Meter constant | Pulses per kWh (e.g. 1600 imp/kWh) | Calibration pulse | LED/optical output for testing |
| SA Utility std | NRS 057 / NRS 097 smart metering | Tamper detect | Magnetic, neutral switch, reversal |
| INSTRUMENT TRANSFORMERS |  |  |  |
| CT ratio | e.g. 100/5A (turns ratio = 20) | CT class 0.5 | ±0.5% ratio error (metering CT) |
| VT/PT ratio | e.g. 11kV/110V (step-down) | VT burden | Rated VA load on secondary |
| CT burden | Max impedance on secondary (Ω or VA) | CT saturation | Avoid: use C-class protection CTs |
| POWER QUALITY PARAMETERS |  |  |  |
| THD-V limit | < 5% total (IEC 61000-2-2) | Flicker Pst/Plt | Short/long-term flicker severity |
| Voltage unbalance | VUF% = Vneg/Vpos × 100% | Frequency tolerance | 50 Hz ± 0.5 Hz (NRS 048) |
| Harmonics | fn = n × f0 (n=1 fund; 3,5,7 odd dominant) | Notch depth | Line notch depth < 20% (IEEE 519 |
| Sag (dip) | RMS drop 10-90%, 10ms-1min | Swell | RMS rise > 110%, < 1min |
| NOTE: South Africa: NRS 048-2 defines PQ limits. Eskom supply frequency 50Hz. Metering to NRS 057 / SANS 62053 standards. |  |  |  |

# 15. WEBSOCKETS

| WEBSOCKET PROTOCOL (RFC 6455) |  |  |  |
| --- | --- | --- | --- |
| Protocol | ws:// (plain) ¦ wss:// (TLS) | Default ports | ws: 80, wss: 443 |
| Upgrade header | HTTP GET + Upgrade: websocket | Sec-WS-Key | Base64(16 random bytes) |
| Sec-WS-Accept | Base64(SHA1(key + GUID)) | GUID | 258EAFA5-E914-47DA-95CA-C5AB0DC85B11 |
| 101 response | HTTP 101 Switching Protocols | Full-duplex | Simultaneous send & receive |
| FRAME FORMAT |  |  |  |
| Frame header | FIN(1)+RSV(3)+Opcode(4)+MASK(1)+Len(7) | Payload len | 7-bit; 126→16-bit ext; 127→64-bit ext |
| Opcode 0x1 | Text frame (UTF-8) | Opcode 0x2 | Binary frame |
| Opcode 0x8 | Close frame | Opcode 0x9/0xA | Ping / Pong (keepalive) |
| Client masking | Client→Server always masked (XOR key) | Server masking | Server→Client unmasked |
| CONNECTION LIFECYCLE |  |  |  |
| Handshake | HTTP→WS upgrade (one-time) | Heartbeat | Ping/Pong every 30-60s typical |
| Close handshake | Close frame exchange before TCP close | Status 1000 | Normal closure |
| Status 1001 | Going away (server restart) | Status 1008 | Policy violation |
| Reconnect strategy | Exponential backoff: 1s,2s,4s,8s,max32s | Sub-protocols | Sec-WebSocket-Protocol header |
| PERFORMANCE & SCALING |  |  |  |
| Overhead vs HTTP | 2-14 bytes/frame vs 200-800 bytes HTTP header | Max connections | ~65k per server IP:port (OS limit) |
| Load balancing | Sticky sessions required (same server) | STOMP | Messaging protocol over WebSocket |
| Socket.IO | WS + long-polling fallback + rooms | Binary WS | ArrayBuffer / Blob for raw data |

# 16. RESTful API

| REST PRINCIPLES (Richardson Maturity Model) |  |  |  |
| --- | --- | --- | --- |
| REST Level 0 | Single URI, all via POST (SOAP/RPC-like) | REST Level 1 | Resource URIs (per-entity) |
| REST Level 2 | HTTP verbs (GET/POST/PUT/DELETE) | REST Level 3 | HATEOAS (self-describing links) |
| Stateless | No client session on server (each req complete) | Cacheable | Responses declare cacheability |
| Uniform Interface | Standard HTTP methods + media types | Layered System | Client unaware of intermediaries |
| HTTP METHODS |  |  |  |
| GET | Read resource (safe + idempotent) | POST | Create (not idempotent) |
| PUT | Full update/replace (idempotent) | PATCH | Partial update (not always idempotent) |
| DELETE | Remove resource (idempotent) | HEAD | GET without body (check existence) |
| OPTIONS | Returns allowed methods (CORS preflight) | Idempotent | Same result regardless of repeat calls |
| HTTP STATUS CODES |  |  |  |
| 200 OK | Success | 201 Created | POST success, Location header |
| 204 No Content | DELETE/PUT success, no body | 400 Bad Request | Invalid input/syntax |
| 401 Unauthorized | Auth required (missing/invalid token) | 403 Forbidden | Auth OK but insufficient rights |
| 404 Not Found | Resource does not exist | 409 Conflict | Duplicate / state conflict |
| 422 Unprocessable | Validation failed | 429 Too Many Requests | Rate limit exceeded |
| 500 Internal Error | Server fault | 503 Service Unavail. | Maintenance / overload |
| AUTHENTICATION & SECURITY |  |  |  |
| JWT structure | Header.Payload.Signature (Base64url) | JWT verify | HMAC-SHA256 or RSA signature |
| Bearer token | Authorization: Bearer <token> | Refresh token | Long-lived, rotated on use |
| OAuth 2.0 flow | Auth Code → Token Exchange → Access | API Key | Simple, static, no expiry |
| HTTPS | All APIs must use TLS 1.2+ (no HTTP) | CORS | Access-Control-Allow-Origin header |
| API DESIGN BEST PRACTICES |  |  |  |
| Versioning | /api/v1/resource or Accept: vnd+v1 | Pagination | ?page=1&limit=50 or cursor-based |
| Filtering | ?status=active&type=sensor | Sorting | ?sort=created_at&order=desc |
| Rate limiting | X-RateLimit-Limit / Remaining / Reset | Idempotency key | Idempotency-Key: <uuid> (POST) |

# 17. PoE STANDARDS (Power over Ethernet)

| IEEE PoE STANDARDS OVERVIEW |  |  |  |
| --- | --- | --- | --- |
| IEEE 802.3af (PoE) | Max 15.4W PSE / 12.95W PD | IEEE 802.3at (PoE+) | Max 30W PSE / 25.5W PD |
| IEEE 802.3bt Type 3 | 60W PSE / 51W PD (4-pair) | IEEE 802.3bt Type 4 | 90W PSE / 71.3W PD |
| Cisco UPOE | Proprietary 60W (pre-bt) | PoE++ (bt) | Also called Hi-PoE |
| ELECTRICAL PARAMETERS |  |  |  |
| PSE Voltage | 44-57 VDC (af/at) / 50-57V (bt) | PD Input Voltage | 37-57V (after cable drop) |
| Cable resistance | ~12.5Ω/100m (Cat5e pair) | Max cable drop | 7V typical @ rated current |
| Pair usage (af/at) | Alt A: pins 1,2,3,6 ¦ Alt B: 4,5,7,8 | bt pair usage | All 4 pairs simultaneously |
| PD signature R | 25 kΩ (±1kΩ) detection R | Classification | 0-1mA to 73mA pulses (Class 0-8) |
| POWER CLASSES |  |  |  |
| Class 0 (af) | 0.44-12.95W (unclassified) | Class 1 (af) | 0.44-3.84W |
| Class 2 (af) | 3.84-6.49W | Class 3 (af/at) | 6.49-25.5W |
| Class 4 (at) | 12.95-25.5W | Class 5 (bt) | up to 40W at PD |
| Class 6 (bt) | up to 51W at PD | Class 7 (bt) | up to 62W at PD |
| Class 8 (bt) | up to 71.3W at PD | LLDP / CDP negotiation | Software power class override |
| CONNECTOR TYPES, CURRENT RATINGS & CABLE SPECIFICATIONS |  |  |  |
| RJ-45 (8P8C) | Standard connector — all PoE standards (af/at/bt) | Contact rating | 1.5 A per pin max (UL/IEC) |
| Max current (af/at) | 600 mA per pair (2 conductors) | Max current (bt) | 960 mA per pair — all 4 pairs active |
| Total bt conductor I | Up to 960 mA per conductor | Total bt port current | ~1.92 A aggregate (4 pairs) |
| Cat5e (24 AWG UTP) | 100Ω ±15%, max 100m, up to 30W safe | Max I Cat5e | 360 mA/conductor @ 60°C ambient |
| Cat6 (23 AWG UTP) | 100Ω ±15%, lower DCR, up to 60W | Max I Cat6 | 600 mA/conductor, lower temp rise |
| Cat6A (23 AWG UTP/STP) | Required for 802.3bt Type 3/4 (90W) | Max I Cat6A | 720 mA/conductor, best thermal perf |
| Cat7 / Cat8 (shielded) | STP/SFTP, Cat8=40GbE/30m; PoE supported | Connector | Still RJ-45 plug on field end |
| M12 D-coded (4-pin) | IP67 industrial Ethernet, up to 1GbE | Max I M12-D | 4 contacts × 4A (power pins shrd) |
| M12 X-coded (8-pin) | IP67/IP69K, 10GbE + PoE bt, IEC 61076-2-109 | Max I X-coded | Up to 4A per power contact |
| M12 L-coded | Emerging: PoE + data + power combined | Use case | Industrial robots, AGVs, field sensors |
| HDBaseT (RJ-45) | Proprietary — up to 100W / 100m | Use case | AV-over-IP, displays, collaboration |
| Cisco UPOE (RJ-45) | CDP/LLDP negotiates 60W on 4 pairs | Use case | High-power IP phones, thin clients |
| CABLE THERMAL DERATING |  |  |  |
| Bundled cable derating | ≥24 cables bundled: derate 50% current | Conduit | Per NEC 310.15 / IEC 60364-5-52 |
| Cat5e 100m loss @ 15W | ≈1.2W cable dissipation | Cat6A 100m @ 90W | ≈8.1W cable loss — verify budget |
| Temp rise formula | ΔT = I² × R_loop × θ_thermal | R loop (Cat5e 100m) | ~5Ω per pair (both conductors) |
| DESIGN RULES |  |  |  |
| PSE midspan | In-line injector (no PoE switch) | PSE endpoint | PoE integrated in managed switch |
| PD isolation | 1500V galvanic isolation required | Inrush limit | PD must limit inrush < 400mA |
| Power budget rule | PSE_total × η ≥ Σ PD_watts | Headroom | Always design with 20% spare |
| NOTE: Use Cat6A for all new 802.3bt installations. In SA harsh environments (mines, outdoors) use M12 X-coded IP67 connectors. Verify per-port AND total switch chassis PoE budget separately. |  |  |  |

| SMART PoE — NEGOTIATED VOLTAGE MODES & TYPES |  |  |  |
| --- | --- | --- | --- |
| STANDARD IEEE 802.3 PoE VOLTAGE NEGOTIATION |  |  |  |
| Passive PoE | Fixed voltage (12V/24V/48V) — NO detection/classification | Risk | Always-on — can damage non-PoE devices |
| IEEE 802.3af Active | Detection → Classification → 48V (44-57V) power-on | Safe | Only powers devices that pass detection |
| IEEE 802.3at Active | Same detection + 2-event Class for 30W | Voltage | 44-57V PSE output |
| IEEE 802.3bt Active | Physical layer + LLDP/CDP negotiation for 60W/90W | Voltage | 50-57V PSE output |
| NON-STANDARD SMART PoE VOLTAGE SYSTEMS |  |  |  |
| 24V Passive PoE | Ubiquiti airMAX default: 24V on pins 4,5(+) 7,8(−) | Max power | ≈24V × 1A = 24W (cable limited) |
| 48V Passive PoE | Cameras, APs: 48V on 4,5(+) 7,8(−) or 1,2(+) 3,6(−) | Compatibility | Only compatible devices — always check first |
| Ubiquiti 24V PoE | Proprietary 24V passive — incompatible with 802.3af/at | Adapter | Use Ubiquiti POE-24-24W or INS-8023AF-I |
| Cisco UPOE (60W) | CDP negotiated — extends 802.3at to 60W using all pairs | Voltage | 50-57V, same as standard |
| PoE++(bt) auto-neg. | LLDP-MED + 802.3bt Layer-1 class — up to 90W | Class 8 Vport | 51-57V at port, derates with cable |
| HDBaseT Alliance | Up to 100W / 100m — 5Play (video+audio+USB+Eth+PoE) | Voltage | Proprietary, NOT 802.3 compatible |
| PoE over Coax | e-PoC (Ethernet over Coax + power) — BNC connector | Voltage | 48V DC on coax; used in CCTV retrofit |
| Industrial 24V DC PoE | PLC-side: 24V DC supply + Ethernet separate (not true PoE) | Standard | IEC 62443 industrial networks — IS1 separation |
| VOLTAGE AT PD vs CABLE LENGTH (802.3af/at) |  |  |  |
| PSE output | 44-57V DC (typical 48V nominal) | Cable drop | ΔV = I × R_loop (R_loop ≈ 25Ω/100m Cat5e) |
| PD min at 0m | 44V | PD min at 100m | ~37V (after max cable drop at rated current) |
| PD must accept | 37-57V input range (bridge rectifier + DCDC converter) | Efficiency | PD converter η typically 85-92% |
| Power at PD (af) | 12.95W = 15.4W × (1 - cable_loss fraction) | Power at PD (at) | 25.5W delivered after cable loss |
| SMART POE DETECTION & CLASSIFICATION SEQUENCE |  |  |  |
| Step 1 Detect | PSE applies 2.7-10V probe ¦ PD must present 25kΩ ±1kΩ signature resistor |  |  |
| Step 2 Class | PSE applies 15.5-20.5V ¦ PD draws class current: Class0=0-4mA, Class3=26-30mA, Class8=58-73mA |  |  |
| Step 3 Power-on | PSE ramps to full voltage (44-57V) after valid class detected |  |  |
| Step 4 LLDP | Optional: LLDP-MED packets negotiate exact power level (bt Class5-8) |  |  |
| Step 5 Monitor | PSE monitors load: if I < 10mA for >300ms → PSE removes power (device disconnect) |  |  |
| NOTE: Never mix passive PoE voltages on the same switch — 24V and 48V passive PoE will damage incompatible devices. Always use active IEEE 802.3 PoE where possible. For SA installations: verify that PoE injectors/switches are 230V-rated (50Hz). |  |  |  |

# 18. MEMORY MAP — ROM, RAM, SRAM, EEPROM

| Region / Type | Base Address | End Address | Size | Notes / Contents |
| --- | --- | --- | --- | --- |
| STM32H7 (Cortex-M7 + M4) MEMORY MAP — Typical @ 0x00000000 Base |  |  |  |  |
| FLASH (ROM) — Execute-in-Place, non-volatile |  |  |  |  |
| Flash Bank 1 | 0x0800 0000 | 0x080F FFFF | 1 MB | Main program code / ISR vectors |
| Flash Bank 2 | 0x0810 0000 | 0x081F FFFF | 1 MB | Dual-bank (OTA, swap) |
| Flash OTP | 0x1FF0 7800 | 0x1FF0 7A0F | 528 B | One-time programmable area |
| Flash Option | 0x1FF0 7800 | 0x1FF0 783F | 64 B | RDP, BOR, WDG config bits |
| SRAM — Tightly Coupled & General Purpose (volatile, fast) |  |  |  |  |
| DTCM-RAM | 0x2000 0000 | 0x2001 FFFF | 128 KB | M7 data TCM — 0-wait state, DMA-able |
| ITCM-RAM | 0x0000 0000 | 0x0000 FFFF | 64 KB | M7 instr TCM — fastest fetch |
| AXI SRAM (D1) | 0x2400 0000 | 0x2407 FFFF | 512 KB | General purpose, DMA2D, MDMA |
| SRAM1 (D2) | 0x3000 0000 | 0x3001 FFFF | 128 KB | Peripheral DMA bus |
| SRAM2 (D2) | 0x3002 0000 | 0x3003 FFFF | 128 KB | Peripheral DMA bus |
| SRAM3 (D2) | 0x3004 0000 | 0x3004 FFFF | 32 KB | USB & Ethernet DMA |
| SRAM4 (D3) | 0x3800 0000 | 0x3800 FFFF | 64 KB | Low-power domain, BDMA |
| Backup SRAM | 0x3880 0000 | 0x3880 0FFF | 4 KB | Retained in VBAT / Standby |
| EEPROM (Emulated in Flash) / External EEPROM |  |  |  |  |
| Emulated EE | 0x0808 0000 | 0x0809 FFFF | 128 KB | Flash pages used as EEPROM (HAL_FLASH_EE) |
| Ext I2C EEPROM | Device addr | 0x50–0x57 | 1–512 KB | AT24Cxx: 7-bit I2C, 400 kHz |
| Ext SPI EEPROM | CS select | 0x00–0xFFFF | 64 KB | 25LCxxx series, SPI, 10M writes |
| PERIPHERAL REGISTERS (Memory-Mapped I/O) |  |  |  |  |
| APB1 Periph | 0x4000 0000 | 0x4000 7FFF | 32 KB | UART2-8, SPI2-3, I2C, TIM2-7 |
| APB2 Periph | 0x4001 0000 | 0x4001 7FFF | 32 KB | UART1, SPI1, TIM1/8, ADC1-3 |
| AHB1 Periph | 0x4002 0000 | 0x4002 1FFF | 8 KB | DMA1/2, DMAMUX, ETH, MDMA |
| AHB2 Periph | 0x4802 0000 | 0x4802 1FFF | 8 KB | GPIO A-K, RNG, SDMMC |
| AHB3 Periph | 0x5100 0000 | 0x5100 3FFF | 16 KB | MDMA, JPEGC, LTDC, QUADSPI |
| Cortex-M SCS | 0xE000 E000 | 0xE000 EFFF | 4 KB | NVIC, SysTick, SCB, MPU, FPU |
| GENERIC MCU MEMORY MAP RULES |  |  |  |  |
| AVR ATmega328P Flash | 0x0000 | 0x7FFF | 32 KB | Harvard: code space (word addressed) |
| AVR ATmega328P SRAM | 0x0100 | 0x08FF | 2 KB | Data + stack (grows downward) |
| AVR ATmega328P EEPROM | 0x000 | 0x3FF | 1 KB | 8-bit EEPROM, 100k write cycles |
| ESP32 IRAM | 0x4007 0000 | 0x4009 FFFF | 192 KB | Instruction RAM (time-critical ISR) |
| ESP32 DRAM | 0x3FFB 0000 | 0x3FFF FFFF | 320 KB | Data RAM (heap+stack) |
| ESP32 Flash | 0x400C 2000 | 0x40BF FFFF | ~4 MB | External SPI Flash, MMU-mapped |

# 18b. BITWISE DATA SCALING — 64-bit Down to 4-bit

| BIT-WIDTH TYPES, RANGES & SCALING OPERATIONS |  |  |  |
| --- | --- | --- | --- |
| uint64_t | 0 to 18,446,744,073,709,551,615 | int64_t | -9.22×10¹⁸ to +9.22×10¹⁸ |
| uint32_t | 0 to 4,294,967,295 | int32_t | -2,147,483,648 to 2,147,483,647 |
| uint16_t | 0 to 65,535 | int16_t | -32,768 to 32,767 |
| uint8_t | 0 to 255 | int8_t | -128 to 127 |
| uint4_t (nibble) | 0 to 15 (4 bits) | Nibble extract | (val >> shift) & 0x0F |
| SHIFT OPERATIONS — EXTRACT ANY FIELD FROM 64-BIT VALUE |  |  |  |
| 64→32 upper half | uint32_t hi = (uint32_t)(val >> 32); | 64→32 lower half | uint32_t lo = (uint32_t)(val & 0xFFFFFFFF); |
| 64→16 byte[3] | (uint16_t)((val >> 48) & 0xFFFF) | 64→16 byte[2] | (uint16_t)((val >> 32) & 0xFFFF) |
| 64→16 byte[1] | (uint16_t)((val >> 16) & 0xFFFF) | 64→16 byte[0] | (uint16_t)(val & 0xFFFF) |
| 64→8 byte n | (uint8_t)((val >> (n*8)) & 0xFF) | Loop all 8 bytes | for(i=7;i>=0;i--) b[i]=val>>(i*8) |
| 64→4 nibble n | (uint8_t)((val >> (n*4)) & 0x0F) | Total nibbles | 16 nibbles in a 64-bit value |
| BITWISE OPERATORS — C SYNTAX |  |  |  |
| AND & | Mask / clear bits: val & 0x0F | OR ¦ | Set bits: val ¦ 0x80 |
| XOR ^ | Toggle bits: val ^ 0xFF | NOT ~ | Invert all: ~val |
| Left shift << | val << n ≡ val × 2ⁿ (fast multiply) | Right shift >> | val >> n ≡ val / 2ⁿ (unsigned) |
| Set bit n | val ¦= (1ULL << n) | Clear bit n | val &= ~(1ULL << n) |
| Toggle bit n | val ^= (1ULL << n) | Test bit n | if (val & (1ULL << n)) |
| SCALING — PROPORTIONAL VALUE MAPPING |  |  |  |
| Linear scale | out = (in * out_max) / in_max | ADC→Engineering | V = (adc_raw * Vref) / (2^bits - 1) |
| 64-bit to 4-bit | nibble = (uint8_t)((val * 15ULL) / in_max) & 0x0F | 4-bit to percent | pct = (nibble * 100) / 15 |
| Fixed-point Q16 | val_fp = (int32_t)(float_val * 65536.0f) | Q16 back to float | f = (float)val_fp / 65536.0f |
| Q format Q(m,n) | n frac bits: LSB = 2^-n; range ±2^(m-1) | Q15 range | -1.0 to +0.99997 (DSP audio) |
| FULL EXAMPLE — 64-BIT SENSOR WORD DECODE |  |  |  |
| 64-bit word layout | [63:48] Timestamp ¦ [47:32] Temp×100 ¦ [31:16] Voltage×1000 ¦ [15:8] Status ¦ [7:4] Mode ¦ [3:0] Channel |  |  |
| Extract Channel | uint8_t ch = (uint8_t)(sensor_word & 0x0F); |  |  |
| Extract Mode | uint8_t mode = (uint8_t)((sensor_word >> 4) & 0x0F); |  |  |
| Extract Status | uint8_t stat = (uint8_t)((sensor_word >> 8) & 0xFF); |  |  |
| Extract Voltage | float vout = ((sensor_word >> 16) & 0xFFFF) / 1000.0f; |  |  |
| Extract Temp | float temp = (int16_t)((sensor_word >> 32) & 0xFFFF) / 100.0f; |  |  |
| Extract Time | uint16_t ts = (uint16_t)((sensor_word >> 48) & 0xFFFF); |  |  |
| NOTE: Always use 1ULL<<n (not 1<<n) for 64-bit shifts. Use uint64_t for portability. On 32-bit MCUs, 64-bit ops compile to 2 instructions. |  |  |  |

# 19. ARM CORTEX-M7 DUAL CORE (STM32H7)

| CORE ARCHITECTURE COMPARISON |  |  |  |
| --- | --- | --- | --- |
| Core | Cortex-M7 (CM7) — Master | Core | Cortex-M4 (CM4) — Co-processor |
| Architecture | ARMv7-M, 6-stage superscalar | Architecture | ARMv7-M, 3-stage pipeline |
| Max Clock | 480 MHz (STM32H755) | Max Clock | 240 MHz (STM32H755) |
| Performance | 3.4 DMIPS/MHz + 7.67 CoreMark/MHz | Performance | 1.25 DMIPS/MHz + 3.34 CoreMark/MHz |
| FPU | Double precision (DP-FPU) + DSP SIMD | FPU | Single precision (SP-FPU) + DSP |
| TCM | 64KB ITCM + 128KB DTCM (0-wait) | TCM | No TCM |
| Cache | 16KB I-cache + 16KB D-cache | Cache | No cache (direct SRAM) |
| MPU | 16 regions | MPU | 8 regions |
| BOOT & STARTUP SEQUENCE (STM32H755) |  |  |  |
| Boot pins BOOT0 | 0: Flash boot (normal) | BOOT0=1 | System memory (ROM bootloader) |
| CM7 boots first | Starts at 0x08000000 (Flash Bank 1) | CM4 held | In reset until CM7 releases |
| CM7 releases CM4 | HAL_RCCEx_EnableBootCore(RCC_BOOT_C2) | CM4 starts | From 0x08100000 (Bank 2) or RAM |
| CM7 vector table | VTOR = 0x08000000 | CM4 vector table | VTOR = 0x08100000 (or configured) |
| INTER-CORE COMMUNICATION (AMP — Asymmetric Multi-Processing) |  |  |  |
| HSEM (HW Semaphore) | 32 semaphores for mutual exclusion | HSEM take | HSEM->RLR[n] == COREID<<8¦PROCID |
| HSEM ISR notify | HSEM_IRQn triggers on release | Shared memory | Use SRAM1/SRAM2/AXI SRAM (no cache) |
| OpenAMP/virtIO | RPMSG for message-based IPC | Mailbox | IPCC peripheral: 6 channels bidirec. |
| Cache coherency | CM7 must flush/inv D-cache before sharing | D-cache flush | SCB_CleanDCache_by_Addr(ptr, size) |
| D-cache invalidate | SCB_InvalidateDCache_by_Addr(ptr,sz) | CM4 rule | CM4 accesses SRAM4/D3 directly (no cache issue) |
| CLOCK DOMAINS |  |  |  |
| D1 domain (CM7) | 480 MHz max, AXI/AHB3, LTDC, GPU | D2 domain (CM4) | 240 MHz, APB1/APB2, UART, SPI, I2C |
| D3 domain | Low-power, runs when D1/D2 in Standby | HSE crystal | 25 MHz external → PLL → sysclk |
| PLL1 | → CM7 core + AXI + Flash | PLL2 | → USB, ADC, SDMMC, SPI |
| PLL3 | → UART, I2C, LP timers | LSE | 32.768 kHz → RTC, LPTIM |
| MEMORY PROTECTION UNIT (MPU) ATTRIBUTES |  |  |  |
| TEX+C+B+S | Cache policy bits for each region | XN (eXec Never) | Prevent code exec from data regions |
| Strongly-ordered | No cache, no buffer, sync (periph regs) | Device | No cache, buffered (AHB periph) |
| Normal cached | WBWA (Write-Back, Write-Alloc) — RAM | Shared | Set S-bit for inter-core shared SRAM |
| CORTEX-M7 PERFORMANCE FEATURES |  |  |  |
| Branch prediction | 2-level adaptive predictor | Speculative fetch | Pre-fetches target before branch resolves |
| Superscalar | Dual-issue: 2 instructions/cycle | Write buffer | 8-entry store buffer (non-blocking) |
| FPU latency | DP FADD: 3cy, FMUL: 5cy, FDIV: 15cy | SIMD | UADD8, SADD16 parallel byte/halfword ops |
| ECC | Flash + TCM ECC (error correction) | Parity | AXI SRAM parity protection |
| NOTE: Place time-critical CM7 ISRs in ITCM (0x00000000). Place DMA buffers in non-cacheable SRAM (use __attribute__((section('.dma_buffer')))). |  |  |  |

# 20. EMBEDDED C — SYNTAX, PATTERNS & PROJECT STRUCTURE

| EMBEDDED C DATA TYPES & QUALIFIERS |  |  |  |
| --- | --- | --- | --- |
| stdint.h types | uint8_t, int16_t, uint32_t, int64_t | stdbool.h | bool, true, false |
| volatile | Prevents optimiser removing read | const | Read-only; stored in Flash if global |
| static (local) | Persists between function calls | static (global) | File-scope only (private to .c file) |
| register | Hint: use CPU register (mostly ignored) | restrict | Pointer aliasing hint for optimiser |
| __attribute__ | GCC: aligned, packed, section, weak | inline | Request function inlining |
| STRUCTS, BITFIELDS & UNIONS |  |  |  |
| Packed struct | typedef struct __attribute__((packed)) { uint8_t id; uint16_t val; uint8_t crc; } Packet_t; |  |  |
| Bitfield (reg) | typedef struct { uint8_t EN:1; uint8_t MODE:2; uint8_t ERR:1; uint8_t :4; } RegCtrl_t; |  |  |
| Union overlay | typedef union { uint32_t word; uint8_t bytes[4]; float f; } Word32_t; |  |  |
| REGISTER-LEVEL GPIO (ARM/STM32) |  |  |  |
| Set pin HIGH | GPIOA->BSRR = (1UL << 5); // Set PA5 |  |  |
| Set pin LOW | GPIOA->BSRR = (1UL << (5 + 16)); // Reset PA5 |  |  |
| Toggle pin | GPIOA->ODR ^= (1UL << 5); // Toggle PA5 |  |  |
| Read pin | if (GPIOA->IDR & (1UL << 0)) { } // Read PA0 |  |  |
| Config output | GPIOA->MODER ¦= (1UL << (5*2)); // Mode=01 output |  |  |
| INTERRUPT & ISR PATTERNS |  |  |  |
| ISR prototype | void EXTI0_IRQHandler(void) { EXTI->PR1 ¦= (1<<0); flag=1; } |  |  |
| Shared flag | volatile uint8_t flag = 0; // in main: while(!flag){}; flag=0; |  |  |
| Atomic read | __disable_irq(); val = shared; __enable_irq(); // critical section |  |  |
| NVIC enable | HAL_NVIC_SetPriority(EXTI0_IRQn, 0, 0); HAL_NVIC_EnableIRQ(EXTI0_IRQn); |  |  |
| STATE MACHINE PATTERN (Embedded) |  |  |  |
| Enum states | typedef enum { ST_IDLE, ST_INIT, ST_RUN, ST_ERROR, ST_COUNT } State_t; |  |  |
| SM loop | State_t state = ST_IDLE; while(1){ switch(state){ case ST_IDLE: ... break; } } |  |  |
| USEFUL MACROS |  |  |  |
| BIT macro | #define BIT(n) (1UL << (n)) |  |  |
| SET_BIT | #define SET_BIT(r,b) ((r) ¦= BIT(b)) |  |  |
| CLR_BIT | #define CLR_BIT(r,b) ((r) &= ~BIT(b)) |  |  |
| TST_BIT | #define TST_BIT(r,b) (((r) >> (b)) & 1U) |  |  |
| ARRAY_SIZE | #define ARRAY_SIZE(a) (sizeof(a)/sizeof((a)[0])) |  |  |
| CLAMP | #define CLAMP(v,lo,hi) ((v)<(lo)?(lo):((v)>(hi)?(hi):(v))) |  |  |
| MIN/MAX | #define MIN(a,b) ((a)<(b)?(a):(b)) ¦ #define MAX(a,b) ((a)>(b)?(a):(b)) |  |  |
| UNUSED param | #define UNUSED(x) (void)(x) // suppress compiler warning |  |  |
| NOTE: Never use int/long for hardware registers — sizes vary by platform. Always use stdint.h fixed-width types. |  |  |  |

EMBEDDED C — PROFESSIONAL PROJECT STRUCTURE

| EMBEDDED C PROJECT STRUCTURE — STM32 / Bare-metal / FreeRTOS |  |
| --- | --- |
| MyProject/ | Root project folder |
| ├─ Core/ | STM32CubeMX-generated core files |
| │ ├─ Inc/ | main.h, gpio.h, usart.h, tim.h, stm32h7xx_hal_conf.h |
| │ ├─ Src/ | main.c, gpio.c, usart.c, tim.c, stm32h7xx_it.c (ISRs) |
| │ └─ Startup/ | startup_stm32h755xx.s — reset handler, vector table |
| ├─ Drivers/ | HAL + BSP drivers (do not edit) |
| │ ├─ STM32H7xx_HAL_Driver/ | Cube HAL .c/.h (generated, version-controlled separately) |
| │ └─ CMSIS/ | ARM CMSIS core headers (core_cm7.h etc.) |
| ├─ Middleware/ | Third-party middleware |
| │ ├─ FreeRTOS/ | tasks.c, queue.c, timers.c, heap_4.c, FreeRTOSConfig.h |
| │ ├─ LwIP/ | TCP/IP stack (optional — Ethernet apps) |
| │ └─ USB_Device/ | USB CDC/MSC/HID class stack |
| ├─ App/ | YOUR application code (clean separation from HAL) |
| │ ├─ Inc/ | app_config.h, sensor.h, comms.h, state_machine.h |
| │ ├─ Src/ | app_main.c, sensor.c, comms.c, state_machine.c |
| │ ├─ Tasks/ | FreeRTOS tasks: task_sensor.c, task_comms.c, task_ui.c |
| │ └─ Util/ | ring_buffer.c, crc.c, logger.c, assert.c |
| ├─ CM4/ | Cortex-M4 sub-project (dual-core STM32H7) |
| │ ├─ Core/ | main_cm4.c, ipc.c, hsem.c, cm4_tasks.c |
| │ └─ Startup/ | startup_stm32h755xx_CM4.s |
| ├─ Tests/ | Unit tests (Unity / CMock framework) |
| │ ├─ test_sensor.c | Mocked HAL, test sensor driver in isolation |
| │ └─ test_crc.c | Pure C — no hardware dependency |
| ├─ Scripts/ | Build & deployment scripts |
| │ ├─ flash.sh | OpenOCD / STM32CubeProg flash script |
| │ ├─ gdb_init.gdb | GDB startup: break main, load, run |
| │ └─ generate_version.py | Embeds git hash + date into firmware binary |
| ├─ Docs/ | Schematics PDF, memory map, protocol specs |
| ├─ .github/ | CI/CD: build.yml (gcc-arm, unit tests, size check) |
| ├─ CMakeLists.txt | CMake build (preferred over Makefile for larger projects) |
| ├─ MyProject.ioc | STM32CubeMX config (do not manually edit) |
| ├─ STM32H755ZITX_FLASH.ld | Linker script — MEMORY regions, SECTIONS |
| └─ .gitignore | Ignore: build/, *.elf, *.o, *.bin, *.map |

| LINKER SCRIPT — MEMORY REGIONS & SECTIONS |  |
| --- | --- |
| MEMORY block | MEMORY { FLASH(rx):ORIGIN=0x08000000,LENGTH=1024K DTCMRAM(rwx):ORIGIN=0x20000000,LENGTH=128K RAM(rwx):ORIGIN=0x24000000,LENGTH=512K } |
| .text section | .text : { *(.isr_vector) *(.text*) *(.rodata*) } > FLASH |
| .data section | .data : { *(.data*) } > DTCMRAM AT>FLASH /* LMA in Flash, VMA in RAM */ |
| .bss section | .bss : { *(.bss*) *(COMMON) } > DTCMRAM /* Zero-init by startup */ |
| DMA buffer | .dma_buf (NOLOAD): { *(.dma_buffer) } > RAM /* non-cached SRAM for DMA */ |
| ITCM section | .itcmram : { *(.itcmram*) } > ITCMRAM AT>FLASH /* Copy ISR to fast TCM */ |
| C ATTRIBUTE PLACEMENT |  |
| Place in ITCM | __attribute__((section(".itcmram"))) void USART1_IRQHandler(void) { ... } |
| DMA buffer | __attribute__((section(".dma_buffer"), aligned(32))) uint8_t rxBuf[256]; |
| No init RAM | __attribute__((section(".noinit"))) uint32_t resetCause; /* survives reset */ |
| FREERTOS TASK TEMPLATE |  |
| Task create | xTaskCreate(vSensorTask, "Sensor", 512, NULL, 3, &hSensor); // 512 words stack |
| Task body | void vSensorTask(void *arg){ for(;;){ xSemaphoreTake(sem,pdMS_TO_TICKS(100)); /* work */ } } |
| Queue send | xQueueSend(xDataQ, &packet, pdMS_TO_TICKS(10)); // from ISR: xQueueSendFromISR() |
| Delay | vTaskDelayUntil(&xLast, pdMS_TO_TICKS(10)); // precise 10ms period (not vTaskDelay) |
| NOTE: Use vTaskDelayUntil() not vTaskDelay() for periodic tasks — eliminates drift. Stack in WORDS (4 bytes each on ARM). |  |

# 22. INDUSTRIAL MEASUREMENT STRUCTS — Embedded C

Digital I/O Payload

// Digital I/O data payload structure

typedef struct {

uint32_t input_states; // 32-bit input state mask (bit0=DI0 ... bit31=DI31)

uint32_t output_states; // 32-bit output state mask (bit0=DO0 ... bit31=DO31)

uint32_t input_change_mask; // bits that changed since last poll

uint32_t output_change_mask; // output bits changed by last command

uint8_t di_count; // number of physical DI channels

uint8_t do_count; // number of physical DO channels

uint8_t reserved[2]; // pad to 4-byte boundary

} digital_io_payload_t;

System Health Payload

// System health payload structure

typedef struct {

uint8_t health_status; // Overall health percentage (0-100)

uint8_t cpu_usage; // CPU usage percentage (0-100)

uint8_t memory_usage; // Memory usage percentage (0-100)

uint8_t temperature; // Internal temperature in °C

uint16_t voltage_mv; // Supply voltage in millivolts

uint16_t error_count; // Number of errors since last reset

uint32_t uptime_s; // Seconds since last power-on

uint32_t last_error_code; // Most recent error/fault code

} system_health_payload_t;

AC Voltage & Current Measurement Payload

// Single-phase AC measurement payload

typedef struct {

uint32_t timestamp_ms; // Measurement timestamp (ms since boot)

int32_t voltage_mv; // RMS voltage in millivolts

int32_t current_ma; // RMS current in milliamps

int32_t active_power_mw; // Active power P in milliwatts

int32_t reactive_power_mvar;// Reactive power Q in milli-VAR

uint32_t apparent_power_mva; // Apparent power S in milli-VA

int16_t power_factor_x1000; // PF × 1000 e.g. 0.95 → 950

int16_t phase_angle_cdeg; // Phase angle in centidegrees e.g. 18.5° → 1850

uint16_t frequency_mhz; // Frequency × 1000 e.g. 50.000Hz → 50000

uint16_t thd_pct_x100; // THD% × 100 e.g. 5.2% → 520

uint8_t channel; // 0=L1 1=L2 2=L3 3=Neutral

uint8_t flags; // bit0=OV bit1=UV bit2=OC bit3=PF_low

uint16_t crc16; // CRC-16/Modbus over all preceding bytes

} ac_measurement_t; // 32 bytes packed

Three-Phase Power Payload

// Three-phase power summary payload

typedef struct {

ac_measurement_t phase[3]; // L1 [0], L2 [1], L3 [2] per-phase data

int32_t total_active_w; // P_total = P_L1 + P_L2 + P_L3 (watts)

uint32_t total_apparent_va; // S_total = sqrt3 × V_L × I_L (VA)

int32_t total_reactive_var; // Q_total = sqrt(S² - P²) (VAR)

int16_t voltage_unbal_x100; // Voltage unbalance factor × 100 (%)

int16_t current_unbal_x100; // Current unbalance factor × 100 (%)

uint32_t meter_serial; // Unique meter serial number

uint8_t tariff_zone; // 0=off-peak 1=standard 2=peak

uint8_t reserved[3]; // alignment padding

} three_phase_payload_t;

Modbus RTU Frame & Float Union

// Modbus float union — dual view: two 16-bit registers OR one IEEE-754 float

typedef union {

uint16_t reg[2]; // reg[0]=high word, reg[1]=low word (big-endian)

uint32_t raw32; // combined 32-bit raw value

float value; // IEEE-754 float (verify byte order with meter!)

} modbus_float_t;

// Modbus RTU request frame

typedef struct __attribute__((packed)) {

uint8_t slave_addr; // 1–247

uint8_t function_code; // 0x03=read regs 0x06=write reg 0x10=write multi

uint16_t start_register; // first register address (big-endian)

uint16_t register_count; // number of registers to read/write

uint16_t crc16; // CRC-16/Modbus (little-endian append)

} modbus_request_t; // 8 bytes

Meter Status / Alarm Bitfield

// Meter alarm and status flags — maps directly to Modbus coil register

typedef struct {

uint16_t overvoltage : 1; // bit 0 — phase voltage > OV threshold

uint16_t undervoltage : 1; // bit 1 — phase voltage < UV threshold

uint16_t overcurrent : 1; // bit 2 — phase current > OC threshold

uint16_t pf_low : 1; // bit 3 — power factor < PF_min setting

uint16_t freq_high : 1; // bit 4 — frequency > 50.5 Hz

uint16_t freq_low : 1; // bit 5 — frequency < 49.5 Hz

uint16_t phase_loss : 1; // bit 6 — one or more phases missing

uint16_t earth_fault : 1; // bit 7 — earth leakage detected

uint16_t ct_open : 1; // bit 8 — CT secondary open circuit

uint16_t reverse_energy : 1; // bit 9 — energy flow reversed (export)

uint16_t tamper : 1; // bit 10 — tamper event detected

uint16_t comms_timeout : 1; // bit 11 — master poll timeout

uint16_t reserved : 4; // bits 12–15 unused

} meter_status_t;

Industrial Sensor Packet Union (UART / CAN Transmission)

// Industrial sensor packet — dual view: structured fields OR raw byte array

typedef union {

struct __attribute__((packed)) {

uint8_t start_byte; // 0xAA — frame sync

uint8_t device_id; // 1–247 (Modbus slave address)

uint8_t msg_type; // 0x01=meas 0x02=alarm 0x03=config

uint8_t length; // payload byte count (excl. header+crc)

ac_measurement_t payload; // 32-byte measurement body

uint16_t crc; // CRC-16 over [device_id .. payload]

};

uint8_t raw[38]; // entire frame as byte array for UART TX

} industrial_frame_t;

// Usage: uart_write(frame.raw, sizeof(industrial_frame_t));

Calibration Data (stored in EEPROM / Flash)

// Calibration coefficients — stored in Flash/EEPROM with CRC integrity check

typedef struct __attribute__((packed, aligned(4))) {

uint32_t magic; // 0xCAFEBABE — marks valid calibration block

float v_gain[3]; // voltage gain correction per phase [L1,L2,L3]

float v_offset_mv[3]; // voltage offset in mV per phase

float i_gain[3]; // current gain correction per CT channel

float i_offset_ma[3]; // current offset in mA per CT channel

float phase_comp_deg[3]; // phase compensation angle in degrees

uint32_t cal_timestamp; // UNIX epoch of last calibration

uint16_t hw_version; // PCB revision e.g. 0x0102 = v1.2

uint16_t fw_version; // firmware version

uint32_t crc32; // CRC-32 integrity check over all above bytes

} calibration_data_t; // 80 bytes total

// Read rule: validate magic == 0xCAFEBABE AND crc32 before using any field

| STRUCT / UNION QUICK REFERENCE |  |  |  |
| --- | --- | --- | --- |
| __attribute__((packed)) | Remove padding — exact byte layout | __attribute__((aligned(n))) | Force n-byte alignment (DMA need 32) |
| volatile member | uint32_t volatile reg — prevents optim. | const pointer | const sensor_t *p — read-only access |
| Padding rule | Member aligns to its own size by default | Zero padding | uint8_t reserved[N] — explicit pad |
| Union size | = size of largest member | Struct size | = sum of members + padding |
| memcpy rule | Always use memcpy() for packed structs | Never deref | Packed ptr deref = bus fault on M4/M7 |
| Endian BE→LE swap 16 | ((v>>8)&0xFF)¦((v&0xFF)<<8) | Endian BE→LE swap 32 | __builtin_bswap32(v) or ntohl(v) |
| Magic number | 0xCAFEBABE / 0xDEADBEEF — validity mark | CRC choice | CRC-16/Modbus for frames; CRC-32 for NVM |
| NOTE: Scale to integers in structs — never store raw floats in protocol frames (endian/NaN issues). Use float only in local processing or calibration NVM with CRC protection. |  |  |  |

# 23. AC VOLTAGE, POWER & POWER FACTOR

| AC WAVEFORM FUNDAMENTALS |  |  |  |
| --- | --- | --- | --- |
| Instantaneous v(t) | v(t) = Vm × sin(ωt + φ) | Instantaneous i(t) | i(t) = Im × sin(ωt + φ - δ) |
| Angular frequency ω | ω = 2πf [rad/s] | Period T | T = 1/f (SA: 1/50 = 20 ms) |
| Peak voltage Vm | Vm = Vrms × √2 | Peak current Im | Im = Irms × √2 |
| RMS voltage | Vrms = Vm / √2 (sine only) | RMS general | Vrms = √(1/T × ∫v² dt) |
| Vrms (non-sine) | Vrms = √(V1²+V2²+V3²+...) (harmonics) | Form factor | FF = Vrms / Vavg = 1.1107 (sine) |
| Crest factor | CF = Vpeak / Vrms = √2 ≈ 1.414 (sine) | SA nominal voltage | 230 V RMS phase, 400 V L-L (NRS 048) |
| POWER TRIANGLE — P, Q, S |  |  |  |
| Active power P | P = Vrms × Irms × cosφ [W] | Resistive load | P = I²R = V²/R |
| Reactive power Q | Q = Vrms × Irms × sinφ [VAR] | Inductive Q | Q = I² × XL (positive, lagging) |
| Apparent power S | S = Vrms × Irms [VA] | Power triangle | S² = P² + Q² |
| Power factor PF | PF = cosφ = P / S | Phase angle φ | φ = arctan(Q / P) |
| Lagging PF | Current lags voltage — inductive load | Leading PF | Current leads voltage — capacitive |
| Unity PF | cosφ = 1 (pure resistive, P = S) | Zero PF | cosφ = 0 (pure reactive, P = 0) |
| THREE-PHASE POWER |  |  |  |
| 3φ Active P | P = √3 × VL × IL × cosφ [W] | Also expressed as | P = 3 × Vph × Iph × cosφ |
| 3φ Reactive Q | Q = √3 × VL × IL × sinφ [VAR] | 3φ Apparent S | S = √3 × VL × IL [VA] |
| Star VL–Vph | VL = √3 × Vph (SA: 400V = √3 × 230V) | Star IL = Iph | Line current = phase current |
| Delta VL = Vph | Line voltage = phase voltage | Delta IL = √3 × Iph | IL = √3 × phase winding current |
| Symmetrical load | All 3 phases equal — neutral current = 0 | Unbalanced load | Neutral carries difference current |
| POWER FACTOR — MEASUREMENT & CORRECTION |  |  |  |
| Displacement PF | DPF = cos(φ₁) — fundamental component only | True PF | TPF = P / S (includes harmonics) |
| True PF with THD | PF = DPF / √(1 + THD_I²) | THD effect | High THD_I reduces true PF even at φ=0 |
| PF from P & Q | PF = P / √(P² + Q²) | PF from energy meter | PF = Wh / VAh (over same interval) |
| Leading / Lagging sign | IEEE: +Q = inductive (lagging) | IEC convention | IEC: +Q = inductive (same as IEEE) |
| Vars required for corr. | Qc = P × (tanφ1 - tanφ2) | Capacitor size | C = Qc / (ω × V²) [F] |
| PF correction target | Eskom industrial target: PF ≥ 0.90 | Penalty zone | PF < 0.90 → Eskom reactive energy charge |
| POWER FACTOR CORRECTION (PFC) |  |  |  |
| Passive PFC | Capacitor bank switched in steps | C bank step size | Qstep = C × ω × V² (per capacitor) |
| Active PFC | Boost converter forces sinusoidal input I | Active PFC THD | < 5% input THD, PF > 0.99 |
| Auto PF controller | Measures KVAR, switches C banks via relay | Hunting prevention | Deadband ± 0.02 PF around setpoint |
| Harmonic filter | Passive: tuned LC ¦ Active: inject anti-harmonic current | Resonance risk | Avoid C + transformer resonance |
| REACTIVE ENERGY & BILLING (SA Eskom NRS 048 / NRS 057) |  |  |  |
| Active energy | kWh = P[kW] × hours (billed) | Reactive energy | kVARh = Q[kVAR] × hours (penalised) |
| Apparent energy | kVAh = S[kVA] × hours | Demand | kVA demand = avg S over 30-min window |
| MD (Max Demand) | Highest 30-min kVA average in billing month | MD charge | R/kVA/month on peak demand |
| NMD (Notified MD) | Agreed max demand with utility | Excess charge | Penalty for exceeding NMD |
| ToU (Time of Use) | Peak 07:00-10:00 & 18:00-20:00 weekdays | Off-peak | 22:00-06:00 weekdays, full weekend |
| VOLTAGE & CURRENT QUALITY INDICATORS |  |  |  |
| Voltage regulation VR | VR% = (VNL - VFL) / VFL × 100% | Good VR target | < 5% for feeders (NRS 048-2) |
| Voltage unbalance VUF | VUF% = Vneg / Vpos × 100% | Limit | < 2% sustained (IEC 61000-2-2) |
| Voltage sag | RMS drop 10–90%, duration 10ms–1min | Swell | RMS rise > 110%, < 1 min |
| Flicker Pst | Short-term perceptibility index (10-min) | Plt | Long-term (2-hour rolling cube mean) |
| Current unbalance | I_unbal% = Imax_dev / Iavg × 100% | Motor derating | 1% V unbalance → ~6–10% I unbalance |
| NOTE: In SA: supply is 230V/400V 50Hz (NRS 048). Industrial PF penalty applies below 0.90. Use kVAR meters or smart meters with reactive energy registers. Always check both true PF and displacement PF on non-linear loads (VFDs, UPS, SMPS). |  |  |  |

# 24. BATTERY SOC — STATE OF CHARGE MEASUREMENTS

| SOC DEFINITION & METHODS |  |  |  |
| --- | --- | --- | --- |
| SOC definition | SOC = Q_remaining / Q_rated × 100 (%) | 100% SOC | Fully charged cell |
| 0% SOC | Fully discharged (Vmin cutoff reached) | Capacity Qn | Rated Ah e.g. 100 Ah @ C10 rate |
| C-rate | C/n = full discharge in n hours | 1C example (100Ah) | 100A discharge → empty in 1 hour |
| METHOD 1 — COULOMB COUNTING (Current Integration) |  |  |  |
| Continuous form | SOC(t) = SOC₀ + (η/Qn) × ∫₀ᵗ I(τ) dτ | Sign convention | +I = charge, –I = discharge |
| Discrete (MCU) | SOC += (I × Δt × η) / (Qn × 3600) × 100 | Units | I[A], Δt[s], Qn[Ah] → SOC[%] |
| Charge efficiency η | Li-ion: η ≈ 0.99 ¦ Lead-acid: η ≈ 0.85–0.90 | Self-discharge | Subtract leakage current from I |
| Error accumulation | Drifts over time — needs periodic reset | Reset trigger | At full charge detection (Vmax + Itaper) |
| ADC requirement | 12–16 bit ADC on shunt resistor | Shunt example | 10mΩ shunt: 100A → 1.0V drop |
| METHOD 2 — OCV LOOKUP (Open Circuit Voltage) |  |  |  |
| OCV-SOC relation | SOC = f(OCV) — lookup table per chemistry | Rest time needed | Li-ion: ≥ 2h rest; Lead-acid: ≥ 4h |
| Li-ion NMC OCV | 100%: 4.20V ¦ 50%: 3.70V ¦ 0%: 3.00V | LFP OCV (flat) | 100%: 3.60V ¦ 50%: 3.30V ¦ 0%: 2.80V |
| Lead-acid OCV | 100%: 12.70V ¦ 50%: 12.20V ¦ 0%: 11.80V | NiMH OCV | 100%: 1.45V ¦ 50%: 1.25V ¦ 0%: 1.10V |
| OCV error under load | IR drop distorts reading — not usable live | Temperature effect | OCV shifts ≈ –2mV/°C (lead-acid) |
| METHOD 3 — KALMAN FILTER (Optimal Estimator) |  |  |  |
| State vector | x = [SOC, Vrc1, Vrc2]ᵀ (+ RC model states) | Measurement | z = Vterminal (measured voltage) |
| Process model | x(k+1) = A×x(k) + B×u(k) + w(k) | Measurement model | z(k) = H×x(k) + v(k) |
| EKF linearise | Jacobian of h(x) at each time step | Advantage | Handles non-linear OCV-SOC curve |
| Typical accuracy | ±1–3% with good cell model + temperature | Compute cost | Feasible on Cortex-M4 @ 1Hz update |
| BATTERY CHEMISTRY — OCV & VOLTAGE RANGES |  |  |  |
| Li-ion (NMC) | Nom 3.6V ¦ Full 4.20V ¦ Empty 3.00V/cell | Li-ion (LFP) | Nom 3.2V ¦ Full 3.60V ¦ Empty 2.80V |
| LiPo | Nom 3.7V ¦ Full 4.20V ¦ Storage 3.85V | Lead-acid (12V) | Full 12.7V ¦ 50% 12.2V ¦ Min 11.8V |
| Series string Vtotal | Vtotal = n × Vcell | Parallel: same V | Itotal = n × Icell |
| SOH & BMS THRESHOLDS |  |  |  |
| SOH (capacity) | SOH = Q_measured / Q_rated × 100% | EOL threshold | SOH < 80% (IEC 62619 / ISO 6469) |
| Cell imbalance trigger | ΔVcell > 50 mV → start balancing | Passive balance | Bleed resistor on high cell (heat) |
| Active balancing | Energy transfer cell-to-cell (inductor/cap) | OVP cutoff | Li-ion: > Vmax per cell (4.25V NMC) |
| UVP cutoff | Li-ion: < 3.00V ¦ LFP: < 2.80V per cell | OTP cutoff | Charge cutoff > 45°C ¦ Disch > 60°C |
| SOC accuracy target | ±3% automotive (ISO 26262) | Stationary systems | ±5% acceptable (NRS 097 / IEC 62619) |
| NOTE: Always combine coulomb counting + OCV reset for best accuracy. In SA solar/UPS: recalibrate SOC at each full-charge cycle. Never charge Li-ion below 0°C — lithium plating causes permanent damage. |  |  |  |

# 25. CORS — CROSS-ORIGIN RESOURCE SHARING

| CORS FUNDAMENTALS (RFC 6454 + Fetch Standard) |  |  |  |
| --- | --- | --- | --- |
| Same-origin policy | Browser blocks cross-origin requests by default | Origin = | scheme + host + port (all 3 must match) |
| Cross-origin trigger | Origin differs in ANY of scheme / host / port | CORS purpose | Server opts-in to allow specific origins |
| Who enforces CORS | Browser only — curl/Postman ignore it | Not a firewall | Server still receives the request always |
| REQUEST TYPES |  |  |  |
| Simple request | GET/POST + no custom headers + safe content-type | No preflight | Browser sends request directly with Origin |
| Preflight request | Browser sends OPTIONS before actual request | Triggers when | PUT/DELETE/PATCH or custom headers used |
| Credentialed request | Includes cookies: fetch({credentials:'include'}) | Server must | Return specific origin (no *) + Allow-Credentials: true |
| Preflight caching | Access-Control-Max-Age: 3600 (seconds) | Chrome max | 7200s ¦ Firefox max: 86400s |
| REQUEST HEADERS (Browser → Server) |  |  |  |
| Origin | Origin: https://app.example.com | Sent by | Browser — automatically on every cross-origin req. |
| Request-Method | Access-Control-Request-Method: PUT | In | OPTIONS preflight only |
| Request-Headers | Access-Control-Request-Headers: X-Auth-Token | In | OPTIONS preflight — lists custom headers |
| RESPONSE HEADERS (Server → Browser) |  |  |  |
| Allow-Origin * | Access-Control-Allow-Origin: * | Restriction | Wildcard blocks credentials (cookies/auth) |
| Allow-Origin specific | Access-Control-Allow-Origin: https://app.com | Required when | credentials=true OR restricted access needed |
| Allow-Methods | Access-Control-Allow-Methods: GET, POST, PUT, DELETE, PATCH, OPTIONS | List | All methods the client may call |
| Allow-Headers | Access-Control-Allow-Headers: Content-Type, Authorization, X-Custom | List | Every custom header client will send |
| Allow-Credentials | Access-Control-Allow-Credentials: true | Critical | Cannot combine with Allow-Origin: * |
| Expose-Headers | Access-Control-Expose-Headers: X-RateLimit-Remaining | Purpose | Makes non-simple response headers readable in JS |
| Max-Age | Access-Control-Max-Age: 3600 | Effect | Browser caches preflight — reduces OPTIONS calls |
| PREFLIGHT FLOW (4 STEPS) |  |  |  |
| 1 Browser→Server | OPTIONS /api/sensor ¦ Origin: https://dashboard.com ¦ Access-Control-Request-Method: PUT ¦ Access-Control-Request-Headers: Authorization |  |  |
| 2 Server→Browser | HTTP 200 OK ¦ Access-Control-Allow-Origin: https://dashboard.com ¦ Access-Control-Allow-Methods: GET,PUT ¦ Access-Control-Allow-Headers: Authorization ¦ Access-Control-Max-Age: 3600 |  |  |
| 3 Browser→Server | PUT /api/sensor (actual request — proceeds only if Step 2 headers are valid) ¦ Origin: https://dashboard.com ¦ Authorization: Bearer <token> |  |  |
| 4 Server→Browser | HTTP 200 OK ¦ Access-Control-Allow-Origin: https://dashboard.com ¦ Content-Type: application/json ¦ { response body } |  |  |
| SERVER CONFIGURATION EXAMPLES |  |  |  |
| Node / Express | app.use(cors({ origin:'https://app.com', methods:['GET','POST','PUT','DELETE'], allowedHeaders:['Content-Type','Authorization'], credentials:true, maxAge:3600 })); |  |  |
| Python FastAPI | app.add_middleware(CORSMiddleware, allow_origins=['https://app.com'], allow_credentials=True, allow_methods=['*'], allow_headers=['*'], max_age=3600) |  |  |
| Nginx config | add_header 'Access-Control-Allow-Origin' 'https://app.com' always; add_header 'Access-Control-Allow-Methods' 'GET,POST,PUT,DELETE,OPTIONS' always; add_header 'Access-Control-Max-Age' '3600' always; |  |  |
| ASP.NET Core | builder.Services.AddCors(o=>o.AddPolicy("Policy", b=>b.WithOrigins("https://app.com").AllowAnyMethod().AllowAnyHeader().AllowCredentials())); |  |  |
| ESP32 / embedded | httpd_resp_set_hdr(req, "Access-Control-Allow-Origin", "*"); // wildcard OK for local IoT with no cookies |  |  |
| COMMON CORS ERRORS & FIXES |  |  |  |
| No Allow-Origin header | Server missing CORS headers entirely | Fix | Add CORS middleware / response headers on server |
| Wildcard + credentials | Allow-Origin:* with credentials:include | Fix | Replace * with exact origin string |
| Preflight 404 / 405 | Server has no OPTIONS route | Fix | Add OPTIONS handler or auto-handle in middleware |
| Header not exposed | Custom response header unreadable in JS | Fix | Add to Access-Control-Expose-Headers |
| Mixed content | HTTPS page calling HTTP API | Fix | Both must use HTTPS — no exceptions in browsers |
| Cookie not sent | credentials:'include' missing in fetch() | Fix | Set credentials option + ensure SameSite=None;Secure |
| CORS IN EMBEDDED / IoT CONTEXT |  |  |  |
| IoT HTTP server | ESP32/STM32-LwIP — wildcard OK without cookies | Reverse proxy | Nginx handles CORS — MCU never sees preflight |
| WebSocket CORS | WS upgrade is NOT subject to CORS headers | WS auth | Use token in URL param or first message payload |
| MQTT over WS | MQTT-WS follows WebSocket rules (not HTTP CORS) | Server-to-server | HTTP calls between servers bypass CORS entirely |
| NOTE: CORS is a browser security feature only — it is NOT an authentication mechanism. Always add JWT/OAuth2 auth in addition to CORS. Misconfigured CORS (overly permissive) can expose APIs to CSRF attacks. |  |  |  |

# 26. MQTT — MESSAGE QUEUING TELEMETRY TRANSPORT

| MQTT FUNDAMENTALS (ISO/IEC 20922) |  |  |  |
| --- | --- | --- | --- |
| Protocol | Pub/Sub over TCP/IP (not request/response) | Transport | TCP port 1883 (plain) ¦ 8883 (TLS) |
| Broker | Central message router (Mosquitto, HiveMQ, EMQX) | Client | Publisher OR Subscriber (or both) |
| Payload | Any format: JSON, binary, plain text (up to 256MB) | MQTT 3.1.1 | Most widely deployed version |
| MQTT 5.0 | Adds: reason codes, message expiry, shared subs, topic aliases | Overhead | Fixed header: 2-5 bytes only — IoT optimised |
| TOPICS & WILDCARDS |  |  |  |
| Topic structure | Hierarchical: factory/line1/machine3/temp | Case sensitive | Temperature ≠ temperature |
| Single-level + | factory/+/machine3 matches factory/line1/machine3 | Multi-level # | factory/# matches everything under factory/ |
| $ topics | $SYS/# — broker system info (clients, messages, load) | Retained msg | Broker stores last message per topic — new subs get it immediately |
| Shared subs MQTT5 | $share/group/topic — load-balance across subscribers | Topic alias | Short int replaces long topic string (MQTT 5.0) |
| QUALITY OF SERVICE (QoS) |  |  |  |
| QoS 0 — At most once | Fire-and-forget: no ACK, possible loss | Best for | High-freq sensor data, loss tolerable |
| QoS 1 — At least once | PUBACK required: may deliver duplicates | Best for | Commands where duplicates are handled |
| QoS 2 — Exactly once | 4-way handshake: PUBREC→PUBREL→PUBCOMP | Best for | Financial/billing data, critical events |
| QoS overhead | QoS0: 0 bytes ¦ QoS1: +2 bytes PacketID ¦ QoS2: +multi round-trips | Choice | Use QoS 0 or 1 on constrained devices |
| CONNECT, KEEP-ALIVE & LAST WILL |  |  |  |
| CONNECT params | clientID, cleanSession, keepAlive(sec), willTopic, willPayload, willQoS, willRetain, username, password |  |  |
| Keep-alive | Client sends PINGREQ every keepAlive interval ¦ Broker expects within 1.5× keepAlive |  |  |
| Last Will (LWT) | Broker publishes willTopic/willPayload if client disconnects unexpectedly — device offline alert |  |  |
| Clean session 0 | Broker persists subscriptions + QoS1/2 queue across reconnects — MQTT persistent session |  |  |
| Clean session 1 | Fresh start on every connect — no stored subs or queued messages |  |  |
| MQTT PACKET TYPES |  |  |  |
| CONNECT / CONNACK | Client→Broker ¦ Broker→Client (return code 0=ok) | PUBLISH | Either direction: topicLen+topic+[packetID]+payload |
| PUBACK (QoS1) | Broker→Client: message received | PUBREC/REL/COMP | QoS2 four-step handshake |
| SUBSCRIBE/SUBACK | Client requests topic filter + QoS | UNSUBSCRIBE | Remove topic filter subscription |
| PINGREQ/PINGRESP | Keep-alive heartbeat (no payload) | DISCONNECT | Graceful disconnect (LWT not sent) |
| SECURITY |  |  |  |
| TLS (port 8883) | Mutual TLS: client cert + server cert validation | Username/pass | CONNECT: username + hashed password |
| ACL (access control) | Broker config: which client can pub/sub which topic | JWT via MQTT5 | Enhanced auth in CONNECT (MQTT 5.0) |
| Payload encryption | End-to-end: encrypt JSON before publish (AES-GCM) | Client ID | Must be unique — duplicate kicks old connection |
| EMBEDDED MQTT (IoT CLIENT) |  |  |  |
| Paho MQTT C | Eclipse Paho: embedded C client, minimal footprint | MQTT-SN | UDP-based variant for mesh/radio (no TCP needed) |
| ESP32 MQTT | esp-mqtt component: configurable QoS, TLS, auto-reconnect | FreeRTOS task | Dedicated MQTT task: subscribe, publish, keepalive |
| Broker memory | Each retained msg stored in broker RAM/disk | Max clients | Mosquitto: ~1000 concurrent; HiveMQ: 10M+ |
| Embedded budget | Paho embedded: ~8KB Flash, ~2KB RAM for MQTTv3 | Buffer size | Rx/Tx buffer ≥ max message size (256B–4KB typical) |
| TOPIC DESIGN — INDUSTRIAL IoT BEST PRACTICES |  |  |  |
| Telemetry topic | facility/site/device_id/measurement e.g. plant1/line3/motor7/temperature |  |  |
| Command topic | facility/site/device_id/cmd/action e.g. plant1/line3/motor7/cmd/setpoint |  |  |
| Status topic | facility/site/device_id/status e.g. plant1/line3/motor7/status (retained=true) |  |  |
| LWT topic | facility/site/device_id/availability payload: 'online'/'offline' (retained=true) |  |  |
| NOTE: Avoid deep topic trees (>5 levels). Use retained messages for device status and last-known values. For SA industrial: Mosquitto + TLS on local edge server reduces cloud dependency and latency. |  |  |  |

# 20b. EMBEDDED C — VARIABLE STORAGE PLACEMENT

| STORAGE CLASS & MEMORY SECTION PLACEMENT IN EMBEDDED C |  |  |  |
| --- | --- | --- | --- |
| DEFAULT PLACEMENT RULES (GCC ARM) |  |  |  |
| Initialised global | int x = 5; → .data section (copied Flash→RAM at startup) | Uninit global | int x; → .bss (zero-initialised in RAM) |
| const global | const int k = 9; → .rodata in Flash (stays in Flash) | Local var | int y; → stack (auto, no section) |
| static local | static int cnt; → .bss or .data (RAM, persists) | register | Compiler hint — usually in CPU register |
| EXPLICITLY PLACE IN INTERNAL FLASH (Read-only data) |  |  |  |
| Constant table | const uint16_t sine_table[256] __attribute__((section(".rodata"))) = { 0, 201, ... }; |  |  |
| String in Flash | const char fw_version[] __attribute__((section(".rodata"))) = "v1.2.3-2024"; |  |  |
| Lookup in Flash | static const float cal_coeff[8] = {1.02f, 0.98f ...}; // const → always Flash |  |  |
| PROGMEM (AVR) | const PROGMEM uint8_t table[] = {0x00, 0xFF}; // AVR Harvard: data in Flash (pgmspace.h) |  |  |
| AVR read back | uint8_t val = pgm_read_byte(&table[i]); // must use pgm_read_* — not direct deref |  |  |
| PLACE IN SPECIFIC SRAM REGION (Section attribute) |  |  |  |
| DTCM (M7 fast) | __attribute__((section(".dtcm_data"))) uint32_t isr_buffer[64]; // 0-wait DTCM RAM |  |  |
| No init RAM | __attribute__((section(".noinit"))) uint32_t reset_cause; // survives soft reset |  |  |
| Backup SRAM | __attribute__((section(".backup_sram"))) RTC_backup_t rtc_data; // VBAT retained |  |  |
| AXI SRAM | __attribute__((section(".axi_sram"))) uint8_t frame_buf[640*480]; // large buffer |  |  |
| DMA-SAFE NON-CACHED BUFFERS (Cortex-M7) |  |  |  |
| DMA buffer | __attribute__((section(".dma_buffer"), aligned(32))) uint8_t rx_buf[256]; |  |  |
| Cache-off via MPU | Configure MPU region as Normal Non-Cacheable for .dma_buffer section |  |  |
| Or use SRAM4 | // SRAM4 is always non-cacheable on STM32H7 — map DMA buffers here |  |  |
| Cache flush | SCB_CleanDCache_by_Addr((uint32_t*)tx_buf, sizeof(tx_buf)); // before DMA write |  |  |
| Cache invalidate | SCB_InvalidateDCache_by_Addr((uint32_t*)rx_buf, sizeof(rx_buf)); // after DMA read |  |  |
| EXTERNAL MEMORY (QSPI Flash, Parallel SRAM, SDRAM) |  |  |  |
| QSPI Flash XIP | // Map QSPI to 0x90000000 (memory-mapped) then const array lives there automatically |  |  |
| Ext. SRAM section | __attribute__((section(".ext_sram"))) uint8_t large_buf[1024*1024]; // 1MB ext. SRAM |  |  |
| Linker MEMORY | EXTRAM (rwx) : ORIGIN = 0x60000000, LENGTH = 8M // FMC Bank1 SRAM |  |  |
| Ext SDRAM section | __attribute__((section(".sdram"))) uint32_t video_buf[800*480]; // frame buffer in SDRAM |  |  |
| EEPROM emulation | // Do NOT store structs directly — use HAL_FLASH_EE API: EE_WriteVariable(addr, val); |  |  |
| VOLATILE — CRITICAL FOR SHARED VARIABLES |  |  |  |
| ISR shared flag | volatile uint8_t data_ready = 0; // prevents optimizer removing ISR-written reads |  |  |
| HW register ptr | volatile uint32_t * const GPIOA_ODR = (uint32_t*)0x48000014; // MMIO register |  |  |
| Atomic 32-bit | __disable_irq(); shared_val = new_val; __enable_irq(); // Cortex-M atomic 32-bit |  |  |
| LINKER SCRIPT SUMMARY (Memory Placement) |  |  |  |
| .text | Code + const — stays in FLASH | .rodata | Read-only data — stays in FLASH |
| .data | Init'd globals — copied Flash→RAM at boot | .bss | Uninit'd globals — zeroed in RAM at boot |
| .noinit | Not touched by startup — retains value across reset | .dma_buffer | Non-cached RAM region — safe for DMA |
| .itcmram | Code copied to ITCM — zero-wait-state execution | .dtcm_data | Data in DTCM — fastest M7 data access |
| NOTE: Key rule: const = Flash. volatile = RAM, not optimised. DMA buffers = non-cached RAM, 32-byte aligned (Cortex-M7 cache line). Never put DMA buffers in cached SRAM without explicit cache flush/invalidate. |  |  |  |

MICROCHIP MPLAB X — PROJECT STRUCTURE

| MICROCHIP MPLAB X IDE — PIC / dsPIC / PIC32 / SAM Project Structure |  |
| --- | --- |
| MyMPLAB_Project.X/ | MPLAB X project root (contains .X folder) |
| ├─ Makefile | Auto-generated by MPLAB X — do not edit manually |
| ├─ nbproject/ | MPLAB X IDE project metadata (XML) |
| │ ├─ project.xml | Project config: tool, device, compiler settings |
| │ ├─ configurations.xml | Build configurations: debug / release / production |
| │ └─ Makefile-*.mk | Auto-generated makefiles per config |
| ├─ main.c | Application entry point — main() + init calls |
| ├─ Header Files/ | Logical MPLAB folder (maps to physical dirs) |
| │ ├─ mcc_generated_files/ | MCC (MPLAB Code Configurator) generated headers |
| │ └─ app/ | Application headers: sensor.h, comms.h, fsm.h |
| ├─ Source Files/ | Logical MPLAB folder |
| │ ├─ mcc_generated_files/ | MCC peripherals: uart.c, spi.c, tmr1.c, adc.c |
| │ │ ├─ uart1.c / uart1.h | MCC UART driver (do not manually edit) |
| │ │ ├─ tmr1.c / tmr1.h | MCC Timer1 driver |
| │ │ ├─ spi1.c / spi1.h | MCC SPI driver |
| │ │ └─ system.c / system.h | MCC clock + oscillator config |
| │ └─ app/ | YOUR application code |
| │ ├─ sensor.c | Sensor read, scale, filter |
| │ ├─ comms.c | Modbus RTU / MQTT / protocol handler |
| │ ├─ state_machine.c | Application FSM |
| │ └─ util.c | Ring buffer, CRC, logging helpers |
| ├─ Important Files/ | Linker script + config bits |
| │ ├─ p33EP512GP806.gld | GCC linker script for target device |
| │ └─ configuration_bits.c | #pragma config — oscillator, WDT, code protect |
| ├─ Libraries/ | Precompiled libs (.a) or MPLAB Harmony v3 |
| │ ├─ Harmony3/ | MPLAB Harmony v3 framework (PIC32 / SAM devices) |
| │ │ ├─ bsp/ | Board Support Package — pinout, clock init |
| │ │ ├─ config/ | Harmony configurator output (device.h, system) |
| │ │ └─ peripheral/ | Harmony PLIB: uart, spi, i2c, tc, adc, dmac |
| │ └─ thirdparty/ | FreeRTOS / lwIP / TinyUSB (via Harmony package) |
| ├─ Linker Files/ | Custom linker script overrides |
| ├─ debug/ | Build output — ELF, HEX, MAP files |
| │ └─ MyProject.X.production.hex | HEX file for programming |
| └─ .gitignore | Ignore: debug/, *.d, *.p1, build/, dist/ |

| MPLAB X KEY CONCEPTS & #pragma CONFIG |  |  |  |
| --- | --- | --- | --- |
| CONFIGURATION BITS (#pragma config) — PIC16/18/dsPIC |  |  |  |
| PIC18 example | #pragma config FOSC=HSMP, PLLCFG=ON, PRICLKEN=ON, WDTEN=OFF, MCLRE=EXTMCLR, LVP=OFF |  |  |
| dsPIC33 example | #pragma config FNOSC=FRCPLL, IESO=OFF, FWDTEN=OFF, WINDIS=OFF, JTAGEN=OFF |  |  |
| PIC32 example | #pragma config FPLLIDIV=DIV_2, FPLLMUL=MUL_20, FPBDIV=DIV_1, FWDTEN=OFF, POSCMOD=HS |  |  |
| MCC (MPLAB CODE CONFIGURATOR) WORKFLOW |  |  |  |
| MCC purpose | GUI peripheral config → auto-generates C driver code | MCC3 vs MCC5 | MCC5 (Melody) is newer — separate plugin |
| Regenerate | After MCC changes: Production > Generate Code | Custom code | Add user code only in USER CODE BEGIN/END blocks |
| MCC Melody | Component-based: drag-and-drop peripherals to canvas | Config file | MCC saves to .mc3 or project config JSON |
| PROGRAMMER / DEBUGGER TOOLS |  |  |  |
| PICkit 4 | USB programmer/debugger, 1.8-5V, ICSP/JTAG/SWD | SNAP | Low-cost debugger, 3.3V only, ICSP/JTAG |
| ICD 4 | High-speed production debugger, 1.2-5.5V, all interfaces | PM4 | Production programmer — gang programming support |
| ICSP pins | MCLR + PGD + PGC (+ VDD + GND = 5 wires) | Pinout | 6-pin RJ-12 or 8-pin MicroStick connector |
| BUILD CONFIGURATIONS |  |  |  |
| Debug build | No optimisation (-O0), debug symbols, fill erased areas | Release | -O2 optimisation, stripped symbols, smaller HEX |
| Production | Code protect fuses enabled, max optimisation | Memory summary | MPLAB shows Program / Data / Config memory usage % |
| NOTE: Never edit mcc_generated_files/ directly — MCC will overwrite on next generation. Place all custom code in app/ folder. Use #pragma config in a dedicated config_bits.c to keep it separate from main.c. |  |  |  |

| BATTERY SOH, INTERNAL RESISTANCE & ADVANCED PARAMETERS |  |  |  |
| --- | --- | --- | --- |
| STATE OF HEALTH (SOH) |  |  |  |
| SOH (capacity) | SOH_C = Q_measured / Q_rated × 100% | Measurement | Full charge → discharge to Vmin → measure Ah |
| SOH (resistance) | SOH_R = Ri_new / Ri_aged × 100% (or inverse) | EOL threshold | SOH < 80% capacity (IEC 62619, ISO 6469) |
| SOH vs cycles | NMC: ~80% SOH after 500 full cycles | LFP longevity | LFP: ~80% SOH after 2000–4000 cycles |
| Calendar ageing | SOH_loss = a × √(time) × exp(Ea/RT) | Arrhenius Ea | Higher temp → exponentially faster ageing |
| Degradation modes | Lithium plating ¦ SEI growth ¦ Active material loss | SEI layer | Solid Electrolyte Interphase forms on anode |
| INTERNAL RESISTANCE (Ri) MEASUREMENT |  |  |  |
| DC Internal Resistance | DCIR = ΔV / ΔI (voltage step / current pulse) | Pulse test | Apply C/2 pulse for 10s, measure ΔV at 1s and 10s |
| DCIR formula | Ri = (VOC - Vload) / I_load [Ω] | Typical Li-ion | Ri: 5–50 mΩ/cell (aged = 2–3× new) |
| AC Impedance (EIS) | Electrochemical Impedance Spectroscopy — frequency sweep | EIS output | Nyquist plot: Ri + Rct + Warburg diffusion |
| Ohmic resistance R0 | Instantaneous voltage drop on current step | Polarisation | Slower RC response: charge transfer + diffusion |
| Temperature effect | Ri doubles for every ~10°C drop below 25°C | Cold start | EV batteries pre-heat to 15°C before fast charge |
| Ri vs SOC | Ri increases at both extremes (SOC < 10% or > 90%) | Ri vs SOH | Ri increases as SOH decreases — aging indicator |
| EQUIVALENT CIRCUIT MODEL (ECM) |  |  |  |
| 1RC Thevenin | Vt = OCV(SOC) - I×R0 - I×R1×(1-e^(-t/τ1)) where τ1 = R1×C1 |  |  |
| 2RC model | Vt = OCV - I×R0 - I×R1(1-e^-t/τ1) - I×R2(1-e^-t/τ2) [higher accuracy] |  |  |
| Parameter ID | Fit R0, R1, C1, R2, C2 from HPPC pulse test data (Least Squares / MATLAB) |  |  |
| STATE OF POWER (SOP) & POWER CAPABILITY |  |  |  |
| SOP definition | Max continuous charge/discharge power at current state | Peak power | Ppeak = (OCV - Vmin)² / (4 × Ri) [W] |
| Charge power limit | P_chg = (Vmax - OCV) × I_chg_max | Discharge limit | P_dch = (OCV - Vmin) × I_dch_max |
| DCFC (fast charge) | Constant current to Vmax then constant voltage to Imin | 1C to 80% SOC | NMC: ~45min ¦ LFP: ~40min at 1C |
| THERMAL MODEL |  |  |  |
| Heat generation Q | Q = I² × Ri + T × ΔS × I (ohmic + entropic) | Thermal runaway | Exothermic reactions above 80°C (NMC) |
| Thermal resistance θ | ΔT = Q_watts × θ [°C/W] | Cell cooling | Liquid cooling: θ ≈ 0.5°C/W ¦ Air: 5–20°C/W |
| Safe temp range | Discharge: -20°C to 60°C ¦ Charge: 0°C to 45°C | Storage temp | 15–25°C optimal for long shelf life |
| BMS PROTECTION PARAMETERS |  |  |  |
| OVP (overvoltage) | NMC cutoff > 4.25V/cell ¦ LFP > 3.65V | UVP (undervolt) | NMC < 2.80V ¦ LFP < 2.50V per cell |
| OCP (overcurrent) | Hardware comparator: < 1µs response | OTP (overtemp) | Charge OTP: 45°C ¦ Discharge OTP: 60°C |
| Balancing trigger | ΔVcell > 30–50mV → activate balancing | Passive bal. | Bleed excess via 50–200mA resistor (heat) |
| Active balancing | Inductor or cap transfers energy between cells | Balance rate | Active: 1–5A ¦ Passive: 50–200mA |
| Pre-charge | Limit inrush to deeply discharged cell < C/10 | Recovery | Apply 0.1C until Vcell > 3.0V (NMC) then normal |
| COULOMBIC EFFICIENCY & ENERGY EFFICIENCY |  |  |  |
| Coulombic eff. CE | CE = Q_out / Q_in × 100% | Li-ion CE | 99.5–99.9% per cycle (near unity at room temp) |
| Energy efficiency | η_E = E_out / E_in = CE × Vdch_avg / Vchg_avg × 100% | Typical | Li-ion: 92–97% round-trip energy efficiency |
| Lead-acid efficiency | CE: 85–90% ¦ Energy: 70–80% | NiMH | CE: 66–92% ¦ Energy: 65–70% |
| NOTE: Measure DCIR monthly in battery management applications — rising Ri is the earliest SOH degradation indicator. Never charge NMC below 0°C. Always log min/max Vcell, max Tcell, max Icell per cycle for warranty tracking. |  |  |  |

QUICK REFERENCE CONSTANTS & UNITS

| PHYSICAL CONSTANTS & UNIT CONVERSIONS |  |  |  |
| --- | --- | --- | --- |
| Boltzmann k | 1.38 × 10⁻²³ J/K | Electron charge q | 1.6 × 10⁻¹⁹ C |
| Permittivity ε₀ | 8.85 × 10⁻¹² F/m | Permeability µ₀ | 4π × 10⁻⁷ H/m |
| Speed of light c | 3 × 10⁸ m/s | Planck h | 6.626 × 10⁻³⁴ J·s |
| 1 kWh | 3.6 × 10⁶ J | 1 Horsepower | 746 W |
| dB Power | 10 × log10(P2/P1) | dB Voltage | 20 × log10(V2/V1) |
| Prefix p (pico) | 10⁻¹² | Prefix n (nano) | 10⁻⁹ |
| Prefix µ (micro) | 10⁻⁶ | Prefix m (milli) | 10⁻³ |
| Prefix k (kilo) | 10³ | Prefix M (Mega) | 10⁶ |
| Prefix G (Giga) | 10⁹ | Prefix T (Tera) | 10¹² |

Comprehensive EE & ECE Formula Cheat Sheet  •  30 Sections  •  Diodes | BJT | MOSFET | SCR | Op-Amp | Filters | Digital | Analogue | Embedded | MCU | Memory | Industrial Comms | DC Metering | AC Metering | WS | REST | PoE | Memory Map | Bitwise | CM7 | Embedded C